3GPP TSG-RAN Meeting#96	RP-221427

# Budapest, HU, 6 – 9 June 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1132 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the NS value applicability for operation in the n77 frequency range in the US |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2022-06-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correct the applicablility of NS_55 and specify the allowed A-MPR for non-contigous CA across 3450-3550 and 3700-3980 MHz with cells indicating different NS values. For operation in the n77 range in the US, the NS value NS_55 is used for preventing UEs not certified for the 3450-3550 MHz range attach to cells in this range. However, for support of UL CA with serving cells in the range 3700-3980 MHz the network should be able to configure the UE with a different NS value (NS_01) in this range, which would be an exception to the case for standard intra-band CA: “Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.” in the 38.331 due to the MPR and A-MPR specification of intra-band UL CA, the same NS value for all uplink serving cells in n77. The allowed A-MPR (A-MPR according to NS_01) should also be specified for the case in which the network exceptionally configures different NS values for the intra-band uplink carriers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2.3: Table 6.2.3.1-1: the NS_55 is applicable for the new range 3450-3550 MHz, but is only applicable for UEs supporting extendedBand-n77-r16. Clause 6.2A.3.1.2: for CA_n77(2A) with one uplink cell in 3450-3550 MHz and the other in 3700-3980 MHz the UE can exceptionally be configured with different NS values for the CCs of this intra-band CA configuration. In this cases A-MPR according to the CA_NC_NS_01 applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour of a UE configured with different NS values for the uplink carrier(s) of an intra-band CA configuration as part of RRC reconfiguration is not specified . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2A.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR 3199 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same AMPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK  is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.4 | n12, n85 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5,10 |  |  |
| NS_07 | 6.5.3.3.26 | n13 | 5,10 | Table 6.2.3.29-1 | Table 6.2.3.29-2 |
| NS_10 |  | n20, n82 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5,10 | Table 6.2.3.21-1 | Table 6.2.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.22-1 | Table 6.2.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10,15,20 | Table 6.2.3.23-1 | Table 6.2.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5,10,15,20 | Table 6.2.3.24-1 | Table 6.2.3.24-2 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.3.3.12 | n30 | 5, 10 |  | Clause 6.2.3.14 |
| NS_24 | 6.5.3.3.13 | n65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.3.15-1 | Clause 6.2.3.15 |
| NS_27 | 6.5.2.3.8 6.5.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.16-1 | Table 6.2.3.16-2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 30, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25, 30, 40 | Table 6.2.3.20-1 | Table 6.2.3.20-1 |
| NS_45 | 6.5.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.25 |
| NS_46 | 6.5.3.3.25 | n7 | 25, 30, 35, 40, 50 | Table 6.2.3.17-1 | Table 6.2.3.17-2 |
| NS_47 | 6.5.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.18-1 | Table 6.2.3.18-2 |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.16 | n39, n98 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.19 |
| NS_51 | 6.5.3.3.22 | n65 | 50 | Table 6.2.3.28-1 | Table 6.2.3.28-2 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.27 | n24, n99 | 5, 10 |  | Clause 6.2.3.30 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWChannel where the lower channel edge is ≥ 1930 MHz,10 MHz BWChannel where the lower channel edge is ≥ 1950 MHz and 15 MHz BWChannel where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for UEs indicating extendedBand-n77 for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

< text omitted >

##### 6.2A.3.1.2	UE additional maximum output power reduction for Intra-band non-contiguous CA

Table 6.2A.3.1.2-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NC_NS value. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.1.2-2.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 |  | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |

For UEs configured with intra-band non-contiguous CA in n77 and with NS_01 indicated for an uplink component carrier the range 3700-3980 MHz and NS_55 for another uplink component carrier in the range 3450-3550 MHz, the allowed A-MPR is according to CA_NC_NS_01.

Table 6.2A.3.1.2-2: Mapping of network signaling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NC_NS_01 | CA_NC_NS_04 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2A.3.1.2.1	AMPR for CA_NC_NS_04 (CA_n41(2A))

< end of changes >