3GPP TSG RAN meeting #96										RP-221436

Budapest, Hungary, June 6-9, 2022

Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Multi-carrier enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MC_enh |  |  |  |  |
| Unique ID | 940094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220834 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 10% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Hiroki Harada |
|  | Company | NTT DOCOMO, INC. |
|  | Email | hiroki.harada@docomo-lab.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

9.10.1	Multi-cell PUSCH/PDSCH scheduling with a single DCI

RAN1#109-e

Agreement

Agree the following terminologies ONLY for convenience of discussion:

- DCI format 0_X is used for scheduling multiple PUSCHs on multiple cells with one PUSCH per cell

- DCI format 1_X is used for scheduling multiple PDSCHs on multiple cells with one PDSCH per cell.

The above does not imply introducing new DCI format(s) at this point.

Agreement

- Different TBs are scheduled on different cells by DCI format 0_X.

- Different TBs are scheduled on different cells by DCI format 1_X.

Agreement

Fallback DCI (i.e., DCI formats 0_0 and 1_0) does not support multi-cell scheduling.

Agreement

The DCI for multi-cell scheduling is monitored only in USS set.

Agreement

- PDSCH cannot be scheduled by DCI format 0_X.

- PUSCH cannot be scheduled by DCI format 1_X.

Agreement

- All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group.

- FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

Agreement

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells including the scheduling cell.

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells not including the scheduling cell.

Agreement

- For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X.

Working Assumption

- All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- DCI format 0_X can be used for single cell PUSCH scheduling.

- DCI format 1_X can be used for single cell PDSCH scheduling.

- FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

Agreement

- DCI format 0-X/1-X can be transmitted on PCell.

- DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell.

- FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Agreement

For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields:

- Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group

- Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group.

  - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2).

- Other types are not precluded.

9.10.2	Multi-carrier UL Tx switching scheme

RAN1#109-e

Conclusion

EN-DC cases are out of scope for Rel-18 UL Tx switching

Conclusion

UL only cell cases are out of scope for Rel-18 UL Tx switching

RAN1 Observation

Four contributions (R1-2203136, R1-2204724, R1-2204909, R1-2205131) from three companies show their evaluation results on UL Tx switching across 3 or 4 bands at RAN1#109-e meeting.

- All evaluation results show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands, assuming TDD bands with different TDD UL/DL configurations are included in 4 bands.

  - Evaluation results in R1-2203136 show the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 3 bands.

  - Evaluation results in R1-2204724 show that the performance gain of UL Tx switching across 4 bands compared with UL Tx switching across 2 bands depends on achievable switching period, and the longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands leads to reduction of the performance gain. Other evaluation results did not consider the impact of longer switching period for UL Tx switching across 4 bands compared with UL Tx switching across 2 bands.

  - Evaluation results in 5131 observe that the gain highly depends on the scheduling mechanism.

  - The range of performance gains shown in four contributions varies depending on the simulation assumptions.

Agreement

Companies are encouraged to investigate pros and cons of following possible mechanisms for dynamic Tx carrier switching across the configured bands, and RAN1 strives for the down-selection at RAN1#110

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via UL grant and/or RRC configuration for UL transmission

- Alt.2: NW indicates 2 bands out of the configured bands (3 or 4 bands) via DCI or MAC-CE, and dynamic Tx carrier switching between indicated bands is same as Rel-17

- Alt.3: One anchor band is selected among configured bands (3 or 4 bands), and dynamic Tx carrier switching can be performed only from the anchor band to a non-anchor band and from a non-anchor band to the anchor band

- Note: Other mechanisms are not precluded

Agreement

Send LS to RAN4 to ask their feedback on the potential increase of switching period and complexity in the case of UL Tx switching across 3 or 4 bands

- In the LS, observations based on the evaluation results and alternative switching mechanisms discussed in RAN1 are captured for the information to RAN4

- In the LS, RAN1 also asks RAN4 feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands

  - When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period

LS is endorsed in R1-2205502.

Conclusion

If Rel-18 UL Tx switching is supported, following assumption is applied for Rel-18 UL Tx switching across up to 3 or 4 bands

- Only when the two Tx chains are linked to one NR band, the 2-ports UL transmission on the NR band is possible

RAN1 Observation

Following proposals to address the concern on UE/gNB complexity increase or scheduling restriction due to UL Tx switching across larger number of bands compared with Rel-16/17 are identified in contributions submitted at RAN1#109-e, and companies are encouraged to investigate pros and cons of the proposals so that one or some of them may be down-selected after the down-selection of the mechanism for dynamic Tx carrier switching across the configured bands

- UE can report the supports of only some of concurrent UL cases (combinations of 2 bands for concurrent UL transmissions)

- Switching across 0/1/2 ports is supported only for 2 configured bands out of 3 or 4 configured bands and other bands support switching across 0/1 port only

- Only switching across 0/1 port is supported across all configured bands when 3 or 4 bands are configured

- Prioritization rules between uplink carriers are specified

- No restriction on the UEs choice of MIMO capability on any of the bands/CCs involved in the UL Tx switching band combination is introduced

- After one RF state switch, the next RF state switch must occur after 14 symbols or later (FFS: which SCS is assumed for the symbol duration)

- Note: Other solutions are not precluded

- Note: each proposal assumes certain mechanism for dynamic Tx carrier switching across the configured bands, and hence some or all of the proposals may not be necessary depending on the down selection of the mechanism for dynamic Tx carrier switching across the configured bands

Conclusion

It is RAN1’s understanding that RAN4 should lead the discussion on UL Tx switching with multiple TAGs for both 2 bands case and more than 2 bands case

- For further discussion in RAN1 with regards to UL Tx switching with multiple TAGs, it will be discussed only if triggered by RAN4

- If it is decided to support UL Tx switching with multiple TAGs, it is RAN1's working assumption that the number of TAGs should be limited to up to 2

RAN1 Observation

Following possible switching configurations can be considered, and RAN1 may discuss if any of the following switching configurations need to be supported after making some progress on the discussion on the switching mechanism

- For 3 bands case

  - Switching configuration.3-1: all the 3 bands support up to 2Tx

  - Switching configuration.3-2: only 1 band out of 3 bands support up to 2Tx

  - Switching configuration.3-3: only 2 bands out of 3 bands support up to 2Tx

- For 4 bands case

  - Switching configuration.4-1: all the 4 bands support up to 2Tx

  - Switching configuration.4-2: only 1 band out of 4 bands support up to 2Tx

  - Switching configuration.4-3: only 2 bands out of 4 bands support up to 2Tx

  - Switching configuration.4-4: only 3 bands out of 4 bands support up to 2Tx

- Note: The Spec should not restrict which Tx chain is fixed or switched across certain bands.

2.1.2	Remaining Open issues

All objectives related to RAN1 from WID on Multi-carrier enhancements for NR as below

-Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Identify the maximum number of cells that can be scheduled simultaneously

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

-Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

All objectives related to RAN2 from WID on Multi-carrier enhancements for NR as below

- Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

  - UE capability and RRC configuration related signalling (RAN2)

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

All objectives related to RAN4 from WID on Multi-carrier enhancements for NR as below

-Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

- Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4)

-Specify RRM test cases related to core requirements on UL Tx switching schemes across up to 3 or 4 bands

[RAN4]

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#109-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2204397 | Work plan for Multi-carrier enhancements | NTT DOCOMO, INC. |
| R1-2203135 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single scheduling DCI | Huawei, HiSilicon |
| R1-2203207 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | ZTE |
| R1-2203276 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Nokia, Nokia Shanghai Bell |
| R1-2203346 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | Spreadtrum Communications |
| R1-2203448 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CATT |
| R1-2203583 | Discussion on multi-cell scheduling | vivo |
| R1-2203664 | Discussion on multi-cell scheduling with a single DCI | China Telecom |
| R1-2203688 | Discussion on Multi-cell PXSCH scheduling with a single DCI | NEC |
| R1-2203706 | Discussion on multi-cell scheduling via a single DCI | Lenovo |
| R1-2203800 | Discussion on the design of multi-cell scheduling with a single DCI | xiaomi |
| R1-2203842 | Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI | Langbo |
| R1-2203925 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Samsung |
| R1-2204026 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | OPPO |
| R1-2204087 | Multi-cell scheduling with a single DCI | InterDigital, Inc. |
| R1-2204186 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CAICT |
| R1-2204262 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Apple |
| R1-2204324 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CMCC |
| R1-2204398 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | NTT DOCOMO, INC. |
| R1-2204631 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | LG Electronics |
| R1-2204697 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | MediaTek Inc. |
| R1-2204816 | Discussions on multi-cell scheduling with a single DCI | Intel Corporation |
| R1-2204865 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Charter Communications |
| R1-2204888 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Ericsson |
| R1-2205051 | Multi-cell PUSCH and PDSCH scheduling with a single DCI | Qualcomm Incorporated |
| R1-2205073 | Discussion on Multicarrier scheduling with a single DCI | FGI |
| R1-2205088 | Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI | Fujitsu Limited |
| R1-2205234 | Feature lead summary#1 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2205236 | Feature lead summary#3 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2205486 | Feature lead summary#4 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2205487 | Feature lead summary#5 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2205488 | Feature lead summary#6 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2203136 | Discussion on multi-carrier UL Tx switching | Huawei, HiSilicon |
| R1-2203347 | Discussion on multi-carrier UL Tx switching scheme | Spreadtrum Communications |
| R1-2203584 | Discussion on UL TX switching | vivo |
| R1-2203665 | Discussion on UL Tx switching across up to 3 or 4 bands | China Telecom |
| R1-2203743 | Considerations on Multi-carrier UL Tx switching scheme | Sony |
| R1-2203801 | Discussion on multi-carrier UL Tx switching scheme | xiaomi |
| R1-2203926 | Multi-carrier UL Tx switching scheme | Samsung |
| R1-2204027 | Discussion on multi-carrier UL Tx switching scheme | OPPO |
| R1-2204088 | Multi-carrier UL Tx switching scheme | InterDigital, Inc. |
| R1-2204263 | On the support of multi-carrier UL Tx switching scheme | Apple |
| R1-2204325 | Discussion on multi-carrier UL Tx switching scheme | CMCC |
| R1-2204399 | Discussion on multi-carrier UL Tx switching scheme | NTT DOCOMO, INC. |
| R1-2204632 | Discussion on Multi-carrier UL Tx switching scheme | LG Electronics |
| R1-2204724 | On multi-carrier UL Tx switching scheme | MediaTek Inc. |
| R1-2204817 | Discussions on multi-carrier UL Tx switching scheme | Intel Corporation |
| R1-2204889 | Multi-carrier UL Tx switching | Ericsson |
| R1-2205052 | Discussion on Rel-18 UL Tx switching | Qualcomm Incorporated |
| R1-2205089 | Initial views on multi-carrier UL Tx switching scheme | Fujitsu Limited |
| R1-2205106 | On Multi-Carrier UL Tx Switching | Nokia, Nokia Shanghai Bell |
| R1-2205131 | Discussion on Multi-carrier UL Tx switching scheme | ZTE |
| R1-2205137 | Discussion on multi-carrier UL Tx switching scheme | CATT |
| R1-2205230 | Summary of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2205363 | Summary#2 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2205471 | Summary#3 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2205501 | [Draft] LS on UL Tx switching across 3 or 4 bands in Rel-18 | Moderator (NTT DOCOMO) |
| R1-2205502 | LS on UL Tx switching across 3 or 4 bands in Rel-18 | RAN1, NTT DOCOMO |
| R1-2205589 | Summary#4 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2203209 | Simulation evaluation on Rel-18 Multi-carrier enhancement | ZTE |
| R1-2204890 | Other aspects related to multi-carrier enhancements | Ericsson |
| R1-2204909 | Simulation evaluations of single user experience data rate for multi-carrier UL Tx switching | Huawei, HiSilicon |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template