3GPP TSG-RAN Meeting #96                                                                    RP-221457

Budapest, Hungary, June 6-9, 2022

Source:	CATT

Title:	Discussion on posSIB broadcast via PC5

Agenda Item:	9.2.5

Document for:	Discussion and Decision

Introduction

In Rel-16, posSIB in System information can be requested by the UE to achieve assistance data for A-GNSS and/or for RAT-Dependent positioning methods via Uu.

During RAN2 #118-e meeting, posSIBs forwarded to the Remote UE via Relay UE was discussed, and the following agreement was reached:

Agreements:

Support for requesting posSIBs is not added in this release (on PC5 or Uu).

The existing support for posSIBs in DedicatedSIBRequest is not removed now.  Can be further discussed if a restriction is needed.

Since posSIB via PC5 may be discussed in two work items, i.e. positioning and sidelink relay, this contribution will discuss and give a proposal on how to handle the issue of the broadcast of posSIB via PC5.

Discussion

Broadcast of positioning assistance data is supported via Positioning System Information Blocks (posSIBs) as specified in TS 36.331 [1] and TS 38.331 [2]. The posSIBs are carried in RRC System Information (SI) messages (TS 36.331 [1] and TS 38.331 [2]).

For NR RRC System Information (SI), a single SIBpos IE is defined in TS 38.331 [2] which is carried in IE PosSystemInformation-r16-IEs specified in TS 38.331 [2]. The mapping of positioning SIB type (posSibType) to assistance data carried in SIBpos includes:

- The GNSS Common and Generic Assistance Data IEs.

- The OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs.

- The Barometric Assistance Data IEs.

- The TBS (based on MBS signals) Assistance Data IEs.

GNSS Common Assistance Data

GNSS Generic Assistance Data

OTDOA Assistance Data

Barometric Assistance Data

TBS Assistance Data

NR DL-TDOA/DL-AoD Assistance Data

On-demand DL-PRS Configurations

Usually what assistance data should be broadcasted is defined and decided by positioning session both in NR_newRAT-Core and NR_pos-Core WIs. For example, the agenda item of NR Positioning Support included broadcast assistance data in Rel-16.

The SID of Study on expanded and improved NR positioning for Rel-18 includes the following objectives:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

Obviously, whether to broadcast or what positioning assistance data can be broadcasted via PC5 should be discussed and decided in FS_NR_pos_enh2 in Rel-18 which is similar to the discussion on assistance data via Uu in NR_pos-Core in Rel-16.

On the other hand, positioning SIBs to the Remote UE via Relay UE was discussed under SL relay WI during RAN2 #118-e, and the following agreement was reached:

Agreements:

Support for requesting posSIBs is not added in this release (on PC5 or Uu).

The existing support for posSIBs in DedicatedSIBRequest is not removed now.  Can be further discussed if a restriction is needed.

From our understanding, whether and what positioning assistance data can be broadcasted via PC5 is in the scope of FS_NR_pos_enh2 in Rel-18, considering the scenario/requirements. For example, the OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs should not be broadcasted to remote UE in the U2N relay scenario since there is no DL-PRS coverage. There may be further assistance data in Rel-18 considering new features introduced. So we propose to discuss related issues in Rel-18 POS SI or the following POS WI.

Proposal: Whether to support forwarding Uu posSIBs via PC5 (e.g. for remote UE in non-connected state) can be discussed in Rel-18 FS_NR_pos_enh2 SI or the following Rel-18 pos WI.

Conclusion

According to the analysis in section 2, the following proposal is provided:

Proposal: Whether to support forwarding Uu posSIBs via PC5 (e.g. for remote UE in non-connected state) can be discussed in Rel-18 FS_NR_pos_enh2 SI or the following Rel-18 pos WI.

Reference

[1].TS36.331 Radio Resource Control (RRC);

[2].TS38.331 Radio Resource Control (RRC);