3GPP TSG RAN Meeting #96												RP-221471

Budapest, Hungary, June 6-9, 2022

Agenda Item:	10.2.1

Source: 	NEC, Rakuten Mobile Inc.

Title:	Views on Supporting Discontinuous Coverage in Rel-18 IoT-NTN

Document for:	Discussion

- Introduction

From RAN94e meeting, a new work item is proposed to define further enhancements for NB-IoT NTN and eMTC NTN, in order to improve mobility aspects, improve performance in terms of throughput, and optimize the GNSS operation and power efficiency for long-term connection [1]. For supporting further enhancement on discontinuous coverage, corresponding objectives need to be revisited subject to Rel-17 IoT-NTN status on discontinuous coverage as follows.

4.1.3 Further enhancement to discontinuous coverage This objective will be revisited at RAN#96e / June 2022, subject to Rel-17 IoT-NTN status on discontinuous coverage at RAN#96e, SA#94e, CT#95e.

This contribution provides our views on supporting discontinuous coverage in Rel-18 IoT-NTN.

- Discussion

In NGSO earth-fixed and earth moving cells, a UE would experience cells moving towards and away from itself. It is possible that there are occasional coverage holes due to satellites moving and not available continuously. In this case, specification work is needed to assist IoT-NTN network in handling the discontinuous coverage provided by sparse IoT-NTN constellations. At least, the following aspects need to be considered to minimize the impact of discontinuous coverage.

Camping procedures in RRC_IDLE state

For unnecessary camping procedures in RRC_IDLE state (i.e., cell searches and ‘NAS-attach’ procedures) when there is no NTN coverage, a brief solution came out after Rel-17 works. Using the satellite assistance information provided by network, UE with GNSS capability can aware of the situation of discontinuous coverage, and also capable of predicting the upcoming satellite's coverage and end-time of satellite's coverage. When out of coverage, AS functions are not required to be performed and power saving can be achieved.

Excessive failures/recovery actions

When the satellite stops serving the area it is currently covering, UE would suddenly lose the connection and initiate corresponding recovery actions, i.e., radio link failure recovery. Since a neighboring NTN coverage may not be available continuously, the recovery actions would lead to excessive power consumption for UE. Similarly, for NW unware of UE’s being out of coverage, it may also lead to waste of energy and sources.

The impact of this problem is more serious for earth moving cells, considering the time for each UE being out-of-coverage varies depending on the UE’s location. Therefore, an assistance information of approaching the out-of-coverage area provided by UE may be needed.

Failures/recovery actions lead to excessive power consumption, especially for earth moving cells.

Consider to support UE providing assistance information on being out-of-coverage.

Make better use of the periodical, brief connection opportunities

It is known that some IoT features are designed for infrequently or non-sudden services (i.e., electric meter). For those patterned, regular services, supporting small data and power saving features such as PUR, can make better use of periodical service of NGSO satellites and reduce the impact of discontinuous connection as far as possible.

Some IoT features are designed for patterned, regular services.

Consider enhancements to existing IoT features such as PUR, to support periodical, brief connection in IoT-NTN discontinuous coverage.

Proposed objective

Based on the observations and proposals above, we would like to suggest to revise the corresponding objective as follows:

4.1.3 Further enhancement to discontinuous coverage - Support UE providing assistance information on being out-of-coverage [RAN2] - Study and specify, if needed, enhancements to existing IoT features such as PUR to support periodical, brief connection [RAN2]

Consider to revise the objective 4.1.3 accordingly.

- Conclusion

Observation 1:	Failures/recovery actions lead to excessive power consumption, especially for earth moving cells.

Observation 2:	Some IoT features are designed for patterned, regular services.

Proposal 1:	Consider to support UE providing assistance information on being out-of-coverage.

Proposal 2:	Consider enhancements to existing IoT features such as PUR, to support periodical, brief connection in IoT-NTN discontinuous coverage.

Proposal 3:	Consider to revise the objective 4.1.3 accordingly.

- References

[1] RP-220979, WID on IoT NTN enhancements, MediaTek Inc.