3GPP TSG RAN #96	RP-221582

June 6th – 9th, 2022

Source:	CableLabs , Charter Communications, Broadcom, HPE

Title:	WID revision. NR sidelink evolution

Agenda Item:	9.3.1.5

Document for: 	Discussion and Decision

# Introduction

RANP #95, approved the updated Sidelink Evolution WID [1]. As part of this WID, the Objectives section #2 specifies

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanism

- <<-omitted ->>

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

This paper discusses related to the SL limitations in the unlicensed spectrum, particularly band n96 and proposes a related WID update.

# Limitations of Band n96 Device-to-Device communications.

FCC issued the ruling on the use of 6GHz spectrum for unlicensed operation [2].

Accordingly, 4 new U-NII 5/6/7/8 bands were defined. More details are provided in the appendix

Figure 1. US 6 GHz spectrum. U-NII bands allocation.

2.1 Client to Client communication. US FCC Regulations.

FCC prohibits direct connection of a client (e.g. STA or UE) to another client for U-NII 5, 6, 7, 8 [2].

(5) In the 5.925-7.125 GHz band, client devices, except fixed client devices, must operate under the control of a standard power access point, indoor access point or subordinate devices; Subordinate devices must operate under the control of an indoor access point.

In all cases, an exception exists for transmitting brief messages to an access point when attempting to join its network after detecting a signal that confirms that an access point is operating on a particular channel. Access points and subordinate devices may connect to other access points or subordinate devices. Client devices are prohibited from connecting directly to another client device.

Accordingly, SL-U operation is not supported in US 5.925-7.125GHz (U-NII 5, 6, 7, 8) bands.

Observation 1: Device to device operation is not supported in US bands U-NII 5/6/7/8, 5.925-7.125 GHz spectrum.

2.2 Client to Client communication. Canada ISED Regulations.

Canada’s unlicensed 6 GHz band are open to public discussions, up to 07/15/2022. Based on the actual proposal, opened up for discusssions [3]:

Section #4.7.1

Low-power indoor access points, indoor subordinate devices, and low-power client

devices shall employ a contention-based protocol.

Observation 2: Device to device operation is not supported in Canada’s bands U-NII 5/6/7/8, 5.925-7.125 GHz spectrum.

Another SL-U related regulation concerns the vehicular applications, which are prohibited in this band”

Section #4.8.2

Standard client devices and low-power client devices shall not connect directly to another standard client device or low-power client device;…

Observation 3: Vehicular operation is not supported in Canada’s bands U-NII 5/6/7/8 5.925-7.125GHz spectrum

Based on all the above limitations, it appears that out-of-coverage SL-U operation (Mode 2) is not permitted neither in US nor in Canada in the un-licensed 5.925-7.125 GHz spectrum.

Proposal 1: The Sidelink Rel 18 WID shall not reference band n96, since device-to-device communication operation is not supported under current US and Canada regulations.

# SL-U WID update proposal

The objective of SL Rel 18 WID [1] should be updated as follows:

………………

### Objective of SI or Core part WI or Testing part WI

…………………

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

……………………….

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4]

- The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI.

- The exact frequency band combinations for both FR1 licensed and ITS-dedicated spectrum are to be determined based on company input during the WI.

- Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements.

- Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

- UE RRM core requirement for the new features introduced in this WI [RAN4]

Rel-18 sidelink should be able to coexist with Rel-16/17 sidelink in the same resource pool. This does not preclude the possibility of operating Rel-18 sidelink in a dedicated resource pool.

# References

[1] RP-220300, NR SideLink Evolution,

[2] FCC Title 47 CFR 15.407(a)3V and (d)5

[3] RSS-248 Issue 2, ‘Radio Local Area Network (RLAN) Devices operating in the 5925-7125 MHz Band.’ Innovation, Science and Economics Development Canada (ISED), May 2022

# Appendix

4.1 Appendix A

An overview of the 6GHz U-NII rules is provided below [2]

Subject to public discussion up to 07/15/2022