3GPP TSG-RAN WG1 Meeting #109-e	R1-2205387

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0310 | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Timeline requirement for retransmitting MSG1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (vivo) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current timeline requirement specified in Clause 8.2 of TS 38.213 for MSG1 retransmission for Rel-15 UEs does not correctly reflect the following agreement made in RAN1#92bis. Agreements: Update the previous agreement on Msg1 retransmission as follows: - If a received Msg2 does not contain a response to the transmitted preamble sequence, or no RAR is received by the end of the RAR window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after no later than the duration of N1 + new + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - new = 250 us - L2=500 usec refers to the MAC layer processing time - Note: UE is not mandated to measure each SSB prior to every Msg1 retransmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the wording “is expected” to “shall be ready” to make it clear that the UE is ready to transmit the MSG1 no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception if requested by higher layers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is mismatch between the specification and agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Isolated impact analysis The proposed change is the common understanding in RAN1 and it is not expected to impact the existing gNB/UE implementation. |  |  |  |  |  |  |  |  |

### 8.2	Random access response

<< Unchanged text is omitted >>

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE shall be ready to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

<< Unchanged text is omitted >>