3GPP TSG-RAN WG1 Meeting #109-e	R1-2205396

# e-Meeting, May 9th - 20th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0314 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL HARQ-ACK reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (vivo), ZTE, Sanechips, Ericsson, Huawei, HiSilicon, OPPO, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 16.5.2.1 of 38.213, NCG is defined as a number of SL configured grants for which the UE transmits corresponding HARQ-ACK information in a same PUCCH as that for HARQ-ACK information corresponding to PSFCH reception occasions within the M PDCCH monitoring occasions, but the statement ‘PSFCH reception occasions within the M PDCCH monitoring occasions’ is incorrect. Moreover, it is not clear if ‘a number of SL configured grants’ includes SL CG transmissions with an activation DCI or not. In addtion, it was agreed in R1-2202898 that the 1st SPS PDSCH with a corresponding activation DCI should be handled in the same way as SPS PDSCH without a corresponding PDCCH in a Type2 codebook, and the DAI field of the DL SPS activation DCI is to be ignored. According to the discussion in [109-e-R16-V2X-01], companies believe that the operation of handling PSSCH transmissions of a Type2 SL CG with a corresponding DCI should be aligned with Uu, i.e., these PSSCH transmissions should be handled in the same way as PSSCH transmissions of a SL CG without a corresponding DCI, and SAI in a DCI format 3_0 for SL CG activation should be ignored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DCI format 3_0(s) for determining VSAI,mlastSL, USAI, Nmreceived do not include DCI format 3_0 for SL CG activation, and SL configured grant for determining NCG includes SL CG with or without a corresponding PDCCH. Correct the statement ‘PSFCH reception occasions within the M PDCCH monitoring occasions’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec can lead to different understandings of how the nHARQ-ACK for obtaining a transmission power for a PUCCH is determined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 16.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR aligns with RAN1 common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 16.5.2.1	Type-2 HARQ-ACK codebook in physical uplink control channel

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=VSAI,mlastSL-USAImod4+m=0M-1Nmreceived+NCG

where

-	VSAI,mlastSL is a value of a counter SAI field in a last DCI format 3_0, excluding the DCI format 3_0 activating a SL configured grant, scheduling PSSCH transmissions associated with PSFCH reception occasions that the UE detects within the M PDCCH monitoring occasions

-	VSAI,mlastSL=0 if the UE does not detect any DCI format 3_0, excluding the DCI format 3_0 activating a SL configured grant, scheduling PSSCH transmissions associated with PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	USAI is a total number of DCI format 3_0, excluding the DCI format 3_0 activating a SL configured grant, scheduling PSSCH transmissions associated with PSFCH reception occasions, that the UE detects within the M PDCCH monitoring occasions. USAI=0 if the UE does not detect any DCI format 3_0, excluding the DCI format 3_0 activating a SL configured grant, scheduling PSSCH transmissions with associated PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	Nmreceived is a number of DCI format 3_0, excluding the DCI format 3_0 activating a SL configured grant, scheduling PSSCH transmissions with associated PSFCH reception occasions that the UE detects in PDCCH monitoring occasion m

-	NCG is a number of SL configured grants for which the UE transmits corresponding HARQ-ACK information in a same PUCCH as for HARQ-ACK information corresponding to PSFCH reception occasions associated with PSSCH transmissions scheduled by a dynamic grant within the M PDCCH monitoring occasions