3GPP TSG RAN WG1 Meeting # 109-e	 R1-2205349

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0309 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCCH reception with last symbol aligning with the first symbol of UL CI for Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-5-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the description in clause 11.2A of TS 38.213, if a UE has already cancelled a PUSCH/SRS by a UL CI, and a PDCCH can schedule another PUSCH/SRS transmission, as long as 1) the last symbol of the PDCCH is later than the first symbol of PDCCH carrying the UL CI, and 2) the scheduled PUSCH/SRS isn’t overlapped with the symbols of the cancelled PUSCH/SRS transmission. However, this conflicts with the agreement in RAN1#101-e. Agreement(RAN1#101-e) (Alt 1) If UE has to cancel a PUSCH transmission or an SRS transmission based on the detected UL CI, another DG-PUSCH2 CAN be scheduled on the resource indicated by the UL CI but not overlapping with cancelled symbols, if the ending symbol of the PDCCH carrying the 2nd UL grant is no earlier than the first symbol of the PDCCH carrying the UL CI. - The above applies regardless whether RRC parameter applicabilityforCI is configured or not - No additional spec impact expected.. There is a missing case that if a UE cancels a first PUSCH/SRS transmission based on an indication by a DCI format 2_4, and the last symbol of a PDCCH for a second PUSCH/SRS transmission is aligning with the first symbol of a PDCCH carrying the DCI format 2_4. Then, the second PUSCH/SRS can be scheduled without any restriction according to the current spec. |
| Summary of change: |  | Changing ‘later’ to ‘no earlier’ in the last paragraph of clause 11.2A to reflect that the scheduling restriction (no overlapping with cancelled symbols) is also applicable to the case that ‘the last symbol of a PDCCH for a PUSCH/SRS transmission aligning with the the PDCCH carrying UL CI’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether a PUSCH/SRS can be scheduled over symbols that include symbols of the cancelled PUSCH/SRS transmission, under the case that the last symbol of a PDCCH for a PUSCH/SRS transmission aligning with the the PDCCH carrying UL CI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on RAN1 common understanding, and it is expected that gNB and UE have been implemented in accordance to the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

## 11.2A	Cancellation indication

If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, search space sets for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of each search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in clause 10.1. UplinkCancellation additionally provides to the UE

-	a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI

-	a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier

-	an information payload size for DCI format 2_4 by dci-PayloadSize-ForCI

-	an indication for time-frequency resources by timeFrequencyRegion

For a serving cell having an associated field in a DCI format 2_4, for the field denote by

-	NCI a number of bits provided by ci-PayloadSize

-	BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion

-	TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, from a number of symbols that

-	is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or

-	is equal to the PDCCH monitoring periodicity, otherwise.

-	GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as BCI=LRB  from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB or FrequencyInfoUL that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4.

For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of the PDCCH reception where the UE detects the DCI format 2_4.

A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetition Type B, as determined in clauses 9 and 9.2.5 or in clause 6.1 of [6, TS 38.214], or an SRS transmission on the serving cell if, respectively,

-	the transmission is PUSCH with priority 0, if the UE is provided uplinkCancellationPriority,

-	a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and

-	a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission,

where

-	the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that is in a group of symbols having corresponding bit values of '1' in the DCI format 2_4;

-	the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4.

If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier than the first symbol of the PDCCH reception providing the DCI format 2_4.

< Unchanged parts are omitted >