3GPP TSG RAN WG1 #109-e	        		   R1-2205671

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0326 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on further enhancements on MIMO for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Unclear association of CSI-RS that shares a same indicated TCI state as for the PDCCH and PDSCH in clause 6. - Missing description that a UE can determine the set q0 from a DLorJointTCIState, in addition to a TCI-State, in clause 6. - For operation with unified TCI state, the number of symbols after successful BFR that a UE transmits PUCCH, PUSCH, and SRS using the specified spatial domain filter and the corresponding power control procedure are not defined in clause 6. - Missing description that a UE can provide q0 and at least one of q0,0 and q0,1 in a same MAC CE in clause 6. - Missing RRC parameters providing periodic CSI-RS resource configuration indexes in clause 6. - Reference to “a first” SRS resource from the SRS resource set can be misinterpreted to mean “the first” SRS resource instead of any SRS resource in clause 7. - Correct the PL-RS association to be with DLorJoint-TCIState or UL-TCIState of an SRS resource with lowest SRS-ResourceId in the SRS resource set instead of the indicated DLorJoint-TCIState or UL-TCIState in clause 7. - Undefined ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ association to SRS resource sets for m-TRP Type 1 CG PUSCH repetitions in clause 7.1.1. - Unclear BWP/carrier/serving cell for PUCCH resource with lowest index in m-TRP operation when a PUCCH resource is activated with two spatial relations in clause 7.1.1. - Unclear whether, for m-TRP PUCCH, two RS resources for DL pathloss estimates are determined when pathlossReferenceLinking is provided in clause 7.1.1. - Determination of power control parameters when sTRP mode is indicated for mTRP PUSCH repetition is not described in clause 7.1.1. - Missing determination of closed-loop index based on one or two power control parameter sets provided for a PUCCH resource for M-TRP PUCCH operation in FR1 in clause 7.2.1. - Unclear default power control parameter set per TRP (for that case SRI field is absent) for virtual PHR determination in clause 7.7.1. - Missing description for determination of power control parameters when sTRP mode is indicated for mTRP PUSCH repetition in clause 7.1.1. - Missing PHR determination for reference PUSCH when a UE is provided DLorJoint-TCIState or UL-TCIstate in clause 7.7.1. - Miscellaneous editorial corrections for typos, syntax, missing or redundant words, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that a CSI-RS is an aperiodic CSI-RS resource in a CSI-RS resource set in clause 6. - Add that a UE can determine the set q0 from a DLorJointTCIState in clause 6. - Add description that a UE can provide q0 and at least one of q0,0 and q0,1 in a same MAC CE in clause 6. - Define the number of symbols after successful BFR that a UE transmits PUCCH, PUSCH, and SRS using the specified spatial domain filter and the corresponding power control procedure in clause 6. - Add RRC parameters providing periodic CSI-RS resource configuration indexes in clause 6. - Remove “first” from SRS resource in clause 7. - Remove “indicated” in clause 7. - Define ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ association to SRS resource sets for m-TRP Type 1 CG PUSCH repetitions in clause 7.1.1. - Clarify the BWP/carrier/serving cell for PUCCH resource with lowest index in m-TRP operation when a PUCCH resource is activated with two spatial relations in clause 7.1.1. - Add an ‘s’ to “RS resource” to allow for two RS resources for DL PL estimates when pathlossReferenceLinking is provided in clause 7.1.1. - Add description for determination of power control parameters when sTRP mode is indicated for mTRP PUSCH repetition in clause 7.1.1. - Clarify how to obtain a TPC command value when more than one sets of power control parameters are applicable in clause 7.2.1 - Clarify default power control parameter set per TRP (for that case SRI field is absent) for virtual PHR determination in clause 7.7.1. - Define virtual PHR determination for default power control parameter set per TRP in clause 7.7.1. - Capture PHR determination for reference PUSCH when a UE is provided DLorJoint-TCIState or UL-TCIstate in clause 7.7.1. - Miscellaneous editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for MIMO enhancements in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7, 7.1, 7.1.1, 7.2.1, 7.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS 38.212, TS 38.214, TS 38.321, TS 38.331 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### *** Unchanged text is omitted ***

# 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321] and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the set q1,1.

If the UE is not provided q0 by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set q0 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State or DLorJointTCIState for respective CORESETs that the UE uses for monitoring PDCCH. If the UE is not provided q0,0 or  q0,1 for a BWP of the serving cell, the UE determines the set q0,0 or  q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for first and second CORESETs that the UE uses for monitoring PDCCH, where the UE is provided two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided coresetPoolIndex value for the first CORESETs and is provided coresetPoolIndex value of 1 for the second CORESETs, respectively. If there are two RS indexes in a TCI state, the set q0 or q0,0, or q0,1 includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states. If a CORESET that the UE uses for monitoring PDCCH includes two TCI states and the UE is provided sfnSchemePdcch set to 'sfnSchemeA' or 'sfnSchemeB', the set q0 includes RS indexes in the RS sets associated with the two TCI states.

The UE expects the set q0 to include up to two RS indexes. If the UE is provided q0,0 or q0,1, the UE expects the set q0,0 or the set q0,1 to include up to a number of NBFD RS indexes indicated by capabilityparametername. If the UE is not provided q0,0 or q0,1, and if a number of active TCI states for PDCCH receptions in the first or second CORESETs is larger than NBFD, the UE determines the set q0,0 or q0,1 to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets associated with the active TCI states for PDCCH receptions in the first or second CORESETs corresponding to search space sets according to an ascending order for PDCCH monitoring periodicity. If more than one first or second CORESETs correspond to search space sets with same monitoring periodicity, the UE determines the order of the first or second CORESETs according to a descending order of a CORESET index.

If a UE

-	is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on an active DL BWP of a serving cell,

-	is provided coresetPoolIndex with a value of 1 for second CORESETs on the active DL BWP of the serving cells, and

-	is provided SSB-MTCAdditionalPCI

SS/PBCH block indexes associated with a physical cell identity other than the one provided by physCellId in ServingCellConfigCommon can be provided in either q1,0 or q1,1 set and the corresponding q0,0 or q0,1 set is associated with the physical cell identity.

The UE expects single port RS in the set q0, or q0,0, or q0,1. The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set q1, or q1,0, or q1,1. The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or rsrp-ThresholdBFR, respectively.

The physical layer in the UE assesses the radio link quality according to the set q0, q0,0, or q0,1, of resource configurations against the threshold Qout,LR. For the set q0, the UE assesses the radio link quality only according to SS/PBCH blocks on the PCell or the PSCell or periodic CSI-RS resource configurations that are quasi co-located, as described in [6, TS 38.214], with the DM-RS of PDCCH receptions by the UE. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained from a SS/PBCH block. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by powerControlOffsetSS.

In non-DRX mode operation, the physical layer in the UE provides an indication to higher layers when the radio link quality for all corresponding resource configurations in the set q0, or in the set q0,0 or q0,1, that the UE uses to assess the radio link quality is worse than the threshold Qout,LR. The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by the maximum between the shortest periodicity among the SS/PBCH blocks on the PCell or the PSCell and/or the periodic CSI-RS configurations in the set q0, q0,0, or q0,1 that the UE uses to assess the radio link quality and 2 msec. In DRX mode operation, the physical layer provides an indication to higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined as described in [10, TS 38.133].

For the PCell or the PSCell, upon request from higher layers, the UE provides to higher layers the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set q1, or q1,0, or q1,1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold.

For the SCell, upon request from higher layers, the UE indicates to higher layers whether there is at least one periodic CSI-RS configuration index or SS/PBCH block index from the set q1, or q1,0, or q1,1 with corresponding L1-RSRP measurements that is larger than or equal to the Qin,LR threshold, and provides the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set q1, or q1,0, or q1,1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, if any.

For the PCell or the PSCell, a UE can be provided a CORESET through a link to a search space set provided by recoverySearchSpaceId, as described in clause 10.1, for monitoring PDCCH in the CORESET. If the UE is provided recoverySearchSpaceId, the UE does not expect to be provided another search space set for monitoring PDCCH in the CORESET associated with the search space set provided by recoverySearchSpaceId.

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in clause 8.1. For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index qnew provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac, where μ is the SCS configuration for the PRACH transmission and kmac is a number of slots provided by K-Mac [12, TS 38.331] or kmac=0 if K-Mac is not provided, within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH receptions, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index qnew until the UE receives by higher layers an activation for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList.

For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0

For the PCell or the PSCell and for sets q0 and q1, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where a UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE assumes same antenna port quasi-collocation parameters as the ones associated with index qnew for PDCCH monitoring in a CORESET with index 0.

If a UE is provided TCI-State_r17 indicating a unified TCI state for the PCell or the PSCell [6, TS 38.214], after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId where the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI, the UE

-	if AdditionalPCIInfo is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH, using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and the PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0-Alpha-CLID-PUSCH-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0-Alpha-CLID-PUCCH-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0-Alpha-CLID-SRS-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

For the remaining of this clause, if a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, as described in clause 10.1, the last symbol of the PDCCH reception is the last symbol of the PDCCH candidate that ends later. The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

For the PCell or the PSCell, if BFR MAC CE [11, TS 38.321] is provided in Msg3 or MsgA of contention based random access procedure, and if a PUCCH resource is provided with PUCCH-SpatialRelationInfo, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in clause 5.1.4a or in clause 5.1.5 of [11, TS 38.321], the UE transmits the PUCCH on a same cell as the PRACH transmission using

-	a same spatial filter as for the last PRACH transmission

-	a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, where qnew is the SS/PBCH block index selected for the last PRACH transmission.

If a UE is provided TCI-State_r17 indicating a unified TCI state for the PCell or the PSCell and the UE provides BFR MAC CE in Msg3 or MsgA of contention based random access procedure, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in [11, TS 38.321], the UE

-	if AdditionalPCIInfo is not provided, monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set with same indicated TCI state as for the PDCCH and PDSCH using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as for the last PRACH transmission using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0-Alpha-CLID-PUSCH-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0-Alpha-CLID-PUCCH-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0-Alpha-CLID-SRS-Set associated with the smallest value of ul-powercontrolId for the PCell or the PSCell

A UE can be provided, by schedulingRequestID-BFR-SCell, a configuration for PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 for the UE to transmit PUCCH [11, TS 38.321]. If the PCell or the PSCell is associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can be provided by schedulingRequestIDForMTRPBFR a first configuration for PUCCH transmission with a LRR and, if the UE provides twoLRRcapability, a second configuration for PUCCH transmission with a LRR. If the UE is provided only the first configuration, the UE transmits a PUCCH with LRR for either set q0,0 or q0,1. If the UE is provided both the first and second configurations, the UE uses the first configuration to transmt a PUCCH with LRR associated with set q0,0 and the second configuration to transmit a PUCCH with LRR associated with set q0,1 [11, TS 38.321].

The UE can provide in a first PUSCH MAC CE index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any

-	transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew, if any, for periodic CSI-RS or SS/PBCH block reception, as described in clause 9.2.2, and using a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, if

-	the UE is provided PUCCH-SpatialRelationInfo for the PUCCH,

-	a PUCCH with the LRR was either not transmitted or was transmitted on the PCell or the PSCell, and

-	the PUCCH-SCell is included in the SCell(s) indicated by the MAC-CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

If a UE is provided TCI-State_r17 indicating a unified TCI state, after 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs, and receives PDSCH and aperiodic CSI-RS resource in a CSI-RS resource set using the same antenna port quasi co-location parameters as the ones associated with the corresponding index qnew, if any

-	transmits PUSCH, PUCCH and SRS that uses a same spatial domain filter with same indicated TCI state as for the PUSCH and PUCCH, using a same spatial domain filter as the one corresponding to qnew, if any, and using the following parameters for determination of a corresponding power as described in clauses 7.1.1, 7.2.1, and 7.3.1

-	the RS index qd=qnew for obtaining the downlink pathloss estimate

-	the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l provided by p0-Alpha-CLID-PUSCH-Set associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the value of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l provided by p0-Alpha-CLID-PUCCH-Set associated with the smallest value of ul-powercontrolId for the corresponding SCell

-	the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and the SRS power control adjustment state l provided by p0-Alpha-CLID-SRS-Set associated with the smallest value of ul-powercontrolId for the corresponding SCell

If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0 and/or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from q1 and/or corresponding sets q1,0 and/or q1,1 for the serving cells.

For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, and having radio link quality worse than Qout,LR, after 28 symbols from a last symbol of a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the second PUSCH and having a toggled NDI field value, the UE assumes antenna port quasi-collocation parameters

-	corresponding to qnew from q1,0, if any, for the first CORESETs,

-	corresponding to qnew from q1,1, if any, for the second CORESETs

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cells.

# 7	Uplink Power control

### *** Unchanged text is omitted ***

In the remaining of this clause, if a UE is provided DLorJoint-TCIState or UL-TCIstate and for an indicated DLorJoint-TCIState or UL-TCIstate as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by PL-RS associated with or included in the indicated DLorJoint-TCIState or UL-TCIstate except for SRS transmission that is not provided useIndicatedTCIState

-	in clause 7.1.1, if p0-Alpha-CLID-PUSCH-Set is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0-Alpha-CLID-PUSCH-Set associated with the indicated DLorJoint-TCIState or UL-TCIstate

-	in clause 7.2.1, if p0-Alpha-CLID-PUCCHSet is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0-Alpha-CLID-PUCCH-Set associated with the indicated DLorJoint-TCIState or UL-TCIstate

-	in clause 7.3.1, if p0-Alpha-CLID-SRS-Set is provided,

-	if useIndicatedTCIState is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0-Alpha-CLID-SRS-Set associated with the indicated DLorJoint-TCIState or UL-TCIState

-	else, if useIndicatedTCIState is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0-Alpha-CLID-SRS-Set associated with DLorJoint-TCIState or UL-TCIState of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by PL-RS associated with or included in the DLorJoint-TCIState or UL-TCIState of an SRS resource with lowest SRS-ResourceId in the SRS resource set

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10, 11.1, and 11.1.1.

### *** Unchanged text is omitted ***

## 7.1	Physical uplink shared channel

### *** Unchanged text is omitted ***

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2,

-	s=1 for full power TPMIs reported by the UE [18, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

### *** Unchanged text is omitted ***

### 7.1.1	UE behaviour

### *** Unchanged text is omitted ***

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a transmission of a configured grant Type 1 PUSCH and for active UL BWP b of carrier f of serving cell

-	a first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig that is associated with the first srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	a second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig that is associated with the second srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format 0_0, and for active UL BWP b of carrier f of serving cell

-	a first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig

-	else, PO_NOMINAL,PUSCH,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by p0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c

### *** Unchanged text is omitted ***

-	For j∈2,…,J-1=SJ, a PO_NOMINAL,PUSCH,f,cj value, applicable for all j∈SJ, is provided by p0-NominalWithGrant, or PO_NOMINAL,PUSCH,f,cj=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant is not provided, for each carrier f of serving cell c and a set of PO_UE_PUSCH,b,f,cj values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' and if, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively.

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between

-	a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value, and

-	a set of values associated with the second SRI field value for a same number of layers as indicated by the first SRI field [5, TS 38.212], and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212].

-	If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1' and if the DCI format scheduling the PUSCH transmission includes an SRI field, the UE determines a value of PO_UE_PUSCH,b,f,cj from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value.

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook'.

-	if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook', respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook',

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook' and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook'

-	if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook, and a second value, that is associated with the second SRS field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook', respectively.

-	if the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission, except for the PUSCH retransmission corresponding to a RAR UL grant, is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2,

-	if P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj from

-	a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00'

-	a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01'

-	a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10'

-	else, the UE determines PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11

-	if P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as

-	first and second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00'

-	first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '1' or '01'

-	second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '10' or '11'

-	else, the UE determines first and second values PO_UE_PUSCH,b,f,cj from the values of the first and second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively

-	if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value '00'

-	if the UE is provided P0-PUSCH-Set and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj as

-	first P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00'

-	first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '1' or '01'

-	second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '10' or '11'

-	else, the UE determines a value of PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-  if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value '01'

-	if P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj as

-	second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00'

-	first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '1' or '01'

-	second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '10' or '11'

-	else, the UE determines a value of PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

PO_UE_PUSCH,b,f,cj

-	For αb,f,cj

-	For j=0,

-	if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha

-	elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, αb,f,c(0) is the value of msg3-Alpha

-	else, αb,f,c0=1

-	For j=1,

-	if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	if the SRS resource set indicator value is '00', first αb,f,c(1) value is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig

-	if the SRS resource set indicator value is '01', first αb,f,c(1) value is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig

-	if the SRS resource set indicator value is '10' or '11', first and second αb,f,c(1) values are respectively provided by p0-PUSCH-Alpha and p0-PUSCH-Alpha2 in ConfiguredGrantConfig

- else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a transmission of a configured grant Type 1 PUSCH and for active UL BWP b of carrier f of serving cell

-	a first αb,f,c(1) value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig that is associated with the first srs-ResourceIndicator in rrc-ConfiguredUplinkGrant.

-	a second αb,f,c(1) value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig that is associated with the second srs-ResourceIndicator in rrc-ConfiguredUplinkGrant.

- else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format 0_0 and for active UL BWP b of carrier f of serving cell

-	a firstPO_UE_PUSCH,b,f,c1 αb,f,c(1) value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig.

-	else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by alpha for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c

-	For j∈SJ, a set of αb,f,cj values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId in p0-AlphaSets,

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and

-	the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value

-	if a DCI format scheduling the PUSCH transmission includes one SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, and the UE determines αb,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value '10' or '11', the UE determines αb,f,cj from first and second P0-PUSCH-AlphaSet in p0-AlphaSets

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value '00' or '01', the UE determines αb,f,cj from first P0-PUSCH-AlphaSet or second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively.

-	MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c

### *** Unchanged text is omitted ***

-	∆TF,b,f,ci=10log102BPRE⋅Ks-1⋅βoffsetPUSCH for Ks=1.25 and ∆TF,b,f,ci=0 for Ks=0 where Ks is provided by deltaMCS for each UL BWP b of each carrier f and serving cell c. If the PUSCH transmission is over more than one layer [6, TS 38.214], ∆TF,b,f,ci=0. BPRE and βoffsetPUSCH, for active UL BWP b of each carrier f and each serving cell c, are computed as below

-	BPRE=r=0C-1KrNRE for PUSCH with UL-SCH data and BPRE=Qm⋅RβoffsetPUSCH for CSI transmission in a PUSCH without UL-SCH data, where

-	C is a number of transmitted code blocks, Kr is a size for code block r, and NRE is a number of resource elements determined as NRE=N⋅MRB,b,f,cPUSCH(i)⋅j=0Nsymb,b,f,cPUSCHi-1Nsc,dataRB(i,j), where N≥1 is provided by numberOfSlotsTBoMS as described in [6, TS 38.214] and N=1 if numberOfSlotsTBoMS is not provided, Nsymb,b,f,cPUSCHi is a number of symbols for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, Nsc,dataRB(i,j) is a number of subcarriers excluding DM-RS subcarriers and phase-tracking RS samples [4, TS 38.211] in PUSCH symbol j and assuming no segmentation for a nominal repetition in case the PUSCH transmission is with repetition Type B, 0≤j<Nsymb,b,f,cPUSCHi, and c, Kr are defined in [5, TS 38.212]

-	βoffsetPUSCH=1 when the PUSCH includes UL-SCH data and βoffsetPUSCH=βoffsetCSI,1, as described in clause 9.3, when the PUSCH includes CSI and does not include UL-SCH data

-	Qm is the modulation order and R is the target code rate, as described in [6, TS 38.214], provided by the DCI format scheduling the PUSCH transmission that includes CSI and does not include UL-SCH data

-	For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i

-	δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3

-	l∈0,1 if the UE is configured with twoPUSCH-PC-AdjustmentStates and l=0 if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in clause 8.3

-	if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell

-	if the SRS resource set indicator value is 00, l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig

-	if the SRS resource set indicator value is 01, l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig

-	if the SRS resource set indicator value is 10 or 11, a first l and a second l are respectively equal to powerControlLoopToUse and powerControlLoopToUse2 in ConfiguredGrantConfig

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a transmission of a configured grant Type 1 PUSCH and for active UL BWP b of carrier f of serving cell

-	a first l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig that is associated with the first srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	a second l is equal to the value of powerControlLoopToUse2 in ConfiguredGrantConfig that is associated with the second srs-ResourceIndicator in rrc-ConfiguredUplinkGrant

-	else if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format 0_0 and for active UL BWP b of carrier f of serving cell

-	l is equal to the value of powerControlLoopToUse in ConfiguredGrantConfig

-	else, for a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of l∈0,1 is provided to the UE by powerControlLoopToUse in ConfiguredGrantConfig.

### *** Unchanged text is omitted ***

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c

### *** Unchanged text is omitted ***

-	If the PUSCH transmission is scheduled by DCI format 0_0, and

-	if two spatial settings from PUCCH-SpatialRelationInfo are activated for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, the UE uses the same RS resource index qd as for a PUCCH transmission with a spatial setting from the two spatial settings with lowest index in the PUCCH resource with the lowest index for active UL BWP b of each carrier f and serving cell c

-	else, if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index

-	If the PUSCH transmission is not scheduled by DCI format 0_0, and if the UE is provided enableDefaultBeamPL-ForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for an SRS resource set with an SRS resource associated with the PUSCH transmission

### *** Unchanged text is omitted ***

-	For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index qd from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first and second SRI values corresponding to each SRS resource set with usage set to 'codebook', respectively.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook' and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first SRI value corresponding to the first SRS resource set with usage set to 'nonCodebook', and the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook'.

-	If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines first and second RS resource indexes qd with respective first and second PUSCH-PathlossReferenceRS-Id value being equal to zero and one.

-	If the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero

where the RS resources are either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

### *** Unchanged text is omitted ***

### 7.2.1	UE behaviour

### *** Unchanged text is omitted ***

-	For the PUCCH power control adjustment state gb,f,ci,l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i

-	δPUCCH,b,f,c(i,l) is a TPC command value included in a DCI format associated with the PUCCH transmission for active UL BWP b of carrier f of the primary cell c that the UE detects for PUCCH transmission occasion i, or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3

-	l∈0,1 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo, or more than one sets of power control parameters for operation in FR1, and l=0 if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo, and more than one sets of power control parameters

-	If the UE obtains a TPC command value from a DCI format associated with the PUCCH transmission and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the l value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of l through the link to a corresponding p0-PUCCH-Id index

-  If the UE obtains a TPC command value from a DCI format associated with the PUCCH transmission, and if the UE is provided more than one sets of power control parameters for operation in FR1, and if the UE receives an activation command [11, TS 38.321] indicating one or two sets of the more than one sets of power control parameters, the UE determines the value of l based on the closedLoopIndex value in the one or two sets of power control parameters

-	If the UE obtains a TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the l value is provided by the closed loop indicator field in DCI format 2_2

### *** Unchanged text is omitted ***

### 7.7.1	Type 1 PH report

### *** Unchanged text is omitted ***

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as

[dB]

where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0.

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier.

If a UE transmits a PUSCH associated with a RS resource index qd, as described in clause 7.1.1, on active UL BWP b of carrier f of serving cell c in slot n and provides a Type 1 power headroom report for an actual PUSCH repetition associated with the RS resource index qd, the Type 1 power headroom report is for the first PUSCH repetition associated with the RS resource index qd that overlaps with slot n.

If a UE qdbfcnis provided twoPHRMode on active UL BWP b of carrier f of serving cell c and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE provides two Type 1 power headroom reports in a slot nqd, where

-	if the UE provides a first Type 1 power headroom report for an actual PUSCH repetition of a PUSCH transmission starting earliest in slot n that is associated with one SRS resource setqd,

-	if the UE transmits PUSCH repetitions associated with the other SRS resource setqd in slot n, the UE provides a second Type 1 power headroom report for a first actual PUSCH repetition associated with the other SRS resource setqd that overlaps with slot n

-	else, the UE provides a second Type 1 power headroom report for a reference PUSCH transmission associated with the other SRS resource set, whereqd

-	if the other SRS resource set is the first SRS resource set, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS or is obtained from PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=0

-	else, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 1, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 1 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS or is obtained from PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=1 if the UE is provided twoPUSCH-PC-AdjustmentStates, or l=0 if the UE is not provided twoPUSCH-PC-AdjustmentStates

-	else, if the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the first SRS resource setqd, the UE provides a Type 1 power headroom report for a reference PUSCH transmission associated with the second SRS resource set, whereqd

-	for the first Type 1 power headroom report, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, or is obtained from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=0.

-	for the second Type 1 power headroom report, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 1, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 1 if the UE is not provided enablePL-RS-UpdateForPUSCH-SRS, or is obtained from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 if the UE is provided enablePL-RS-UpdateForPUSCH-SRS, and l=1 if the UE is provided twoPUSCH-PC-AdjustmentStates or l=0 if the UE is not provided twoPUSCH-PC-AdjustmentStates

### *** Unchanged text is omitted ***