3GPP TSG-RAN WG1 Meeting #109-e 	R1-2205681

e-Meeting, May 9-20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0112 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on extension of current NR operation to 71 GHz in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture the endorsed Text Proposal 5-31-1-B in section 5-31 of R1-2205125 on CG-UCI. CG-UCI for FR2-2 is not defined in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incorporate the endorsed Text proposal 5-31-1-B in section 5-31 of R1-2205125, to clarify that CG-UCI applies to FR2-2 as well with UL to DL COT sharing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL to DL COT sharing for CG-PUSCH is not supported in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.3.2.1.3	CG-UCI

For CG-UCI bits transmitted on a CG PUSCH when the higher layer parameter cg-RetransmissionTimer is configured, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ul-toDL-COT-SharingED-Threshold and higher layer parameter cg-COT-SharingList are configured, or if both higher layer parameter ue-SemiStaticChannelAccessConfig and higher layer parameter cg-COT-SharingList are configured, or if higher layer parameter cg-COT-SharingList is configured in frequency range 2-2, where C is the number of combinations configured in cg-COT-SharingList; 1 if higher layer parameter ul-toDL-COT-SharingED-Threshold is not configured, and if higher layer parameter ue-SemiStaticChannelAccessConfig is not configured, and if higher layer parameter cg-COT-SharingOffset is configured; 0 otherwise; If a UE indicates COT sharing other than "no sharing" in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE's initiated COT such that the same DL starting point and duration are maintained. |