3GPP TSG-RAN WG1 Meeting #109-e	R1-2205653

e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0032 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to semi-static channel access procedures for PUSCH scheduled via RAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For semi-static channel access procedures when a UE can initiate a channel occupancy, the UE behaviour to determine the associated channel occupancy for a scheduled UL transmission in Clause 4.3.1.2.4 are specified according to whether the scheduled UL transmission and the corresponding scheduling DCI are confined within same gNB FFP or within different gNB FFP. The current description in the specification does not cover the case where the scheduled UL transmission is a PUSCH transmission that is not scheduled by a DCI, but it is scheduled via the corresponding RAR (during RACH procedure). - Some editorial corrections for clarity, alignment with similar clauses, and accuracy of the specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description to include the case that the schedule UL transmission is scheduled by a RAR message in Clause 4.3.1.2.4 and follow-up clasues 4.3.1.2.4.1 and 4.3.1.2.4.2. - Improve accuracy and readbility of the specification by editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of semi-static channel access procedures when a UE can initiate a channel occupancy, the UE behaviour to determine the channel occupancy association for a UL transmission that is scheduled by a RAR message would be unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.3	Channel access procedures for semi-static channel occupancy

Channel access procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc.

If a gNB provides UE(s) with higher layer parameters channelAccessMode-r16 ='semiStatic' by SIB1 or dedicated configuration for a serving cell, a periodic channel occupancy can be initiated by the gNB on a channel(s) within the bandwidth of the serving cell every Tx within every two consecutive radio frames, starting from the even indexed radio frame at i∙Txx∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx= period Tx=Periodin ms, is a higher layer parameter provided in SemiStaticChannelAccessConfig and i∈0,1,…,20Tx-1. A duration of  Tz=max0.05Tx , 100uμs at the end of a period is referred to as the idle duration of that period.

If the gNB additionally configures a UE with higher layer parameter ue-SemiStaticChannelAccessConfig consisting of ue-Period and ue-Offset, the UE can initiate a channel occupancy on a channel(s) within the bandwidth of the serving cell every Tu  = ue-Period in  ms with corresponding maximum channel occupancy time Tv=0.95Tu. The offset of the periodic channel occupancy is determined by To= ue-Offset as the number of symbols from the beginning of an even indexed radio frame to the start of the first period in that radio frame in which the UE can initiate a channel occupancy. A duration of Tw=max0.05Tu , 100μus  at the end of a period is referred to as the idle duration of that period.

For determining a Channel Occupancy Time based on semi-static channel access procedures, duration of any transmission gap within a period excluding the corresponding idle duration is counted in the channel occupancy time.In the following procedures in this clause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9μus, unless longer sensing duration is required (e.g. by level of regulation), in which case sensing is performed within a duration of Tsl=16μus. When sensing is performed within a duration of Tsl=16μus, the channel is considered to be idle if the channel is sensed to be idle for total of at least 5μus with at least 4μus of sensing occurring in the last 9μus time interval in the sensing duration. The corresponding  XThresh adjustment for performing sensing by a gNB or a UE is described in clauses 4.1.5 and 4.2.3, respectively.

### 4.3.1	Channel access procedures to initiate a channel occupancy

For semi-static channel occupancy, the procedures in Clause 4.3.1.1 are followed if ue-SemiStaticChannelAccessConfig is absent. Otherwise, the procedures in Clause 4.3.1.2 are applicable.

If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure.

#### 4.3.1.1	Channel occupancy initiated only by gNB

A channel occupancy that is initiated by a gNB and is shared with UE(s), satisfies the following:

-	The gNB shall transmit a DL transmission burst starting at the beginning of a period of duration Tx in which the channel occupancy is initiated immediately after sensing the channel to be idle for at least a sensing slot duration Tsl. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period.

-	The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16uμs.

-	The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16μus.

-	A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows:

-	If the gap between the UL and DL transmission bursts is at most 16μus,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel.

-	If the gap between the UL and DL transmission bursts is more than 16μus,  the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl within a 25μus interval ending immediately before transmission.

-	A UE may be indicated by the gNB to transmit UL transmission burst(s) within the channel occupancy time without sensing the channel or after sensing the channel to be idle for at least a sensing slot duration Tsl within a 25μus interval ending immediately before transmission.

-	The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100μus before the start of the next period.

#### 4.3.1.2	Channel occupancy initiated by gNB or UE

##### 4.3.1.2.1	Channel occupancy initiated by gNB and sensing procedures

The gNB initiates a channel occupancy in a period of duration  Tx if the gNB transmits a DL transmission burst starting at the beginning of the period immediately after sensing the channel to be idle for at least a sensing slot duration Tsl and ends the transmission of the DL transmission burst before the start of the idle duration of that period. When the gNB initiates a channel occupancy in that period, the gNB shall not transmit any transmission(s) within the idle duration of that period. A UE determines that the gNB has initiated the channel occupancy in that period by detection of a DL transmission burst(s) in that period or an indication by a DCI as described in Clause 4.3.1.2.4.

When a UL or DL transmission burst(s) is associated with the channel occupancy that is initiated in that period by the gNB, the following are applicable:

-	The UL or DL transmission burst(s) is confined within that period and ends before the start of the idle duration of that period.

-	If the gap between the DL transmission burst(s) and any previous transmission burst in that period is more than 16uμs, the DL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl immediately before the DL transmission.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is at most 16uμs, the DL transmission burst(s) may be transmitted without sensing.

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is more than 16uμs, the UL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl within a 25µs 25usinterval ending immediately before the UL transmission burst(s).

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is at most 16uμs, the UL transmission burst(s) may be transmitted without sensing.

##### 4.3.1.2.2	Channel occupancy initiated by UE and sensing procedures

A UE initiates a channel occupancy in a period of duration Tu if the UE transmits a UL transmission burst starting at the beginning of the period immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9usTsl and ends the transmission of the UL transmission burst before the start of the idle duration of that period. When the UE initiates a channel occupancy in that period, the UE shall not transmit any transmission(s) within the idle duration of that period.

When a UL or DL transmission burst(s) is associated with the channel occupancy that is initiated in that period by the UE, the following are applicable:

-	The UL or DL transmission burst(s) is confined within that period and ends before the start of the idle duration of that period.

-	If the gap between the UL transmission burst(s) and any previous transmission burst in that period is more than 16uμs, the UL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl immediately before the UL transmission burst(s).

-	If the gap between the UL transmission burst(s) and any previous DL transmission burst in that period is at most 16uμs, the UL transmission burst(s) may be transmitted without sensing.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is more than 16uμs, the DL transmission burst(s) may be transmitted if the channel is sensed to be idle for at least a sensing slot duration Tsl=9usTsl within a 25µs interval ending immediately before the DL transmission.

-	If the gap between the DL transmission burst(s) and any previous UL transmission burst in that period is at most 16uμs, the DL transmission burst(s) may be transmitted without sensing.

When a DL transmission burst(s) is associated with a channel occupancy that is initiated in a period of duration Tu by a UE, the DL transmission burst(s) shall include unicast user plane data or control information intended for the UE that has initiated the channel occupancy in that period. The gNB may include in the DL transmission burst(s) an additional transmission(s) intended to other UEs than the UE that has initiated the channel occupancy in that period or broadcast transmission(s), only if the gNB satisfies the condition that the detection of the additional DL transmission(s) at any UE will not be associated with a channel occupancy that is initiated by gNB following the procedures described in Clause 4.3.1.2.3 and 4.3.1.2.4.

When a UE is configured with a configured grant for which cg-RetransmissionTimer-r16 is provided and if the UE is provided cg-COT-SharingList-r16 by higher layers, the UE is configured with a table wherein each row is given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing is not available and other rows of the table each provides a channel occupancy sharing information. In this case, each configured grant PUSCH transmission includes 'COT sharing information' in CG-UCI as described in [10] that indicates a row index to the table, which is chosen by the UE independently of the CAPC information that the row may carry. If the gNB shares a channel occupancy initiated by the UE and detects a CG-UCI in slot n that includes 'COT sharing information', the gNB may transmit a transmission that follows the configured grant PUSCH transmission starting from slot n+O, where O= offset-r16 slots, for a duration of D=duration-r16 slots where duration-r16 and offset-r16 are higher layer parameters provided by CG-COT-Sharing-r16.

##### 4.3.1.2.3	Association with initiated channel occupancy for configured UL transmissions

When a UE is configured with a UL transmission, the UE follows the following procedures to determine if the configured UL transmission is associated with a channel occupancy that is initiated by the gNB or the UE.

-	If the configured UL transmission would occur at the beginning of a period of duration Tu  and would end before the idle duration corresponding to that period, the following is applied:

-	If the configured UL transmission would occur within a period of duration Tx and would end before the idle duration corresponding to that period and if the UE has already determined that the gNB has initiated a channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE assumes that the configured UL transmission is associated with a channel occupancy to be initiated by the UE.

-	If the configured UL transmission would occur at the beginning of a period of duration Tu  and would overlap with the idle duration corresponding to that period, the following is applied:

-	If the configured UL transmission would occur within a period of duration Tx and would end before the idle duration corresponding to that period, and if the UE has already determined that the gNB has initiated a channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE drops the configured UL transmission.

-	If the configured UL transmission would occur after the beginning of a period of duration Tu  and would end before the idle duration corresponding to that period, the following is applied:

-	If the UE has already initiated a channel occupancy in that period as described in Clause 4.3.1.2.2, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the UE.

-	If the UE has not already initiated a channel occupancy in that period as described in Clause 4.3.1.2.2, then if the configured UL transmission would occur within a period of duration Tx and would end before the idle duration corresponding to that period and if the UE has already determined that the gNB has initiated a channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB; otherwise, the UE drops the configured UL transmission(s).

-	If the configured UL transmission would occur after the beginning of a period of duration Tu  and would overlap with the idle duration corresponding to that period, the following is applied:

-	If the UE has not already initiated a channel occupancy in that period as described in Clause 4.3.1.2.2:

-	If the configured UL transmission would occur within a period of duration Tx and would end before the idle duration corresponding to that period and the UE has already determined that the gNB has initiated a channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE drops the configured UL transmission.

-	Otherwise, the UE drops the configured UL transmission.

If the configured UL transmission is a PUSCH with PUSCH repetition type B that does not overlap with an idle duration corresponding to a period of duration Tu or a period of duration Tx, the above procedures are applicable to its corresponding nominal repetition as described in [8]. Otherwise, the following procedures are applicable to its corresponding nominal repetition.

-	If the configured UL transmission would occur at the beginning of a period of duration Tu  and within a period of duration  the following is applied:

-	If the UE has already determined that the gNB has initiated channel occupancy in that period as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE assumes that the configured UL transmission is associated with a channel occupancy to be initiated by the UE.

-	If the configured UL transmission would occur after the beginning of a period of duration Tu and within a period of duration   the following is applied:

-	If the UE has already initiated a channel occupancy in the period of duration Tu as described in Clause 4.3.1.2.2, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the UE.

-	Otherwise,

-	if the UE has already determined that the gNB has initiated a channel occupancy in the period of duration  as described in Clause 4.3.1.2.1, the UE assumes that the configured UL transmission is associated with the channel occupancy that is initiated by the gNB.

-	Otherwise, the UE drops the configured UL transmission.

##### 4.3.1.2.4	Association with initiated channel occupancy for scheduled UL transmissions

When a UL transmission(s) is scheduled by a DCI or a RAR message, the scheduling DCI or the RAR message indicates the channel access parameters for the UL transmission(s) as described in [10]. Based on the DCI or the RAR message, the UE determines if the scheduled UL transmission(s) is associated with a channel occupancy that is initiated by the gNB or the UE, and whether sensing and CP extension are applicable.

###### 4.3.1.2.4.1	Intra-period scheduled UL transmissions

The procedures in this clause are applicable when a scheduled UL transmission and the corresponding scheduling DCI or the corresponding RAR transmission are confined within the same period of duration Tx corresponding to the carrier within which the UL transmission is scheduled and regardless of whether they are transmitted on the same carrier or different carriers.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the gNB, and the UE is indicated to perform the UL transmission without sensing, the UE applies CP extension if applicable following the procedures described in [11] and is expected to transmit the scheduled UL transmission without sensing as described in Clause 4.3.1.2.1.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the gNB and the UE is indicated to perform the UL transmission after sensing, the following are applied:

-	If the UE determines that the UL transmission follows a previous transmission after a gap of at most 16us, the UE is expected to transmit the UL transmission without sensing. Otherwise, the UE senses the channel for at least a sensing slot duration Tsl=9usTsl within a 25µs interval immediately before the scheduled UL transmission as described in Clause 4.3.1.2.1. If the channel is sensed to be idle, the UE is expected to transmit the scheduled UL transmission, and drop otherwise.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the UE, the following are applied:

-	If the UL transmission would occur at the beginning of a period of duration Tu, the UE is expected to sense the channel for at least a sensing slot duration Tsl=9usTsl immediately before the UL transmission as described in Clause 4.3.1.2.2. If the channel is sensed to be idle, the UE is expected to transmit the UL transmission, and drop otherwise.

-	If the UL transmission would occur after the beginning of a period of duration Tu

-	if the UE has not initiated a channel occupancy in that period as described in Clause 4.3.1.2.2, the UE is expected to drop the transmission;

-	otherwise, if the UE has already initiated a channel occupancy in that period as described in Clause 4.3.1.2.2:

-	if the UE determines that the UL transmission would follow a previous transmission after a gap of at most 16us, the UE is expected to transmit the UL transmission without sensing;

-	otherwise, if the UE determines that the UL transmission would follow a previous transmission, if any, after a gap of more than 16us, the UE is expected to sense the channel for at least a sensing slot duration Tsl=9usTsl immediately before the UL transmission as described in Clause 4.3.1.2.2 where the UE is expected to transmit the UL transmission if the channel is sensed to be idle, and drop the UL transmission otherwise.

###### 4.3.1.2.4.2	Cross-period scheduled UL transmissions

The procedures in this clause are applicable when a scheduled UL transmission and the corresponding scheduling DCI or the corresponding RAR transmission are confined within different periods of duration Tx corresponding to the carrier within which the UL transmission is scheduled and regardless of whether they are transmitted on the same carrier or different carriers.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the gNB and the UE is indicated to perform the UL transmission without sensing, the following are applied:

-	If the scheduled UL transmission starts after the beginning of a period of duration Tx corresponding to the carrier within which the UL transmission is scheduled and ends before the start of the idle duration corresponding to that period and if the UE has determined that a channel occupancy corresponding to that period is initiated by the gNB as described in Clause 4.3.1.2.1, the UE applies CP extension if applicable and is expected to transmit the scheduled UL transmission without sensing as described in Clause 4.3.1.2.1.

-	Otherwise, the UE drops the scheduled UL transmission.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the gNB and the UE is indicated to perform the UL transmission after sensing, the following are applied:

-	If the scheduled UL transmission starts after the beginning of a period of duration Tx corresponding to the carrier within which the UL transmission is scheduled and ends before the start of the idle duration corresponding to that period and if the UE has determined that a channel occupancy corresponding to the period is initiated by the gNB as described in Clause 4.3.1.2.1,

-	if the UE determines that the UL transmission follows a previous transmission after a gap of at most 16us, the UE is expected to transmit the UL transmission without sensing. Otherwise, the UE senses the channel for at least a sensing slot duration Tsl=9usTsl within a 25µs interval immediately before the scheduled UL transmission as described in Clause 4.3.1.2.1 where the UE is expected to transmit the scheduled UL transmission if the channel is sensed to be idle and drop the scheduled UL transmission otherwise.

-	Otherwise, the UE drops the scheduled UL transmission.

If the UE is indicated that the scheduled UL transmission is associated with a channel occupancy that is initiated by the UE the following are applied:

-	If the UL transmission would start at the beginning of a period of duration Tu  and would end before the start of the idle duration corresponding to that period, the UE is expected to sense the channel for at least a sensing slot duration Tsl=9usTsl  immediately before the UL transmission as described in Clause 4.3.1.2.2 where the UE is expected to transmit the UL transmission if the channel is sensed to be idle, and drop the UL transmission otherwise.

-	If the UL transmission would start after the beginning of a period of duration Tu  and would end before the start of the idle duration corresponding to that period,

-	if the UE has already initiated a channel occupancy in that period as described in Clause 4.3.1.2.2 the following is applied:

-	If the UE determines that the UL transmission follows a previous transmission after a gap of more than 16us, the UE is expected to sense the channel for at least a sensing slot duration Tsl=9usTsl  immediately before the UL transmission as described in Clause 4.3.1.2.2 where the UE is expected to transmit the UL transmission if the channel is sensed to be idle, and drop otherwise.

-	If the UE determines that the UL transmission follows a previous transmission after a gap of at most 16us, the UE is expected to transmit the UL transmission without sensing.

-	Otherwise, the UE drops the transmission.