3GPP TSG RAN WG1 #109-e	        		   R1-2205675

# e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0330 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on small data transmission in RRC_INACTIVE state for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on small data transmission in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove “except when PUSCH configuration period is less than 5 msec” from Table 19.1-1 in clause 19.1 because CG period values less than 5ms are not supported. - Replace “a number of SSB indexes” by “a number of preambles” in clause 19.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for small data transmissions in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1, 19.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged text is omitted ***

# 19	PUSCH transmission in RRC_INACTIVE state

## 19.1	Configured-grant based PUSCH transmission

A UE indicated to release a dedicated RRC connection can be provided one or more configurations by respective one or more ConfiguredGrantConfig, for configured grant Type 1 PUSCH transmissions on the initial UL BWP [12, TS 38.331]. For the remaining of this clause, PUSCH transmissions refer to configured grant Type-1 PUSCH transmissions for a configuration provided by ConfiguredGrantConfig.

A UE can be provided by sdt-SSB-Subset a number of SS/PBCH block indexes NPUSCHSS/PBCH to map to a number of valid PUSCH occasions for PUSCH transmissions over an association period. If the UE is not provided sdt-SSB-Subset, the UE determines NPUSCHSS/PBCH from the value of ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon. A PUSCH occasion for a PUSCH transmission is defined by a time resource and a frequency resource and is associated with a DM-RS provided by cg-DMRS-Configuration for the configuration of PUSCH transmissions.

An association period, starting from frame with SFN 0, for mapping NPUSCHSS/PBCH SS/PBCH block indexes, from the number of SS/PBCH block indexes, to valid PUSCH occasions and associated DM-RS resources is the smallest value in the set determined by the PUSCH configuration period provided by periodicity in ConfiguredGrantConfig according to Table 19.1-1 such that NPUSCHSS/PBCH SS/PBCH block indexes are mapped at least once to valid PUSCH occasions and associated DM-RS resources within the association period. A UE is provided a number of SS/PBCH block indexes associated with a PUSCH occasion and a DM-RS resource by sdt-SSB-perCG-PUSCH. If after an integer number of SS/PBCH block indexes to PUSCH occasions and associated DMRS resources mapping cycles within the association period there is a set of PUSCH occasions and associated DMRS resources that are not mapped to NPUSCHSS/PBCH SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PUSCH occasions and associated DMRS resources. An association pattern period includes one or more association periods and is determined so that a pattern between PUSCH occasions with associated DMRS resources and SS/PBCH block indexes repeats at most every 640 msec. PUSCH occasions and associated DMRS resources not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PUSCH transmissions.

Table 19.1-1: Mapping between PUSCH configuration period and SS/PBCH block to configured PUSCH resource association period

| PUSCH configuration period Tcg (msec) | Association period (number of PUSCH configuration periods) |
| --- | --- |
| 5 | {1, 2, 4, 8,16, 32, 64, 128} |
| 8 | {1, 2, 4, 5, 8, 10, 16, 20, 40, 80} |
| 10 | {1, 2, 4, 8,16, 32, 64} |
| 16 | {1, 2, 4, 5, 8,10,20,40} |
| 20 | {1, 2, 4, 8,16, 32} |
| 32 | {1, 2, 4, 5, 10, 20} |
| 40 | {1, 2, 4, 8, 16} |
| 64 | {1, 2, 5, 10} |
| 80 | {1, 2, 4, 8} |
| 128 | {1, 5} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

NPUSCHSS/PBCH SS/PBCH block indexes are mapped to valid PUSCH occasions and associated DMRS resources in the following order

-	first, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	second, in increasing order of PUSCH configuration period indexes

A PUSCH occasion is valid if it does not overlap with a valid PRACH occasion as described in clause 8.1.

For unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	does not precede a SS/PBCH block in the PUSCH slot, and

-	starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

-	if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion

-	is within UL symbols

-	starts at least Ngap symbols after a last downlink symbol, and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2

A UE determines a power of a PUSCH transmission as described in clause 7.1.1, where the UE obtains PLb,f,c(qd) using a RS resource from an SS/PBCH block with index associated with the PUSCH transmission.

A UE can be provided a USS set by sdt-CG-SearchSpace, or a CSS set by sdt-SearchSpace, to monitor PDCCH for detection of DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI for scheduling PUSCH transmission or of DCI format 1_0 with CRC scrambled by C-RNTI for scheduling PDSCH receptions [12, TS 38.331]. The UE may assume that the DM-RS antenna port associated with the PDCCH receptions, the DM-RS antenna port associated with the PDSCH receptions, and the SS/PBCH block associated with the PUSCH transmission are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties. The UE transmits a PUCCH with HARQ-ACK information associated with the PDSCH receptions as described in clause 9.2.1 using a same spatial domain transmission filter as for the last PUSCH transmission.

## 19.2	Random-access based PUSCH transmission

A UE indicated to release a dedicated RRC connection can be provided a configuration for a Type-1 and/or a Type-2 random access procedure on the initial UL BWP [12, TS 38.331]. PRACH occasions can have either a common configuration as, or a separate configuration from, PRACH occasions for Type-1 or Type-2 random access procedure as described in clause 8.1. The UE procedure is as described in clause 8, including clauses 8.1 through 8.4. The UE transmits a PRACH preamble with a power determined as described in clause 7.4.

For a common configuration of PRACH occasions and a Type-1 or a Type-2 random access procedure, a UE can be provided a number of contention based preambles per SS/PBCH block index per valid PRACH occasion by sdt-CB-PreamblesPerSSB-PerSharedRO or sdt-msgA-CB-PreamblesPerSSB-PerSharedRO, respectively. A PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle as determined by a PRACH mask index provided by sdt-SSB-SharedRO-MaskIndex or sdt-msgA-SSB-SharedRO-MaskIndex according to [11, TS 38.321].

A UE can be provided by sdt-SearchSpace a CSS set to monitor, after contention resolution as described in clause 8.4, PDCCH for detection of a DCI format 0_0 or DCI format 1_0 with CRC scrambled by C-RNTI for scheduling respective PUSCH transmissions or PDSCH receptions; otherwise, if the UE is not provided sdt-SearchSpace, the UE monitors PDCCH according to a Type1-PDCCH CSS set as described in clause 10.1. The UE may assume that the DM-RS antenna port associated with the PDCCH receptions, the DM-RS antenna port associated with the PDSCH receptions, and the SS/PBCH block associated with the PRACH transmission are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.