3GPP TSG-RAN WG4 Meeting #103-e	R4-2209446

Electronic Meeting, May 09- May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7157 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for RedCap for TS 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-5-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This big CR merges the mutiple endorsed draft CRs. The reason for change in each endorsed draft CR is copied below. R4-2211046 To introduce the inter-RAT NR measurement for RedCap UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of change in each endorsed draft CR is copied below. R4-2211046 - Add the section for inter-RAT NR measurement for RedCap UE. - Delete the brackets - Update the TBD based on latest agreements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequences if not approved for each endorsed draft CR are coppied below. R4-2211046 The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | R4-2211046 8.1.2.1.1d, 8.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------NEXT CHANGE----------------------------

##### 8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)

When monitoring of multiple inter-frequency E-UTRAN and inter-RAT (NR) using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (RSRP, RSRQ, RSTD, NR SS-RSRP, NR SS-RSRQ, NR SS-SINR, etc.) of detected cells on all the layers

The effective total number of frequencies excluding the frequencies of the PCell being monitored is Nfreq_RedCap, which is defined as:

Nfreq_RedCap = Nfreq_RedCap, E-UTRA + Nfreq_RedCap, NR

where

Nfreq_RedCap, E-UTRA is the number of E-UTRA carriers being monitored (FDD and TDD),

Nfreq_RedCap, NR is the number of NR inter-RAT carriers being monitored.

8.1.2.1.1d.1	Maximum allowed layers for multiple monitoring

The RedCap UE shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, 6 FDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 6 TDD E-UTRA inter-frequency carriers, and

-	Depending on UE capability, 6 NR SSB inter-RAT carriers configured by PCell, and

-	Depending on UE capability, 7 NR inter-RAT carriers including SSB and CSI-RS in total configured by PCell

In addition to the requirements defined above, the RedCap UE shall be capable of monitoring a total of at least 10 effective carrier frequency layers comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

----------------------END OF CHANGES----------------------------

----------------------NEXT CHANGE----------------------------

## 8.20	Inter-RAT NR Measurements for RedCap UE

### 8.20.1	Introduction

This clause contains requirements on the NR capable E-UTRAN RedCap UE regarding measurement in RRC_CONNECTED state. The requirements are specified for inter-RAT NR frequency measurements for serving E-UTRAN cell and measurement accuracies are specified in clause 9.

The UE shall be able to identify new inter-RAT NR cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-RAT cells if carrier frequency information is provided by the PCell, even if no explicit neighbour list with physical layer cell identities is provided.

Requirements in this clause are applicable to both E-UTRA FDD and E-UTRA TDD serving cell.

### 8.20.2	E-UTRAN FDD – NR measurements

8.20.2.1	Identification of a new NR cell

When measurement gaps are scheduled, the RedCap UE shall be able to identify a new detectable cell within Tidentify_irat_without_index_RedCap if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise, UE shall be able to identify a new detectable inter-RAT cell within Tidentify_irat_with_index_RedCap. The UE shall be able to identify a new detectable inter-RAT SS block of an already detected cell within Tidentify_irat_without_index_RedCap.

Tidentify_irat_without_index_RedCap = (TPSS/SSS_sync_irat_RedCap + T SSB_measurement_period_irat_RedCap) ms

Tidentify_irat_with_index_RedCap = (TPSS/SSS_sync_irat_RedCap + T SSB_measurement_period_irat_RedCap + TSSB_time_index_irat_RedCap) ms

Where:

TPSS/SSS_sync_irat_RedCap: it is the time period used in PSS/SSS detection given in table 8.20.2.1-1 and table 8.20.2.1-2 for 2 Rx RedCap, table 8.20.2.1-1a for 1 Rx RedCap.

TSSB_time_index_irat_RedCap: it is the time period used to acquire the index of the SSB being measured given in table 8.20.2.1-3 and table 8.20.2.1-4 for 2 Rx RedCap, table 8.20.2.1-3a for 1 Rx RedCap.

TSSB_measurement_period_irat_RedCap: equal to a measurement period of SSB based measurement given in table 8.20.2.1-5 and table 8.20.2.1-6 for 2 Rx RedCap, table 8.20.2.1-5a for 1 Rx RedCap.

Mpss/sss_sync_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, Mpss/sss_sync_irat_RedCap = 40 samples.

MSSB_index_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, MSSB_index_irat_RedCap = 40 samples.

Mmeas_period_irat_RedCap: For a 2 Rx RedCap supporting FR2 power class 7, Mmeas_period_irat = 40 samples.

Nfreq_RedCap is defined in clause 8.1.2.1.1d.

For per-FR measurement gap capable UE, when serving cells are in E-UTRA and measurement objects are only in FR2,

-	UE can perform such measurements without gap, and

-	UE fulfils the requirements for FR2 measurement objects based on effective MGRP = 20 ms.

Table 8.20.2.1-1: Time period for PSS/SSS detection for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-1a: Time period for PSS/SSS detection for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max([600]ms, 7 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max([600]ms, Ceil(7 *1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 7 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-2: Time period for PSS/SSS detection for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TPSS/SSS_sync_irat_RedCap |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | Mpss/sss_sync_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-3: Time period for time index detection for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 3 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-3a: Time period for time index detection for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(160ms, [6] Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil([6] 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | [6] DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-4: Time period for time index detection for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TSSB_time_index_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | MSSB_index_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

In the requirements, an NR cell is considered detectable when:

-	NR SS-RSRP related conditions in the accuracy requirements in Section 9.11.x are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-RSRQ related conditions in the accuracy requirements in Section 9.11.y are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-SINR related conditions in the accuracy requirements in Section 9.11.z are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50].

The RedCap UE operating in NR HD-FDD shall meet the current PSS/SSS detection requirements provided that at least 5 SMTC windows are available at the UE during the PSS/SSS detection time.

The RedCap UE operating in NR HD-FDD shall meet the current time index detection requirements provided that at least 3 SMTC windows are available at the UE during the time index detection time.

When measurement gaps are scheduled for NR measurements the UE physical layer shall be capable of reporting NR SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clause 9.11, with measurement period as shown in table 8.20.2.1-5 and 8.20.2.1-6 for 2Rx UE, table 8.20.2.1-5a for 1Rx UE:

Table 8.20.2.1-5: Measurement period for inter-RAT measurements for 2 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-5a: Measurement period for inter-RAT measurements for 1 Rx RedCap (Frequency range FR1)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | 8 DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.20.2.1-6: Measurement period for inter-RAT measurements for 2 Rx RedCap (Frequency range FR2)

| Condition NOTE1 | TSSB_measurement_period_irat_RedCap |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_irat_RedCap Max(MGRP, SMTC period)) Nfreq_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_irat_RedCap) Max(MGRP, SMTC period, DRX cycle)) Nfreq_RedCap |
| DRX cycle > 320ms | Mmeas_period_irat_RedCap DRX cycle Nfreq_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

The 2 Rx RedCap UE shall be capable of performing NR SS-RSRP, SS-RSRQ and SS-SINR for up to 7 NR carrier frequencies.

- For each RAT E-UTRAN FDD-NR layer on FR1 or FR2, the UE shall be capable of monitoring at least 4 cells.

- For each RAT E-UTRAN FDD-NR layer on FR1, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer.

- For each RAT E-UTRAN FDD-NR layer on FR2, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer. The UE shall be capable of monitoring at least one SSB per cell.

The 1 Rx RedCap UE shall be capable of performing NR SS-RSRP, SS-RSRQ and SS-SINR for up to 6 NR carrier frequencies.

- For each RAT E-UTRAN FDD-NR layer on FR1 or FR2, the UE shall be capable of monitoring at least 4 cells.

- For each RAT E-UTRAN FDD-NR layer on FR1, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer.

- For each RAT E-UTRAN FDD-NR layer on FR2, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer. The UE shall be capable of monitoring at least one SSB per cell.

The NR SS-RSRP measurement accuracy for all measured NR cells shall be as specified in clause 9.11.x. The NR SS-RSRQ measurement accuracy for all measured NR cells shall be as specified in clause 9.11.y. The NR SS-SINR measurement accuracy for all measured NR cells shall be as specified in clause 9.11.z.

The RedCap UE operating in NR HD-FDD shall meet the current measurement requirements provided that at least 5 SMTC windows are available at the UE during the measurement period.

8.20.2.2	Periodic Reporting

Reported measurements in periodically triggered measurement reports shall meet the requirements in clause 9.

8.20.2.3	Event Triggered Reporting

Reported measurements in event triggered measurement reports shall meet the requirements in clause 9.

The UE shall not send any event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The measurement reporting delay is defined as the time between any event that will trigger a measurement report until the UE starts to transmit the measurement report over the Uu interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is twice the TTI of the uplink DCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_irat_without_index_RedCap or Tidentify_irat_with_index_RedCap defined in Clause 8.20.2.1 for the minimum requirements. When L3 filtering is used or IDC autonomous denial or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

If an NR cell which has been detectable at least for the time period Tidentify_irat_without_index_RedCap. or Tidentify_irat_with_index_RedCap defined in clause 8.20.2.1 for the minimum requirements and then triggers the measurement report as per TS 36.331 [2], the event triggered measurement reporting delay shall be less than TSSB_measurement_period_irat_RedCap defined in clause 8.20.2.1 provided the timing to that cell has not changed more than 3200 Tc while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

8.20.2.4	Event-triggered Periodic Reporting

Reported measurements contained in event triggered periodic measurement reports shall meet the requirements in clause 9.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 8.20.2.3.

### 8.20.3	E-UTRAN TDD – NR measurements

Requirements in this clause shall apply for NR capable E-UTRAN TDD RedCap UE.

The requirements in clause 8.20.2 also apply for this section.

----------------------END OF CHANGES----------------------------