3GPP TSG-RAN WG4 Meeting # 103-e	R4-2211124

# Electronic Meeting, May 9 - May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7156 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maintenance for NB-IoT R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - 200 ms and 50ms were agreed to be the minimum length of a single measurement occasion. However, it is mistakenly used in the condition for interval between two samples. If a measurement occasion is 200 ms, it means the interval between samples within this occasion is always shorter than 200 ms, and UE will always restart measurement according current description. - It is redundant to state detection occasion containing at least least subframes #0, #4, or #9 as it is already stated that the length of at least 200 ms containing NPSS and NSSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the lower bound of 200 ≤ Ta,i and 50 ≤ Tb,i - Remove the “containing at least least subframes #0, #4, or #9” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1 >

#### 8.14.6.4	Inter-frequency neighbour cell measurements

The UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_NB1-NC when the criteria for inter-frequency measurement is fulfilled [1]. An inter frequency cell is considered to be detectable according to NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot defined in Annex B.x.y for a corresponding Band.

Tidentify _inter_NB1-NC = Tdetect_inter NB1-NC + Tmeasure _inter NB1-NC

Where

Tdetect_inter_NB1-NC=iNMin(5000, Ta,i) ms

-	N = 70,

-	Ta,i is the interval between available measurement samples in measurement occasions (MOdetect_inter_NB1-NC) for inter-frequency detection, where

40 ms ≤  Ta,i ≤ 5000 ms

-	The UE shall restart the cell detection when the interval between two samples are larger than 5000 ms.

-	The UE is not required to monitor NPSS/NSSS more frequent than once per 40ms.

-	MOdetect_inter_NB1-NC are time occasions containing NPSS/NSSS and fulfil the following conditions:

-	Resources on which the UE is not scheduled for data transmission or reception,

-	Resources on which the UE is not required to do NPDCCH monitoring,

-	Resources occurring during the DRX inactive period

-	Length of MOdetect_inter_NB1-NC  is at least 200 ms.

-	The inter-frequency detection requirements apply when Tdetect_inter_NB1-NC ≤ 60 seconds per inter-frequency carrier.

Tmeasure_inter_NB1-NC is the physical layer measurement period of NRSRP on the detected inter-frequency cell as defined below:

Tmeasure_inter_NB1-NC=iMMin(5000, Tb,i) ms

-	M = 60 for NRS-based RRM measurement and M = 40 for NSSS based RRM measurement,

-	Tb,i is the interval between available measurement samples in measurement occasions (MOmeasure_inter_NB1-NC) for inter-frequency measurement, where

20 ms ≤  Tb,i ≤ 5000 ms for NRS based measurement or

40 ms ≤ Tb,i ≤ 5000 ms for NSSS-based measurement

-	The UE shall restart the measurement when the interval between two samples are larger than 5000 ms.

-	The UE is not required to monitor NRS more frequent than once per 20ms for NRS-based measurement and NSSS more frequent than 40 ms for NSSS-based measurement.

-	MOmeasure_inter_NB1-NC are time occasion containing at least NRS or NSSS that fulfil the following conditions:

-	Resources on which the UE is not scheduled for data transmission or reception,

-	Resources on which the UE is not required to do NPDCCH monitoring,

-	Resources occurring during the DRX inactive period,

-	Length of MOmeasure_inter_NB1-NC  is at least 50 ms.

-	The inter-frequency measurement requirements apply when Tmeasure_inter_NB1-NC ≤ 50 sec seconds per inter-frequency carrier.

When UE is monitoring multiple carriers, Tidentify_inter_NB1-NC = Tdetect_NB1-NC + Tmeasure_NB1-NC, where Tdetect_NB1-NC = Tdetect _intra_NB1-NC +Nfreq* Tdetect_inter_NB1-NC and Tmeasure_NB1-NC = Tmeasure _intra_NB1-NC +Nfreq* Tmeasure_inter_NB1-NC. Nfreq is number of inter-frequency carriers to be measured according to the measurement capability, and Tdetect_intra_NB1-NC and Tmeasure_intra_NB1-NC are defined in clause 8.14.6.3.

<End of Change 1 >

<Start of Change 2 >

### 8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM

The requirements in this clause shall apply for UE Category NB2 supporting 16QAM for unicast NPDSCH as defined in TS 36.331 [2] when triggered by the MAC-CE command as specified in TS 36.321 [17].

The DL channel quality provides the serving eNB with the following information with the parameters specified in Table 8.14.5-1:

-	The reported candidateRep should correspond to the minimum number of NPDCCH repetitions that achieves a hypothetical NPDCCH block error rate no larger than 1%, and correspond to the NPDSCH transport block size that achieves a NPDSCH block error rate no larger than 10%.

-	If the candidateRep entry corresponds to NPDCCH repetitions level X and NPDSCH transport block size Y, then no other candidate with NPDCCH repetitions level ≤ X and NPDSCH transport block size ≥ Y should satisfy the block error rate conditions for both NPDCCH and NPDSCH.

-	If there is no candidateRep entry to satisfy the condition, UE should report ‘noMeasurement’.

The report mapping is defined in Table 9.1.22.17-1.

Table 8.14.5-1: NPDCCH transmission parameters for downlink quality reporting

| Parameters | Values |
| --- | --- |
| DCI format | Format N1 |
| Number of information bits (excluding CRC) | 23bits |
| System bandwidth | 200kHz |
| Aggregation level | 2 |
| DRX | OFF |

The reported candidateRep value shall be derived from the channel quality measured from the time UE finishes the decoding of Downlink Channel Quality report MAC CE to the end of NPDCCH period which carries the uplink grant of channel quality report for measurement of DL channel quality of the configured carrier. During the measurement period, UE expects that NRS is transmitted in at least 4 consecutive subframes.

<End of Change 2 >