3GPP TSG-RAN4 Meeting #103e	R4-2211071

# Electronic Meeting,  9th May - 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2293 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR for SCG activation on 38.133 R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When DRX is not in use, RLM requirement TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS defined in Table 8.1.3.2-3 and Table 8.1.3.2-4 for deactivated PSCell should have no relevance to TDRX |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete TDRX in RLM requirement TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS defined in Table 8.1.3.2-3 and Table 8.1.3.2-4 for the item “no DRX”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

8.1.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS ms period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS ms evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS ms period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS ms evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-2 for FR2 with scaling factor N=1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-3 for FR1 (deactivated PSCell).

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-4 for FR2 (deactivated PSCell) with scaling factor N=1.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For a UE that supports either concurrent measurement gaps, pre-MG gaps or NCSG, measurement gaps in this section includes any configured and active gap.

Editor’s note: the term “[measurement gap]” in this clause could be either the legacy gap or the VIL or NCSG. RAN4 to further study whether a better terminology can be used to replace [measurement gap].

For FR1, for a UE not supporting [concurrent gaps] or when the UE is not configured with concurrent gaps,

-	P=11-TCSI-RSxRP, when in the monitored cell there are[measurement gaps] configured for intra-frequency, inter-frequency or inter-RAT measurements, and these [measurement gaps] are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no [measurement gaps] overlapping with any occasion of the CSI-RS.

For FR2, for a UE not supporting [concurrent gaps] or when the UE is not configured with concurrent gaps,

-	P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with [measurement gap] and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is not overlapped with [measurement gap] and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the RLM-RS resource is not overlapped with [measurement gap] and RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with [measurement gap] and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with [measurement gap] and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with [measurement gap] and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with [measurement gap] and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with [measurement gap]

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with measurement gap and the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < xRP)

When concurrent gaps are configured,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target RLM-RS.

where,

Psharing factor = 1, if the RLM-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

Psharing factor = 3, otherwise.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

When a measurement gap is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be as overlapped with the [measurement gap] if it overlaps a measurement gap occasion, and

-	xRP = MGRP

When NCSG is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be as overlapped with the [measurement gap] if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

and

-	xRP = VIRP

If the UE is configured with Pre-MG, an RLM-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps are configured, an RLM-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to [9.1.2B].

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and [measurement gap] configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of Mout and Min used in Table 8.1.3.2-1, Table 8.1.3.2-2, Table 8.1.3.2-3 and Table 8.1.3.2-4 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P)×TCSI-RS) | Max(100, Ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P) × TDRX | Ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N)×TCSI-RS) | Max(100, Ceil(Min×P×N) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N) × TDRX | Ceil(Min×P×N) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Table 8.1.3.2-3: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P) × measCyclePSCell | Ceil(Min×P) × measCyclePSCell |
| DRX ≤ [320]ms | Ceil(Mout×P) × Max(1.5 × TDRX, measCyclePSCell) | Ceil(Min×P) × Max(1.5 × TDRX, measCyclePSCell) |
| DRX > [320]ms | Ceil(Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(Min×P) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement period of deactivated PSCell. |  |  |

Table 8.1.3.2-4: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P×N) × measCyclePSCell | Ceil(Min×P×N) × measCyclePSCell |
| DRX ≤ [320]ms | Ceil(Mout×P×N) × Max(1.5 × TDRX, measCyclePSCell) | Ceil(Min×P×N) × Max(1.5 × TDRX, measCyclePSCell) |
| DRX > [320]ms | Ceil(Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(Min×P×N) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement period of deactivated PSCell. |  |  |

<< End of change 1>>