3GPP TSG-WG4 Meeting #103-e	R4-2208599

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.837 | CR | 0003 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revision of General part of MPR of 38.837 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-Core |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The general part of MPR is not complete and not adaptable with the key part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the current part is for early stage and the key evaluationa and results are in later clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general part of MPR is updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

4.4	Maximum output power reduction

4.4.1	General

4.4.1.1	Agreements

Here are some early stage agreements before a comprehensive evaluation.

It has been agreed that MPRs for UL-MIMO would need to be re-visited.

For MPR Requirement for Transparent TxD:

RAN4 agree MPR defined for TxD is applied to the total output power rather than at each antenna connector

For TxD and UL-MIMO, it has been agreed:

2 Tx MPR should be the same MPR requirement for TX Diversity and UL MIMO for the same power class.

For eMIMO and ULFPTx related, The agreement reached is as following:

“Chair: It is agreed that one set of MPR requirements should be adopted for both UL MIMO (including ULFPTx) and TxD”

WF R4-2105331 is agreed MPR evaluation assumptions, and it is agreed that an evaluation is necessary to further progress the MPR work.

The key agreements and the related evaluation are incorporated in clause 4.4.2, 4.4.3 etc.

4.4.1.2	Study process

Here are some early stage studies before a comprehensive evaluation.

In RAN4#94-e-bis, in the WF R4-2005216, it has been agreed

-	R15 UL MIMO emission requirements shall apply to UE level.

-	Relating MPRs are need to be re-visited.

In RAN4#95-e, a WF R4-2008465 was agreed in which:

Issue 3-3-2: Unwanted emissions for Transparent TxD: MPR study

Possible WF:

Simulation/measurement assumptions for MPR study for 2Tx UE’s

Follow 29 dBm WI assumptions in R4-2005190

Two 20dBm Tx chains are not precluded

Two 23dBm Tx chains are not precluded

Two 26dBm Tx chains are precluded

MPRs are defined for each power class separately

PC3 = 2x20dBm

PC2 = 2x23dBm

In RAN4#96-e, the agreed WF R4-2011768 has the following contents:

MPR Requirement for Transparent TxD

RAN4 agree MPR defined for TxD is applied to the total output power rather than at each antenna connector

In the meantime, for eMIMO WI, the MPR was an remaining issue:

“The Chairmain commented that for PC2 and PC3, MPR issues related to 2TX, including UL-MIMO, uplink full power transmission, and TxD, will be further discussed in TEI16.”

In RAN4#97-e meeting, the transparent TxD was discussed under TEI16 as documented in [R4-2016959] and a WF [R4-2016830] was also agreed.

MPR for Transparent and UL MIMO

Whether 2 Tx MPR should be the same MPR requirement for TX Diversity and UL MIMO for the same power class.

Agreement

-	Option 1: Yes

For eMIMO and ULFPTx related, there is very few maintenance remains and only MPR was discussed. The agreement reached is as following:

“Chair: It is agreed that one set of MPR requirements should be adopted for both UL MIMO (including ULFPTx) and TxD”

In RAN4#98-e-bis, the MPR was extensively discussed, but no agreements can be reached, but an evaluation is agreed to be started:

CR related - MPR

Proposals:

Option 1: As in last meeting’s Endorsed CR R4-2107307

Option 2: Base on the proposals in R4-2104538

1.5dB offset for Edge and outer, 0.5dB offset for inner compared to 1Tx

Option 3: Reconsider separating MPR requirements for UL-MIMO and TxD

Also consider A-MPR impact in next issue and as in R4-2107283

Option 4: Keep the same MPR with 1Tx

Option 5: Other solution

Agreements (GTW) :

RAN4 to start a evaluation campaign to derive the MPR values for both UL-MIMO and TxD, with agreed evaluation assumptions and UE implementations. Decisions will be made in the May meeting

Another WF R4-2105331 is MPR evaluation assumptions, it is agreed that an evaluation is necessary to further progress the MPR work, and a detailed assumptions were agreed. The contents were not listed.

Later studies are not listed here in a meeting by meeting minor. The key evaluation and agreements are incorporated in clause 4.4.2, 4.4.3 etc.

<End of Change>