3GPP TSG RAN meeting #96										RP-221699

Budapest, Hungary, June 6-9, 2022

## Status Report to TSG

Agenda item:	9.4.4.8

| WI / SI Name | Further RRM enhancement for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh2 |  |  |  |  |
| Unique ID | 890157 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213067 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 50% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Corporation |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Qiuge Guo |
|  | Company | CATT |
|  | Email | guoqiuge@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #103-e meeting

(1) Work plan for R17 FeRRM performance part

The Work plan for R17 FeRRM performance part was agreed in R4-2207766.

(2) SRS antenna port switching

Following CRs were agreed and endorsed.

| R4-2207767 | CR on SRS antenna port switching in TS38.133 | Apple | Agreed |
| --- | --- | --- | --- |
| R4-2208092 | 38.133 CR on introduction of SRS antenna port switching (resubmission) | Nokia, Nokia Shanghai Bell | Agreed |
| R4-2210993 | draftCR on interruptions at SRS antenna switching | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2210995 | CR on SRS antenna port switching requirements 36.133 | Huawei, Hisilicon | Agreed |

The test case list and way forward was agreed in R4-2210996.

Issue 3-1-1: MR-DC or CA modes in SRS antenna port switching testing

Agreement in GTW (May 13th)

- In SRS antenna port switching testing, only following MR-DC or CA modes would be considered:

  - NR-SA (FR1+FR1 NR CA) and EN-DC (LTE Pcell + FR1 NR PSCell + FR1 NR Scell)

    - If UE passes the performance requirements of EN-DC, then the performance for NE-DC is expected to be guaranteed.

Issue 3-1-2: scenarios for SRS antenna port switching testing

Agreement in GTW (May 13th)

- Interruption requirement (symbol-level) for scenario 1 sync case

- Interruption requirement (slot-level) for scenario 2

Issue 3-1-3: SRS antenna port switching together with SRS carrier switching

Agreements:

- No need to have test case for SRS antenna port switching + SRS carrier-based switching.

- SRS antenna switching requirement only applies when SRS antenna switching is configured without SRS carrier switching.

Issue 3-1-4: Other clarification on the testing scope

Agreements:

- Combination of FR1+FR2 or LTE+ FR1 + FR2 would not be tested for SRS antenna port switching requirement, until the applicability of FR1-FR2 joint test case is available.

- Only test inter-band NR CA for the case of interruption to NR cells, while DL CA and UL CA can be both considered.

- Do not test interruption by SRS ant switching if aggressor CC is an FR2 CC.

Issue 3-2-2: General configuration – synch/async

Issue 3-2-2-1: can we agree the 1T2R configuration as a baseline, and add 2T4R and 1T4R cases later?

- Discuss in issue 3-2-2-4

Issue 3-2-2-2 (this issue is somewhat related with issue 3-2-5): for scenario 1(1 SRS symbol in a slot):

Agreements:

For scenario 1(1 SRS symbol in a slot):

- Periodic SRS resource set is used in the test cases to verify that the length of each interruption is less than the interruption requirements. The interval between SRS sources in the test cases are configured larger than 3/6 slots for 15 KHz/30KHz to align with SRS configuration in Scenario 2.

- configure one resource with one symbol in a slot

Issue 3-2-2-3: For scenario 2(more than 1 SRS symbol for antenna port switching in a slot):

Agreements:

- Configure two resources in a slot and each resource has one symbol

Issue 3-2-2-4: can we agree the 1T2R/2T4R/1T4R resource configuration as following?

Agreements:

- SRS.x1 is for 15kHz, SRS.x2 is for 30kHz

- For scenario 1, each SRS slot has 1 SRS resource; for scenario 2, each SRS slot has 2 SRS resources. Each SRS resource has 1 symbol, no repetition

- Offset is determined by TDD pattern: consider special slot for SRS transmission

- The first table is the common configuration for each SRS,

|  | SRS.x1 | SRS.x2 |  |
| --- | --- | --- | --- |
| Field | Value | Value | Comment |
| c-SRS | 12 | 24 |  |
| b-SRS | 0 | 0 |  |
| b-hop | 0 | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | antennaSwitching |  |
| nrofSymbols | 1 | 1 |  |
| repetitionFactor | n1 | n1 | without repetition. |
| transmissionComb | n2 | n2 |  |
| combOffset | 0 | 0 | transmissionComb setting |
| cyclicShift | 0 | 0 |  |
| resourceType | Periodic | Periodic |  |

- the following tables are configurations for each SRS, scenario and xTyR configuration.

| 15kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, [2] | sl40, [7] | sl40, [2] | sl40, [7] | sl40, [2] | sl40, [7] | sl40, [12] | sl40, [17] |

| 15kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl40, [2] | sl40, [2] | sl40, [2] | sl40, [2] | sl40, [2] | sl40, [2] | sl40, [7] | sl40, [7] |

| 30kHz, scenario 1 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, [2] | sl80, [12] | sl80, [2] | sl80, [12] | sl80, [2] | sl80, [12] | sl80, [22] | sl80, [32] |

| 30kHz, scenario 2 | 1T2R |  | 2T4R |  | 1T4R |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| srs-ResourceId | 0 | 1 | 0 | 1 | 0 | 1 | 2 | 3 |
| startPosition | 5 | [2] | 5 | [2] | 5 | [2] | 5 | [2] |
| nrofSRS-Ports | port1 | port1 | port2 | port2 | port1 | port1 | port1 | port1 |
| periodicityAndOffset-p | sl80, [2] | sl80, [2] | sl80, [2] | sl80, [2] | sl80, [2] | sl80, [2] | sl80, [12] | sl80, [12] |

Issue 3-2-3: General configuration – SCS and BW

Agreements:

- for configuration of test cases for SRS antenna port switching, to consider following cases:

  - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

  - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

  - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

- FFS:

  - Option 1: above configurations applies for aggressor cell, FFS for victim cell

  - Option 2: above configurations applies for both aggressor cell and victim cell, and such configuration may be different between aggressor cell and victim cell.

  - Option 3: above configurations applies for both aggressor cell and victim cell, and such configuration is same between aggressor and victim

Issue 3-2-4: General configuration – band combination selection for CA/DC

Agreements:

- Operating bands of aggressor CC and victim CC should be selected based on UE capability of txSwitchImpactToRx or txSwitchWithAnotherBand.

Test case list:

| TC index | TC | DC/CA mode | Purpose | Section ID | Candidate companies |
| --- | --- | --- | --- | --- | --- |
| 1 | E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in synchronous EN-DC | EN-DC tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PCC and FR1 NR SCC for scenario 1 sync case | A.4.5.2.x1 | Nokia |
| 2 | E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | EN-DC tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on LTE PCC and FR1 NR SCC for scenario 2 | A.4.5.2.x3 | Huawei |
| 3 | NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | NR standalone tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on FR1 NR SCC for scenario 1 (only sync case) | A.6.5.2.x1 | vivo |
| 4 | NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | NR standalone tests with all NR cells in FR1 | To verify the interruption of FR1 NR SRS antenna switching on FR1 NR SCC for scenario 2 | A.6.5.2.x2 | Intel |

(3) HO with PSCell

The way forward was agreed in R4-2210583. Following LS and CRs were agreed and endorsed.

| R4-2210583 | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo | Approved |
| --- | --- | --- | --- |
| R4-2211170 | LS on handling of RACH occasion collision between PCell and PSCell for HO with PSCell for NR-U | Ericsson | Approved |
| R4-2210997 | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 | Apple | Agreed |
| R4-2210998 | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | Apple | Agreed |
| R4-2210999 | Completing requirement of HO with PSCell | CATT | Agreed |
| R4-2211191 | Correction on HO with PSCell requirements in 36133 | Nokia, Nokia Shanghai Bell | Agreed |
| R4-2211000 | CR on R17 core requirements for HO with PSCell | vivo | Agreed |
| R4-2211001 | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson | Agreed |
| R4-2211002 | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson | Agreed |
| R4-2211006 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT | Endorsed |
| R4-2211007 | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) | OPPO | Endorsed |
| R4-2211009 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, Hisilicon | Endorsed |
| R4-2211010 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo | Endorsed |
| R4-2211012 | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell | Ericsson | Endorsed |

Sub-topic 1-1 Core requirements for HO with PSCell for NR-U

<Agreement>:

- For HO with PSCell when NR PSCell is in FR1 unlicensed spectrum (band n46), requirements shall be defined for NR SA to EN-DC and EN-DC to EN-DC cases

- For HO with PSCell from EN-DC to EN-DC, only parallel processing is considered for NR-U.

- For HO with PSCell from NR-SA to EN-DC, both parallel and sequential processing shall be considered for NR-U.

- For HO with PSCell from EN-DC to EN-DC and from NR-SA to EN-DC with parallel processing, Tprocessing is 25ms.

- For HO with PSCell from NR-SA to EN-DC with sequential processing, Tprocessing is 30ms.

- PRACH preamble transmission is the end point for PCell HO and PSCell addition/change.

- Separate end points are assumed for PCell HO and PSCell addition/change

- The RO transmission failure due to both UL LBT failure and RO collision with PCell UL channel shall be limited by PREAMBLE_TRANSMISSION_COUNTER, and the UE behavior when PREAMBLE_TRANSMISSION_COUNTER reaches the preambleTransMax should follow TS38.321.

  - LS to RAN1 to confirm the agreement.

Sub-topic 2-1 Test cases coverage, design and configurations

<Agreement>:

- Define test cases for the scenarios including from NR SA to EN-DC, from EN-DC to EN-DC (LTE+FR1 NR), from NE-DC to NE-DC.

- Define test cases to cover both parallel processing and sequential processing cases.

- Do not verify all intra-frequency and inter-frequency HO scenarios in the HO w PSCell test cases.

Issue 2-2: General test cases configuration

- If target PSCell is unknown, single SMTC configuration of target unknown PScell from either source Pcell MO or source PSCell MO can be configured.

- The requirement for HO with PSCell can be verified by testing the transmission time of PRACH to target PCell and target PSCell.

- For the test configuration of test cases for HO with PSCell, following configurations is considered:

  - For FR1

    - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

    - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

    - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

  - For FR2

    - 120 kHz SSB SCS with 100 MHz bandwidth for TDD

    - Note：Whether FR1+FR2 test cases will be introduced in Rel-17 are discussed separately.

Issue 2-3: Test cases configuration for HO with PSCell from EN-DC to EN-DC

- The test can consist of three successive time periods, T1, T2 and T3. The target cells turn off during T1, and turn on during T2. UE will send Event A3 reports and receive RRC message implying handover with PSCell during T2, and perform the handover with PSCell during T3.

- For known target PCell and known target PSCell

  - The test handover requirement for handover with PSCell is defined as 60ms.

  - The test requirement for PSCell change delay is defined as 87ms.

Issue 2-4: Test cases configuration for HO with PSCell from NR SA to EN-DC

- For the test case of HO with PSCell from NR SA to EN-DC, the sequential processing case should be verified when target PCell and target PSCell are unknown and the target PSCell is in FR1.

Issue 2-2: General test cases configuration

- Option 1: For the case when both target PCell and target PSCell are known, the test system sends RRC message implying HO with PSCell only after it receives 2 MRs.

Issue 2-5: Test cases list for HO with PSCell

- Option 1

  - TC #1: From SA to EN-DC (All cells are in FR1)

    - PCell: unknown target PCell

    - PSCell: unknown target PSCell

    - Purpose: To verify delay requirements for sequential processing

  - TC #2: From SA to EN-DC (All cells are in FR1)

    - PCell: known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for parallel processing

  - TC #5: From NE-DC to NE-DC (All cells are in FR1)

    - PCell: Intra-frequency handover from FR1 to FR1, known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for known PCell and known PSCell combination

  - TC #6: From NE-DC to NE-DC (All cells are in FR1)

    - PCell: Inter-frequency handover from FR1 to FR1, unknown target PCell

    - PSCell: unknown target PSCell

    - Purpose: To verify delay requirements for unknown PCell and unknown PSCell combination

  - TC #9: From EN-DC to EN-DC (All cells are in FR1)

    - PCell: E-UTRA intra-frequency handover, known target PCell

    - PSCell: known target PSCell

    - Purpose: To verify delay requirements for known PCell and known PSCell combination

  - TC #11: From EN-DC to EN-DC (All cells are in FR1)

    - PCell: E-UTRA inter-frequency handover, unknown target PCell

    - PSCell: unknown target PSCell

    - Purpose: To verify delay requirements for unknown PCell and unknown PSCell combination

(4) PUCCH SCell activation

The way forward was agreed in R4-2211208. Following CRs were agreed and endorsed.

| R4-2211209 | 38.133 draftCR on PUCCH SCell activation delay | Nokia | Endorsed |
| --- | --- | --- | --- |
| R4-2211014 | CR on PUCCH SCell activation and deactivation delay requirements | MTK | Agreed |
| R4-2211015 | CR on PUCCH SCell activation | Huawei | Agreed |
| R4-2211016 | CR on SCell activation/deactivation with PUCCH | Ericsson | Agreed |

Issue 1-1-1: The known condition of PL-RS

Agreements:

- the known condition of PL-RS for known PUCCH SCell could be defined as:

  - The pathloss reference signal is known for known PUCCH SCell during activation if the following conditions are met.

    - PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource for L3 measurement reporting.

    - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

      - SNR of the target pathloss reference signal≥-3dB

    - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

      - SNR of the associated SSB ≥-3dB

  - Otherwise, the pathloss reference signal is unknown.

- the known condition of PL-RS for unknown PUCCH SCell could be defined as:

  - The pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met.

    - PUCCH SCell activation command is received within 1280 ms upon the last transmission of the RS resource for for beam reporting or measurement.

    - The target pathloss reference signal remains detectable during the PUCCH SCell activation period

      - SNR of the target pathloss reference signal≥-3dB

    - The associated SSBs with the target pathloss reference signal remain detectable during the PUCCH SCell activation period

      - SNR of the associated SSB ≥-3dB

Issue 1-1-3: Update the PUCCH Scell activation delay requirements about PL-RS measurement in FR2 for invalid TA case?

Agreements:

- Update the previous agreement about FR2 PL-RS measurement, from a sequential processing to a parallel processing, i.e. PL-RS can be measured in parallel with “DL CSI-RS reception/processing” and “UL TA acquisition”.

  - Previous agreement: THARQ+Tactivation_time+[X]+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3)}+TCSI_reporting_after

  - Proposed update: THARQ+Tactivation_time+max{(Tfirst_available_CSI+TCSI_processing),(T1+T2+T3),[X]}+TCSI_reporting_after

Issue 1-1-4: Whether the PL-RS will introduce extra delay time when the known condition is met in FR1?

Agreements:

- When PL-RS of target FR1 PUCCH SCell is known, the same number of samples for PL-RS measurement as FR2 should be added to the FR1 PUCCH Scell activation latency requirement.

Issue 1-1-5: Interruption due to PRACH transmission of PUCCH Scell (related to RAN1 reply LS R4-2206147(R1-2202599))?

Agreements:

- Clarify that there could be interruption on UL transmission on other serving Cells when colliding with RACH transmission on PUCCH Scell if UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH.

Issue 1-2-1: The scenarios of PUCCH Scell activation with multiple DL Scells?

Agreements:

- The delay requirements for PUCCH SCell activation with multiple DL SCells are defined for EN-DC, NE-DC, NR SA.

- Define delay requirements for PUCCH SCell activation with multiple DL Scells for the following scenarios:

  - All SCells belonging to secondary PUCCH group

  - SCells belonging to primary plus secondary PUCCH group

- For the delay requirements for PUCCH SCell activation with multiple DL Scells, do not consider the case where unknown PUCCH Scell is being activated with known DL Scells in the secondary PUCCH group.

Issue 1-2-2: The activation delay requirements for the PUCCH Scell to be activated?

Agreements:

- PUCCH Scell activation delay requirements can be derived from single PUCCH Scell activation delay by replacing Tactivation_time with Tactivation_time_multiple_scells.

Issue 1-2-3: The activation delay requirements for the other DL SCells to be activated?

Agreements:

- For the activation latency requirement of multiple DL Scells being activated with PUCCH Scell by a single MAC command, the legacy requirements (in 8.3.7) are reused.

Issue 2-1-1: Reference signals configuration for PL-RS, TCI state, and spatial relation

Agreements:

- PL-RS, TCI state, and spatial relation if applicable shall be associated with one SSB ID. And an explicit PL-RS indication can be skipped in the test cases.

Issue 2-1-2: Whether to define test cases for both known and unknown case

Agreements:

- Define test cases for both known case and unknown case.

- For unknown PUCCH SCell activation in FR2, the test cases can be specified based on UE having capability of cross PUCCH group CSI reporting and SS capable of controlling UE cross cells.

Issue 2-1-3: Whether to define test cases for both valid TA and invalid TA case

Agreements:

- Define two sub tests in each test case for valid TA and invalid TA scenarios. And for invalid TA scenario, PUCCH Scell is configured with sTAG.

- TimeAlignmentTimer should expires before UE receives the activation command for invalid TA case

- UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG for valid TA case.

Issue 2-1-4: Test cases for interruption requirements due to PUCCH SCell activation/deactivation

Agreements:

- The interruption requirements due to PUCCH Scell activation/deactivation can be verified in the test cases for PUCCH Scell activation/deactivation delay requirements.

Issue 2-1-5: Whether the test cases for the following band combinations are needed?

Agreements:

- Define test cases for:

  - FR1 PCell/PSCell + FR1 PUCCH SCell

  - FR2 PCell/PSCell + FR2 PUCCH SCell(inter-band)

- FFS whether to define test cases for:

  - FR1 PCell/PSCell + FR2 PUCCH SCell

Issue 2-1-6: Testability for FR1 +FR2 combination

Candidate options for open discussion:

- Proposal 1:

  - For NR SA (or EN-DC) case, when the to-be-activated PUCCH Scell is in FR2 and PCell (or PSCell) is in FR1, only define test cases for known case with valid TA.

  - Another FR2 serving cell should be configured in the test, and TE shall guarantee that the FR2 serving cell is successfully activated before activation the PUCCH Scell.

Issue 2-1-7: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL SCells in FR1?

Agreements:

- Define test case for:

  - FR1 known PUCCH SCell + FR1 unknown SCell

  - FR1 unknown PUCCH SCell + FR1 unknown SCell

Issue 2-1-8: Whether the following test cases are needed for PUCCH SCell activation/deactivation with multiple DL SCells in FR2?

Agreements:

- Define test case for:

  - FR2 known PUCCH SCell + FR2 unknown SCell

  - FR2 unknown PUCCH SCell + FR2 unknown SCell

Issue 2-1-9: For the test cases in issue 2-1-8 if agreed, whether the PUCCH SCell and the other SCell should be at the same band?

Agreements:

- For the test cases in issue 2-1-8, the PUCCH SCell and the other SCell should be at the same band.

Test case list and working split

Agreements:

| Test No. | Test | DC/CA mode | Section | Company |
| --- | --- | --- | --- | --- |
| TC for SA |  |  |  |  |
| TC 1-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x1 | Huawei |
| TC 1-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x2 | MTK |
| TC 1-3 (TBD) | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | NR SA with one or more cells in FR2 | 7.5.3.x1 | Ericsson |
| TC 1-4 (TBD) | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell | NR SA with one or more cells in FR2 | 7.5.3.x2 | Qualcomm |
| TC 1-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x3 | Nokia |
| TC 1-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x4 | CATT |
| TC 1-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x3 | Xiaomi |
| TC 1-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | NR SA with all cells in FR1 | 6.5.3.x4 | Apple |
| TC 1-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x5 | Ericsson |
| TC 1-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PCell | NR SA with one or more cells in FR2 | 7.5.3.x6 | Qualcomm |
| TC for EN-DC |  |  |  |  |
| TC 2-1 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known cell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x1 | OPPO |
| TC 2-2 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x2 | CATT |
| TC 2-3 (TBD) | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x1 | Huawei |
| TC 2-4 (TBD) | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x2 | Huawei |
| TC 2-5 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with inter-band FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x3 | Huawei |
| TC 2-6 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x4 | Nokia |
| TC 2-7 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x3 | vivo |
| TC 2-8 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | EN-DC with all NR cells in FR1 | 4.5.3.x4 | Ericsson |
| TC 2-9 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x5 | MTK |
| TC 2-10 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | EN-DC with one or more NR cells in FR2 | 5.5.3.x6 | Apple |
| Note1: Both valid TA and invalid TA cases are included in each test case if applicable. Note2: TC 1-7~TC 1-10 and TC 2-7~TC 2-10 are for the PUCCH SCell activation/deactivation delay requirements with multiple DL SCells. |  |  |  |  |

#### 2.4.2	Remaining Open issues

(1) SRS antenna port switching

Issue 3-2-5: SRS configuration for scenario 1 sync case (symbol-level interruption)

(2) HO with PSCell

Issue 2-1-1: Test cases design principle - FR1+FR2 test cases

(3) PUCCH SCell activation

Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #103-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2207730 | SRS RRM performance scope | Qualcomm, Inc. |
| R4-2207731 | SRS configuration correction | Qualcomm, Inc. |
| R4-2207766 | Work plan for R17 FeRRM performance part | Apple |
| R4-2207767 | CR on SRS antenna port switching in TS38.133 | Apple |
| R4-2207768 | On test case list for SRS antenna port switching | Apple |
| R4-2207769 | On remaining issues of HO with PScell | Apple |
| R4-2207770 | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 | Apple |
| R4-2207771 | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | Apple |
| R4-2207772 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple |
| R4-2207773 | On remaining issue for PUCCH SCell activation | Apple |
| R4-2207774 | CR on PUCCH SCell activation in TS38.133 R17 | Apple |
| R4-2207775 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) R17 | Apple |
| R4-2207956 | PUCCH SCell activation and deactivation | Qualcomm Incorporated |
| R4-2207957 | Performance requirements | Qualcomm Incorporated |
| R4-2208063 | Discussion on remaining issue about PUCCH SCell activation | Intel Corporation |
| R4-2208064 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation |
| R4-2208065 | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | Intel Corporation |
| R4-2208087 | Open issues on PUCCH SCell activation | Nokia, Nokia Shanghai Bell |
| R4-2208088 | draft CR on PUCCH SCell activation delay | Nokia, Nokia Shanghai Bell |
| R4-2208089 | draftCR on TC3 PUCCH SCell activation of FR2 known cell in SA | Nokia, Nokia Shanghai Bell |
| R4-2208090 | Interruption requirements at SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2208091 | Discussion on test cases for SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2208092 | 38.133 CR on introduction of SRS antenna port switching (resubmission) | Nokia, Nokia Shanghai Bell |
| R4-2208093 | draftCR on interruptions at SRS antenna switching | Nokia, Nokia Shanghai Bell |
| R4-2208094 | draftCR on TC1 SRS antenna switching with 1 SRS symbol in sync EN-DC | Nokia, Nokia Shanghai Bell |
| R4-2208106 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi |
| R4-2208107 | CR on TC for PUCCH SCell activation and deactivation delay requirements | Xiaomi |
| R4-2208108 | Draft CR on NR FR1 - E-UTRAN interruptions at NR SRS antenna port switching with 1 SRS symbol in asynchronous NE-DC | Xiaomi |
| R4-2208170 | Discussion on issues for requirement of HO with PSCell | CATT |
| R4-2208171 | Completing requirement of HO with PSCell | CATT |
| R4-2208172 | Discussion on remaining issues for PUCCH Scell activation | CATT |
| R4-2208173 | Completing PUCCH SCell activation requirement | CATT |
| R4-2208174 | Discussion on test for SRS antenna port switching | CATT |
| R4-2208175 | Test case for Interruptions at NR FR1 one SRS symbol with antenna port switching in asynchronous EN-DC | CATT |
| R4-2208176 | Discussion on test for HO with PSCell | CATT |
| R4-2208177 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT |
| R4-2208178 | Discussion on test for PUCCH SCell activation | CATT |
| R4-2208179 | Test case for PUCCH SCell activation and deactivation delay requirements | CATT |
| R4-2208313 | Discussion on interruption due to SRS antenna port switching | LG Electronics Inc. |
| R4-2208316 | CR on interruption limitation due to SRS antenna switching | LG Electronics Inc. |
| R4-2208349 | Initial discussion on test cases for PUCCH SCell ActivationDeactivation | OPPO |
| R4-2208350 | draft CR on NR FR1-E-UTRAN interruptions at NR SRS antenna port switching with 1 SRS symbol in synchronous NE-DC (TC6) | OPPO |
| R4-2208351 | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) | OPPO |
| R4-2208352 | Maintenance to requirements for PUCCH SCell activationdeactivation | OPPO |
| R4-2208353 | draft CR for PUCCH SCell activation and deactivation delay requirements of FR1 known cell for EN-DC (TC10) | OPPO |
| R4-2208458 | RRM performance requirements for HO with PSCELL | Qualcomm Incorporated |
| R4-2208461 | Draft CR on TC for interruptions at SRS antenna port switching with more than 1 SRS symbol in async NE-DC | MediaTek Inc. |
| R4-2208462 | Discussion on PUCCH SCell activation and deactivation | MediaTek Inc. |
| R4-2208463 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell | MediaTek Inc. |
| R4-2208464 | CR on PUCCH SCell activation and deactivation delay requirements | MediaTek Inc. |
| R4-2208500 | Correction on HO with PSCell requirements in 38133 | Nokia, Nokia Shanghai Bell |
| R4-2208501 | Correction on HO with PSCell requirements in 36133 | Nokia, Nokia Shanghai Bell |
| R4-2208502 | discussion on HO with PSCell performance requirements | Nokia, Nokia Shanghai Bell |
| R4-2208503 | dratCR on test case for HO with PSCell - TC10 | Nokia, Nokia Shanghai Bell |
| R4-2208518 | Discussion on test cases for HO with PSCell | CMCC |
| R4-2208519 | Discussion on test cases for PUCCH SCell activation/deactivation | CMCC |
| R4-2208520 | Discussion on test cases for SRS antenna port switching | CMCC |
| R4-2208530 | CR on Handover with PSCell | CMCC |
| R4-2208935 | CR on SRS antenna port switching requirements 36.133 | Huawei, Hisilicon |
| R4-2208936 | Draft CR on SRS antenna port switching requirements 38.133 | Huawei, Hisilicon |
| R4-2208937 | CR on HO with PSCell requirements | Huawei, Hisilicon |
| R4-2208938 | Discussion on maintenance for PUCCH SCell activation | Huawei, Hisilicon |
| R4-2208939 | CR on maintenance for PUCCH SCell activation | Huawei, Hisilicon |
| R4-2208940 | Discussion on performance requirements for SRS antenna switching | Huawei, Hisilicon |
| R4-2208941 | Draft CR on TC for NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | Huawei, Hisilicon |
| R4-2208942 | Discussion on performance requirements for HO with PSCell | Huawei, Hisilicon |
| R4-2208943 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, Hisilicon |
| R4-2208944 | Discussion on performance requirements for PUCCH SCell activation | Huawei, Hisilicon |
| R4-2208945 | Draft CR on TC for PUCCH SCell activation and deactivation | Huawei, Hisilicon |
| R4-2209133 | Draft CR on SRS antenna port switching requirements 38.133 | Huawei, Hisilicon |
| R4-2209492 | Discussion on test cases for R17 interruption requirements for SRS antenna switching | vivo |
| R4-2209493 | draft CR on test cases for NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | vivo |
| R4-2209494 | CR on R17 core requirements for HO with PSCell | vivo |
| R4-2209495 | Discussion on test cases for R17 requirements for HO with PSCell | vivo |
| R4-2209496 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo |
| R4-2209991 | Draft CR 38.133 on the test case of handover with PSCell from NR-DC to NR-DC with parallel processing | MediaTek Inc. |
| R4-2210114 | RRM performance requirements for HO with PSCELL | Qualcomm Incorporated |
| R4-2210130 | Remaining issue in SRS antenna port switching | Ericsson |
| R4-2210131 | Discussion RRM requirements for handover with PSCell for NR-U | Ericsson |
| R4-2210132 | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson |
| R4-2210133 | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson |
| R4-2210134 | RRM requirements for SCell activation/deactivation with PUCCH | Ericsson |
| R4-2210135 | CR on SCell activation/deactivation with PUCCH | Ericsson |
| R4-2210136 | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell | Ericsson |
| R4-2210137 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | Ericsson |
| R4-2210278 | Email discussion summary for [103-e][206] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2210279 | Email discussion summary for [103-e][207] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2210280 | Email discussion summary for [103-e][208] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2210475 | Email discussion summary for [103-e][206] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2210476 | Email discussion summary for [103-e][207] NR_RRM_enh2_2 | Moderator (vivo) |
| R4-2210477 | Email discussion summary for [103-e][208] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2210583 | WF on further RRM enhancement for NR and MR-DC – HO with PSCell | vivo |
| R4-2210584 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2210993 | draftCR on interruptions at SRS antenna switching | Nokia, Nokia Shanghai Bell |
| R4-2210994 | CR on interruption limitation due to SRS antenna switching | LG Electronics Inc. |
| R4-2210995 | CR on SRS antenna port switching requirements 36.133 | Huawei, Hisilicon |
| R4-2210996 | On test case list for SRS antenna port switching | Apple |
| R4-2210997 | CR on HO with PSCell for NE-DC to NE-DC in TS38.133 R17 | Apple |
| R4-2210998 | CR on HO with PSCell for EN-DC to EN-DC in TS36.133 R17 | Apple |
| R4-2210999 | Completing requirement of HO with PSCell | CATT |
| R4-2211000 | CR on R17 core requirements for HO with PSCell | vivo |
| R4-2211001 | CR on requirements for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson |
| R4-2211002 | CR on requirements for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson |
| R4-2211003 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi |
| R4-2211004 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple |
| R4-2211005 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation |
| R4-2211006 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT |
| R4-2211007 | draft CR on HO with PSCell from NR SA to EN-DC with parallel processing(TC2) | OPPO |
| R4-2211008 | dratCR on test case for HO with PSCell - TC10 | Nokia, Nokia Shanghai Bell |
| R4-2211009 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, Hisilicon |
| R4-2211010 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo |
| R4-2211011 | Draft CR 38.133 on the test case of handover with PSCell from NR-DC to NR-DC with parallel processing | MediaTek Inc. |
| R4-2211012 | TC for HO with PSCell from NE-DC to NE-DC with unknown target PSCell | Ericsson |
| R4-2211013 | draft CR on PUCCH SCell activation delay | Nokia, Nokia Shanghai Bell |
| R4-2211014 | CR on PUCCH SCell activation and deactivation delay requirements | MediaTek Inc. |
| R4-2211015 | CR on maintenance for PUCCH SCell activation | Huawei, Hisilicon |
| R4-2211016 | CR on SCell activation/deactivation with PUCCH | Ericsson |
| R4-2211170 | LS on handling of RACH occasion collision between PCell and PSCell for HO with PSCell for NR-U | Ericsson |
| R4-2211191 | Correction on HO with PSCell requirements in 36133 | Nokia, Nokia Shanghai Bell |
| R4-2211208 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2211209 | draft CR on PUCCH SCell activation delay | Nokia, Nokia Shanghai Bell |

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template