3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2206860

# Online Meeting, 9th – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3061 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on BAP entity release in MR DC release procedures in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is the R17 shadow correction corresponding to R16 CR#3060 on MR DC release procedures in TS 38.331. Detailed reason for change is given as below: According to the current specification TS 38.331, clause 5.3.5.10, the IAB-MT performing MR DC release will release bap-Config associated with the SCG, but the spec does not clarify whether the IAB-MT releases the BAP entity even if the last and only configured bap-Config is released. If the IAB node working in EN-DC or NR-DC modes releases all the configured bap-Config, it should also release the BAP entity. This is similar to the sec 5.3.5.12 on BAP entity release if the bap-Config is set to release. Also, to cover both R16 EN-DC and R17 NR-DC cases, it is proposed to add “if there is no configured bap-Config” as the condition for BAP entity release, i.e. not releasing the BAP entiry in NR-DC case, if both MN and SN provide bap-Config and only one is release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the operation and the condition to release the BAP entity in IAB-MT’s MR DC release procedures, in clause 5.3.5.10: “release the BAP entity as specified in TS 38.340 [47], if there is no configured bap-Config” Rev1 [MCC]: WI Code changed to match the cat F CR. Impact analysis Impacted functionality: BAP entity release Inter-operability: - If the IAB-MT is implemented according to this CR but the network is not, there is no inter-operability issue forseen. - If the network is implemented according to this CR but the IAB-MT is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BAP entity is useless and still kept in the IAB node after the IAB-MT in the EN/NR DC mode releases all the NR SCG configuration and all the configured bap-Config. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

#### 5.3.5.10	MR-DC release

The UE shall:

1>	as a result of MR-DC release triggered by E-UTRA or NR:

2>	release SRB3, if established, as specified in 5.3.5.6.2;

2>	release measConfig associated with SCG;

2>	if the UE is configured with NR SCG:

3>	release the SCG configuration as specified in clause 5.3.5.4;

3>	release otherConfig associated with the SCG, if configured;

3>	stop timers T346a, T346b, T346c, T346d and T346e associated with the SCG, if running;

3>	release bap-Config associated with the SCG, if configured;

3>	release the BAP entity as specified in TS 38.340 [47], if there is no configured bap-Config;

3>	release iab-IP-AddressConfigurationList associated with the SCG, if configured;

2>	else if the UE is configured with E-UTRA SCG:

3>	release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

End of Change