3GPP TSG RAN meeting #96									RP-221746

Budapest, June 6 - 9th, 2022									revised RP-221170

## Status Report to TSG

Agenda item:			9.3.2.7

| WI / SI Name | Rel-18 NR NTN (Non-Terrestrial Networks) enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR-NTN-enh |  |  |  |  |
| Unique ID | 941006 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220953 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 5% RAN1: 10% RAN2: 0% RAN3: 0% RAN4: 0% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Nicolas Chuberre |
|  | Company | Thales |
|  | Email | Nicolas.chuberre@thalesaleniaspace.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

- RAN1#109-e, May 9 – 20th 2022, e-meeting

[General]

Agreements on “Coverage enhancements”

Agreement

For NR NTN coverage enhancement, evaluate only handset terminals as UE type.

- i.e., VSAT is not considered.

Agreement

Coverage performance in NR NTN is evaluated according to the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821

  - For polarization loss,

    - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used

- Step 2: Required SNR of target service is evaluated by LLS

- Step 3: The CNR and the required SNR are compared

Agreement

Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios.

- Note: Service type for each scenario is discussed separately

- Note: Parameter set (Set-1/2) is discussed separately

Note: MEO can be evaluated optionally

Agreement

For evaluation of coverage performance in NR NTN,

- It is assumed that carrier bandwidth is sufficiently large to transmit each channel.

- Companies are encouraged to report BWP bandwidth, when necessary (e.g. for frequency hopping).

- Note: each channel bandwidth is discussed separately.

Agreement

For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- Each packet is transmitted within 20 ms, if packet combining is not used.

  - Companies are encouraged to evaluate at least packet transmission without combining

  - Companies are encouraged to report how to apply packet combining, if used.

  - Note: in packet combining, two packets can be combined into a single packet at TX side

    - Companies should report the impact on E2E latency

- VoIP is evaluated only in LEO scenario.

- Note 1: PRB/MCS/TBS determinations are discussed separately

- Note 2: companies should report if HARQ is used in the evaluations, and if evaluations depart from the assumption that each packet is transmitted within 20 ms

Agreement

Reuse Set-1/2 satellite parameters as in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, and as in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band.

- In addition, evaluations assuming relevant ITU regulatory limitations on power flux density can be reported in the study phase.

  - Companies should report which value of EIRP density is used and corresponding justification.

Agreement

For link budget calculation, parameters in the following table is assumed.

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| Channel bandwidth | FFS |
| Satellite altitude | 600 km, 1200 km, 10000 km, 35786 km |
| Target elevation angle | [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)] |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | FFS |
| Satellite RF parameters | FFS |
| Polarization loss | As agreed separately |
| Outcome | CNR |
| - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)" |  |

Agreement

If corresponding channel (including SCS) is agreed as evaluation target channel, the following features introduced in Rel-17 Coverage enhancement WI can be applied in coverage evaluation of NR NTN.

- For VoIP, max 20 PUSCH repetitions if SCS = 15 kHz and packet combining/HARQ are not applied; otherwise, max 32 PUSCH repetitions with consideration of the impact on E2E latency

- For low-data rate service, max 32 PUSCH repetitions

- TBoMS

- Joint channel estimation (DMRS bundling)

  - Companies are encouraged to report how to apply

  - Max 16 Msg.3 PUSCH repetitions

Agreement

For low-data rate service, the following target data rate is assumed.

- For DL, 3 kbps if satellite EIRP density lower than values in table 6.1.1.1-1/2 of TR38.821 for GEO/LEO-1200/LEO-600 and S-band, or values in table 6.1.1.1-1/2 of RP-220590 for MEO and S-band due to ITU regulatory limitations on power flux density is considered; otherwise, 1 Mbps

- For UL, 3 kbps and 100 kbps

  - FFS: which data rate applies for GEO/MEO/LEO

Agreement

For NR NTN coverage enhancement, the following channels/signals can be evaluated.

- PUSCH for VoIP

- PUSCH for low data rate service

- PUCCH format 1 with 2 bits

- PUCCH format 3 with 11 bits

- PRACH format 0

- PRACH format 2

- PRACH format B4

- PUSCH Msg.3

- PUCCH for Msg.4 HARQ-ACK

- SSB

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDCCH

- Broadcast PDCCH (PDCCH of Msg.2)

Agreement

Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration.

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization |
| Polarisation | Linear |
| Rx Antenna gain | [X] dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | [X] dBi per element |

- X = -5 as working assumption

  - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

R1-2205622	[Draft] LS on UE antenna gain for NR NTN coverage enhancement	Moderator (NTT DOCOMO, INC.)

R1-2205623	LS on UE antenna gain for NR NTN coverage enhancement	RAN1, NTT DOCOMO, INC.

Final LS is endorsed in R1-2205623.

Agreement

For coverage performance evaluation, the following elevation angle is assumed.

- 30 deg for LEO, 12.5 deg for GEO-Set 1, 20 deg for GEO-Set 2, as in in Table 6.1.3.2-1 of TR38.821

  - Note: For GEO-Set 1, channel parameters for 10 deg is used in LLS.

- 30 deg for MEO

- Other elevation angles can be evaluated as optional

- Note: these values are elevation angles at the edge of the edge beam.

Agreement

For NR NTN coverage enhancement, evaluate the following cases.

| Case | Satellite orbit | Satellite parameter set | Elevation angle (deg) | Terminal | Frequency band | Service type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | 1 | 12.5 | Handset | S-band | Low-data rate service |
| 2 | GEO | 2 | 20 | Handset | S-band | Low-data rate service |
| 3 (Optional) | LEO-1200 | 1 | 30 | Handset | S-band | VoIP |
| 4 | LEO-1200 | 2 | 30 | Handset | S-band | VoIP |
| 5 | LEO-1200 | 2 | 30 | Handset | S-band | Low-data rate service |
| 6 (Optional) | LEO-600 | 1 | 30 | Handset | S-band | VoIP |
| 7 | LEO-600 | 2 | 30 | Handset | S-band | VoIP |
| 8 (Optional) | LEO-600 | 2 | 30 | Handset | S-band | Low-data rate service |
| 9 (Optional, with higher priority than case 10) | MEO | 1 | 30 | Handset | S-band | Low-data rate service |
| 10 (Optional) | MEO | 2 | 30 | Handset | S-band | Low-data rate service |

Agreement

For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread

  - Channel model as in Table 6.1.2-4 of TR38.821, assuming NTN-TDL-A (NLOS) and NTN-TDL-C (LOS)

- Evaluation scenario

  - Rural (LOS/NLOS)

  - Sub-urban (LOS/NLOS) (optional)

- Channel estimation: Realistic estimation

  - Companies are encouraged to report channel estimation method.

- SCS

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: Not assumed

- Frequency offset: 0.1 ppm

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PUCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, 11 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | Number of DMRS symbols for PUCCH Format 3: Reported by companies |
| Repetitions | w/ repetition. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PRACH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability 10% missed detection: reported by companies if this value is used |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PUSCH Msg.3 in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| HARQ configuration | Whether/How is adopted is reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of SSB in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Other parameters | Reported by companies. |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreed LS out

- -

Other documents agreed

- -

Email discussions

- [109-e-R18-NTN-01] Email discussion on coverage enhancement for NR NTN by May 20 – Shohei (Docomo)

#### 2.1.2	Remaining Open issues

Coverage enhancements

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18

  - VoIP and low-data rate services for commercial handset terminals

Network verified UE location

- Study and evaluate, if needed, solutions for network to verify UE reported location information

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

Coverage enhancements

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18

  - VoIP and low-data rate services for commercial handset terminals

Network verified UE location

- Study and evaluate, if needed, solutions for network to verify UE reported location information

NTN-TN and NTN-NTN mobility and service continuity enhancements

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

Network verified UE location

- Study and evaluate, if needed, solutions for network to verify UE reported location information

NTN-TN and NTN-NTN mobility and service continuity enhancements

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

NR-NTN deployment in above 10 GHz

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios.

- Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band

- Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets.

Coverage enhancements

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18

  - VoIP and low-data rate services for commercial handset terminals

NTN-TN and NTN-NTN mobility and service continuity enhancements

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Aspects related to Network verified UE location

## 4.	References

## 4.1	RAN1

- RAN1#109-e, May 9 – 20th 2022, e-meeting

Submitted TDOCs:

- R1-2203097	discussion	Rel-17 UE features for NR coverage enhancement	Huawei, HiSilicon

- R1-2203198	discussion	Discussion on UE features for NR coverage enhancement	ZTE

- R1-2203318	discussion	UE features for NR coverage enhancement	Spreadtrum Communications

- R1-2203536	discussion	Remaining issues on UE features for NR coverage enhancement	vivo

- R1-2204223	discussion	Views on Rel-17 Coverage Enhancement UE Features	Apple

- R1-2204363	discussion	Discussion on Rel-17 UE features for NR coverage enhancement	NTT DOCOMO, INC.

- R1-2204285	discussion	Discussion on Rel.17 UE features for NR coverage enhancement	CMCC

- R1-2204593	discussion	On UE features for NR coverage enhancement	Nokia, Nokia Shanghai Bell

- R1-2203654	discussion	Remaining issues on UE features for Rel-17 NR coverage enhancements	China Telecom

- R1-2203998	discussion	Rel-17 Coverage Enhancement UE features	OPPO

- R1-2204665	discussion	UE feature for Rel-17 coverage enhancement	Sharp

- R1-2203883	discussion	UE features for NR coverage enhancement	Samsung

- R1-2203783	discussion	Discussion on UE features for NR coverage enhancement	xiaomi

- R1-2205005	discussion	UE Features for Coverage Enhancements	Qualcomm Incorporated

- R1-2204874	discussion	UE Features for NR Coverage Enhancement	Ericsson

- R1-2204783	discussion	Discussion on UE features for NR coverage enhancement	Intel Corporation

- R1-2204729	discussion	Discussion on UE features for NR Coverage Enhancement	MediaTek Inc.