3GPP TSG-RAN Meeting #96	RP-221811

# Budapest, Hungary, 6th – 9th June 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0571 | rev | 8 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for handling unknown length of gNB identifier [gNB_ID_Length] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Huawei, ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-06-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are scenarios where the gNB ID length is variable and it may not be possible for the detecting gNB to easily derive the gNB ID from the cell identifier. When discovering the Xn IP address of a new neighbour (i.e using the Uplink RAN Configuration Transfer procedure), the detecting gNB cannot correctly fill the target’s address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the NR CGI to the Target RAN Node IE within the SON Configuration Transfer IE in the UPLINK RAN CONFIGURATION TRANSFER message. Presence of this IE indicates that the CN shall, if supported, ignore the legacy target signalling, and instead, use the NR CGI to identify the target. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Xn IP address discovery when the RAN does not know the length of target gNB ID. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.1.2, 9.3.3.6, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.331 CR 3181 TS 36.331 CR 4821 TS 38.306 CR 0747 TS 36.306 CR 1850 TS 38.300 CR 0474 TS 36.300 CR 1363 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r8: include full list of other specs affected and respective CRs |  |  |  |  |  |  |  |  |

### 8.8.1	Uplink RAN Configuration Transfer

#### 8.8.1.1	General

The purpose of the Uplink RAN Configuration Transfer procedure is to transfer RAN configuration information from the NG-RAN node to the AMF. The AMF does not interpret the transferred RAN configuration information. This procedure uses non-UE associated signalling.

#### 8.8.1.2	Successful Operation

Figure 8.8.1.2-1: Uplink RAN configuration transfer

The NG-RAN node initiates the procedure by sending the UPLINK RAN CONFIGURATION TRANSFER message to the AMF.

If the AMF receives the SON Configuration Transfer IE, it shall transparently transfer the SON Configuration Transfer IE towards the NG-RAN node indicated in the Target RAN Node ID IE which is included in the SON Configuration Transfer IE. If the NG-RAN CGI IE is included within the Target RAN Node ID IE, the AMF shall, if supported, ignore the Global RAN Node ID IE within the Target RAN Node ID IE, and use it to identify the target gNB as described in TS 38.300 [8].

If the AMF receives the EN-DC SON Configuration Transfer IE, it shall transparently transfer the EN-DC SON Configuration Transfer IE towards an MME serving the eNB indicated in the Target eNB-ID IE which is included in the EN-DC SON Configuration Transfer IE.

If the AMF receives the Inter-system SON Configuration Transfer IE, it shall transparently transfer the Inter-system SON Configuration Transfer IE towards an MME serving the eNB indicated in the Target eNB-ID IE which is included in the Inter-system SON Configuration Transfer IE.

#### 8.8.1.3	Abnormal Conditions

Void.

>>>> NEXT CHANGE <<<<

#### 9.3.3.6	SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, and additionally includes the NG-RAN node identifier of the destination of this configuration information and the NG-RAN node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Target RAN Node ID | M |  |  |  |  |  |
| >Global RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | TAI 9.3.3.11 |  |  |  |
| >NG-RAN CGI | O |  | 9.3.1.73 | This IE is ignored if the SON Information IE contains the SON Information Reply IE. | YES | ignore |
| Source RAN Node ID | M |  |  |  |  |  |
| >Global RAN Node ID | M |  | 9.3.1.5 |  |  |  |
| >Selected TAI | M |  | TAI 9.3.3.11 |  |  |  |
| SON Information | M |  | 9.3.3.7 |  |  |  |
| Xn TNL Configuration Info | C-ifSONInformationRequest |  | 9.3.3.9 | Source NG-RAN node Xn TNL Configuration Info. |  |  |

| Condition | Explanation |
| --- | --- |
| ifSONInformationRequest | This IE shall be present if the SON Information IE contains the SON Information Request IE set to "Xn TNL Configuration Info" |

>>>> NEXT CHANGE <<<<

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

***  skip unchanged text in same section ***

TargetRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NR-CGI	CRITICALITY ignore	EXTENSION NR-CGI PRESENCE optional },

...

}

TargetRNC-ID ::= SEQUENCE {

lAI					LAI,

rNC-ID				RNC-ID,

extendedRNC-ID		ExtendedRNC-ID											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TargetRNC-ID-ExtIEs} } 	OPTIONAL,

...

}

TargetRNC-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}