3GPP TSG RAN meeting #97-e	RP-221937

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:	9.3.1.5

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_SL_enh2 |  |  |  |  |
| Unique ID | 940097 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221798 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: 06/2024 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx % | Core part: 20% | Performance Part: 0% | Testing part: xx % |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Kevin Lin |
|  | Company | OPPO |
|  | Email | Kevin.Lin@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

NR sidelink CA operation

- On hold until further checking in RAN#97

NR Sidelink operation in FR1 unlicensed spectrum

Agreement

The following evaluation scenario can be used for evaluating performance of SL-U designs, resource allocation schemes, and coexistence study with another RAT in a shared channel.

- Scenario 1 (commercial use cases) – recommended:

  - Evaluation methodology baseline is NR-U from TR 38.889 with the following updates.

  - Indoor layout

    - Option 1: a pairs topology for SL-U from R1-2205033 – recommended

      - a = 20m, b = 60m, c = 20m, d = 80 m

      - There are two operators to model two RATs at a time. The red one is SL-U UE, the blue one is Wi-Fi or NR-U.

      - For NR-U / Wi-Fi, the same number of UEs / Wi-Fi STA as the total number of SL-U devices are dropped in the area. The NR-U UE / Wi-Fi nodes are dropped uniformly per gNB/AP per 20 MHz.

        - Companies should report if they used a different number of UEs / Wi-Fi STA as the total number of SL-U devices, as an additional evaluation scenario.

      - For evaluation of unicast traffic, the topology of SL-U is pair topology and the SL-U UEs are dropped uniformly at random in the area.

        - Companies should report how SL-U UEs are paired

        - 6 SL-U pairs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

      - For evaluation of groupcast traffic, SL-U UEs are dropped uniformly at random in the area, SL-UEs form groupcast UE group based on TX-RX UE distancing, the distance is provided by each company.

        - Companies should report how SL-U UEs form a group

        - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

      - For evaluation of broadcast traffic, SL-U UEs are dropped uniformly at random in the area.

        - 12 SL-U UEs and 4 NR-U UEs / Wi-Fi nodes per gNB/AP per 20 MHz

    - Option 2: SL UE clusters (R1-2203146)

      - Indoor layout and UE dropping model with N = 3 or 6 clusters and each with M=5 UEs

      - Each cluster is a circle, with a central point and radius Rmax = 15 or 10m and Rmin = 5 or 1m

      - No overlapping among the N clusters

      - For coexistence, there are two operators to model two RATs at a time, where the red one is Wi-Fi AP or NR-U gNB. NR-U UE / Wi-Fi STA are dropped uniformly per gNB/AP.

      - Simulation bandwidth can be larger than 20MHz (e.g., 80MHz)

  - Channel model follows NR InH Mixed Office model used in NR-U (TR38.889)

  - Traffic model

    - Option 1: R17 sidelink commercial traffic model with periodic model 3 with packet size reduced by a factor of (high: 1; mid: 5; low: 10)

      - FFS whether/how the PDB requirement can be captured

    - Option 2: FTP model 3 with arrival rate satisfying one of the followings:

      - BO Low load: 10%~25%

      - BO Mid load: 35%~50%

      - BO High load: above 55%

    - Option 3: XR cloud gaming model in TR38.838

      - FFS whether/how the PDB requirement can be captured

    - It is up to each company to use either Option 1 or 2 or Option 3 or mixed of them

  - Interference model:

    - Layout option 1: Explicit modelling of NR-U / WiFi transmissions (as per TR38.889)

    - Note, for the interference traffic model:

      - The same or equivalent traffic model setting as SL-U should be used as much as possible to achieve equal load (e.g., SL-U RAT offered load equal the interfering RAT’s offered load).

      - The same number of traffic flows should be used between SL-U and the interfering RAT (e.g., 10 UEs with 10 flows, and 5 STAs with 2 flows each, one for DL and one for UL)

        - Companies should report if they used a different assumption, as an additional evaluation scenario.

  - Performance metric: UPT, latency, and PRR which regards the packet whose delay exceeding the remaining PDB as transmission failure.

    - FFS: UE satisfaction/system capacity as section 7.2 in TR 38.838 for XR traffic evaluation

    - FFS for groupcast and broadcast

  - Fair coexistence criterion between SL-U and the interfering RAT (e.g., according to NR-U TR38.889)

Agreement

- CW adjustment

  - NR-U DL CW adjustment mechanism is used as the baseline for SL-U when SL-HARQ feedback is enabled in SCI for unicast

    - FFS any necessary update for SL-U operation

  - FFS: how to determine CW size when SL-HARQ feedback is disabled in SCI

  - FFS the case of groupcast option 1 (NACK-only) and groupcast option 2

Agreement

- Type 2A/2B/2C SL channel access procedures

  - Type 2A channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy

    - FFS any other transmission by a UE (e.g., other than COT sharing)

    - FFS whether Type 2A is used also for the case of short control signalling transmission

  - Type 2B channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy

    - FFS the case when the gap is between 16 and 25us

    - FFS any other transmission by a UE (e.g., other than COT sharing)

  - Type 2C channel access procedure is applicable to the following case:

    - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us.

    - FFS any other transmission by a UE (e.g., other than COT sharing)

    - FFS whether Type 2C is used also for the case of short control signalling transmission

  - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures

  - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs

Agreement

Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U.

- FFS details

Agreement

- For UE-to-UE COT sharing, continue considering the following alternatives:

  - Alt. 1: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the at least COT initiating UE’s PSSCH data transmission in the COT.

    - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information

    - FFS any additional conditions

  - Alt. 2: A responding SL UE can utilize a COT shared by a COT initiating UE when the responding SL UE is a target receiver of the COT initiating UE’s transmission in the COT.

    - When the responding UE uses the shared COT for its transmission has an equal or smaller CAPC value than the CAPC value indicated in a shared COT information

    - FFS how to determine a SL UE is a target receiverFFS: details of the channel type of the COT initiating UE’s transmission

    - FFS any additional conditions

  - For Alt1 and Alt2: When a responding UE uses a shared COT for its transmission(s), the COT initiating UE is a target receiver of the responding UE’s transmission(s).

    - FFS: details of the channel type of the responding UE’s transmission(s)

- gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18

- FFS whether a Mode 1 UE can report a COT or related information to gNB for aiding Mode 1 RA

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

NR sidelink enhancement in FR2 licensed spectrum

- On hold until further checking in RAN#97

Co-channel coexistence for LTE sidelink and NR sidelink

Working assumption

Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Conclusion

For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

  - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning.

  - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe

  - FFS: potential enhancements for synchronization can be further investigated

Agreement

For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints:

  - NR SL resource pool is configured with 15 kHz SCS.

    - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools

  - For NR PSFCH (if configured), at least the following alternatives are studied:

    - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions.

      - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH.

    - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots.

      - FFS: periodicities of the set.

#### 2.1.2	Remaining Open issues

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only

- Study and specify enhanced sidelink operation on FR2 licensed spectrum (This part of the work is put on hold until further checking in RAN#97)

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any

## 2.2	RAN2

#### 2.2.1	Agreements

- No scheduled meeting TU since the last RAN

#### 2.2.2	Remaining Open issues

- Mechanism to support NR sidelink CA operation based on LTE sidelink CA operation (This part of the work is put on hold until further checking in RAN#97)

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- No scheduled meeting TU since the last RAN

#### 2.4.2	Remaining Open issues

- UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation

- UE RRM core requirement for the new features introduced in this WI

- UE demodulation performance requirements

- UE RRM performance requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#110

- R1-2205744	Channel access mechanism for sidelink operation in unlicensed spectrum	FUTUREWEI

- R1-2205839	On Channel Access Mechanism and Evaluation Methodology for SL-U	Nokia, Nokia Shanghai Bell

- R1-2205850	Discussion on channel access mechanism for sidelink on unlicensed spectrum	LG Electronics

- R1-2205886	Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2205991	Discussion on channel access mechanism for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2206041	Channel access mechanism for sidelink on unlicensed spectrum	vivo

- R1-2206097	Discussion on channel access mechanism for SL-U	ZTE,Sanechips

- R1-2206119	Discussion on channel access mechanism for SL-unlicensed	Sony

- R1-2206171	Discussion on channel access mechanism for SL-U	Fujitsu

- R1-2206290	Access mechanisms and resource allocation for NR sidelink in unlicensed channel	OPPO

- R1-2206400	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2206438	NR Sidelink Unlicensed Channel Access Mechanisms	Fraunhofer HHI, Fraunhofer IIS

- R1-2206448	Channel access mechanism for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2206469	Channel Access of Sidelink on Unlicensed Spetrum	NEC

- R1-2206585	Channel Access Mechanisms for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2206644	Discussion on channel access mechanism for sidelink-unlicensed	Xiaomi

- R1-2206669	Discussion of channel access mechanism for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2206691	Discussion on channel access mechanism for sidelink on unlicensed spectrum	China Telecom

- R1-2206826	On channel access mehanism for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2206860	On Channel Access Mechanism for SL-U	ITL

- R1-2206913	Discussion on channel access mechanism for sidelink on unlicensed spectrum	CMCC

- R1-2207015	Discussion on channel access mechanism	MediaTek Inc.

- R1-2207110	Discussion of Channel Access Mechanisms	Johns Hopkins University APL

- R1-2207128	SL Channel access in unlicensed spectrum	InterDigital, Inc.

- R1-2207136	On Evaluation Methodology for SL-U	CableLabs

- R1-2207233	Channel Access Mechanism for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2207279	Discussion on Channel access mechanism for NR sidelink evolution	Sharp

- R1-2207298	Sidelink channel access on unlicensed spectrum	Panasonic

- R1-2207337	Channel access mechanism for sidelink on FR1 unlicensed band	Apple

- R1-2207408	Discussion on channel access mechanism in SL-U	NTT DOCOMO, INC.

- R1-2207504	Discussion on sidelink on unlicensed spectrum	ASUSTeK

- R1-2207511	Discussions on channel access mechanism for sidelink on unlicensed spectrum	ROBERT BOSCH GmbH

- R1-2207566	Channel access mechanism for SL-U	Ericsson

- R1-2207599	Discussion on channel access mechanism for SL on unlicensed spectrum	WILUS Inc.

- R1-2207709	Discussion on channel access mechanism for SL-U	ZTE,Sanechips

- R1-2207743	On Evaluation Methodology for SL-U	CableLabs, Broadcom, Charter Communications

- R1-2205745	Physical channel design for sidelink operation in unlicensed spectrum	FUTUREWEI

- R1-2205840	On Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

- R1-2205851	Discussion on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

- R1-2205887	Physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2205992	Discussion on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2206042	Physical channel design framework for sidelink on unlicensed spectrum	vivo

- R1-2206098	Discussion on physical layer structures and procedures for SL-U	ZTE,Sanechips

- R1-2206120	Discussion on physical channel design framework for SL-unlicensed	Sony

- R1-2206291	Physical channel designs of NR sidelink in unlicensed channel	OPPO

- R1-2206401	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2206439	NR Sidelink Unlicensed Physical Channel Design	Fraunhofer HHI, Fraunhofer IIS

- R1-2206449	Physical layer design framework for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2206470	Discussion on physical channel design framework	NEC

- R1-2206586	Physical Channel Design for SL Operating in Unlicensed Spectrum	Intel Corporation

- R1-2206645	Discussion on physical channel design for sidelink-unlicensed	Xiaomi

- R1-2206661	Discussion on Physical channel design framework for sidelink on unlicensed spectrum	Hyundai Motor Company

- R1-2206670	Discussion of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2206827	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2206861	On Physical Channel Design framework for SL-U	ITL

- R1-2206914	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CMCC

- R1-2207016	Discussion on physical channel design framework	MediaTek Inc.

- R1-2207064	Physical channel design for sidelink on unlicensed spectrum	EURECOM

- R1-2207111	Discussion on Design of Short Synchronization Signals	Johns Hopkins University APL

- R1-2207129	SL U physical layer design framework	InterDigital, Inc.

- R1-2207234	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2207280	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

- R1-2207299	Physical channel design for sidelink on unlicensed spectrum	Panasonic

- R1-2207338	Physical Channel Design Framework for Sidelink on FR1 Unlicensed Spectrum	Apple

- R1-2207409	Discussion on channel design framework in SL-U	NTT DOCOMO, INC.

- R1-2207567	PHY channel design framework for SL-U	Ericsson

- R1-2207600	Discussion on PHY channel design framework for SL on unlicensed spectrum	WILUS Inc.

- R1-2205841	On Co-channel Coexistence for LTE Sidelink and NR Sidelink	Nokia, Nokia Shanghai Bell

- R1-2205852	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	LG Electronics

- R1-2205888	Co-channel coexistence for LTE sidelink and NR sidelink	Huawei, HiSilicon

- R1-2205993	Discussion on Co-channel coexistence for LTE sidelink and NR sidelink	Spreadtrum Communications

- R1-2206043	Co-channel coexistence for LTE sidelink and NR sidelink	vivo

- R1-2206065	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	TOYOTA Info Technology Center

- R1-2206099	Study on co-channel coexistence for LTE sidelink and NR sidelink	ZTE,Sanechips

- R1-2206121	Discussion on co-channel coexistence for LTE sidelink	Sony

- R1-2206292	Discussion on co-channel coexistence for LTE and NR V2X	OPPO

- R1-2206402	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CATT, GOHIGH

- R1-2206440	Discussion on Co-Channel Coexistence for LTE and NR Sidelink	Fraunhofer HHI, Fraunhofer IIS

- R1-2206450	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Lenovo

- R1-2206471	Co-existence between LTE and NR sidelink	NEC

- R1-2206587	Considerations for LTE Sidelink and NR Sidelink Co-channel Coexistence	Intel Corporation

- R1-2206646	Discussion on co-channel coexistence for LTE and NR sidelink	Xiaomi

- R1-2206660	Discussion on Sidelink Co-channel Coexistence	Panasonic

- R1-2206671	Discussion of co-channel coexistence for LTE sidelink and NR sidelink	Transsion Holdings

- R1-2206681	Considerations on co-channel coexistence for LTE SL and NR SL	CAICT

- R1-2206692	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	China Telecom

- R1-2206828	On co-channel coexistence for LTE sidelink and NR sidelink	Samsung

- R1-2206915	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	CMCC

- R1-2206956	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	ETRI

- R1-2207010	Co-channel coexistence for NR sidelink and LTE sidelink	MediaTek Inc.

- R1-2207147	Co-channel coexistence for LTE sidelink and NR sidelink	InterDigital, Inc.

- R1-2207154	On sidelink co-channel coexistence issues	Mitsubishi Electric RCE

- R1-2207235	Co-channel Coexistence Between LTE SL and NR SL	Qualcomm Incorporated

- R1-2207281	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	Sharp

- R1-2207339	Co-channel Coexistence for LTE Sidelink and NR Sidelink	Apple

- R1-2207410	Discussion on co-channel coexistence of LTE-SL and NR-SL	NTT DOCOMO, INC.

- R1-2207509	Discussions on LTE and NR sidelink co-channel coexistence	ROBERT BOSCH GmbH

- R1-2207565	Co-channel coexistence between LTE sidelink and NR sidelink	Ericsson

- R1-2207601	Discussion on co-channel coexistence for LTE sidelink and NR sidelink	WILUS Inc.

- R1-2207625	Discussion on Co-channel coexistence for LTE sidelink and NR sidelink	Continental Automotive GmbH

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template