3GPP TSG-RAN5 Meeting #96-e	R5-225729

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1791 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General updates of clause 5 for R17 new CBW configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some R17 new bands and CBWs which are already 100% completed in RAN4 are currently missing in TS38.521-1 clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General updates of clause 5: Update the format of chapter title in 5.3.3； Update note 11 and added note 11 on n14, n38, n47, n79; n53 was moved to the right position in Table 5.3.5-1. - Added the following R17 new CBWs into Table 5.3.5-1 n3 35MHz, n3 45MHz; n8 35MHz; Added 35MHz and 45MHz into Table 5.3.2-1 and Table 5.3.3-1. - Remove original Table 5.3.5-1, add a new column of 35MHz in new Table 5.3.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activated new CBW configurations in clause 5 will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2 5.3.3 5.3.5 5.5A.0 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is an outcome from R5-224287r1. |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

5.3.3	Minimum guard band and transmission bandwidth configuration

The minimum guard band for each UE channel bandwidth and SCS is specified in Table 5.3.3-1.

Table 5.3.3-1: Minimum guard band for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

NOTE: The minimum guard bands have been calculated using the following equation: (BWchannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2, where NRB are from Table 5.3.2-1.

Figure 5.3.3-1: Void

The number of RBs configured in any channel bandwidth shall ensure that the minimum guard band specified in this clause is met.

Figure 5.3.3-2: UE PRB utilization

In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB, the minimum guard band on each side of the carrier is the guard band applied at the configured channel bandwidth for the numerology that is received immediately adjacent to the guard.

If multiple numerologies are multiplexed in the same symbol and the UE channel bandwidth is >50 MHz, the minimum guardband applied adjacent to 15 kHz SCS shall be the same as the minimum guardband defined for 30 kHz SCS for the same UE channel bandwidth.

Figure 5.3.3-3: Guard band definition when transmitting multiple numerologies

NOTE: Figure 5.3.3-3 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

[For a UE supporting wideband operation], the nominal intra-cell guard bands and the corresponding sizes of the RB sets separated by the said guard bands are as specified in Table 5.3.3-2 for each UE channel bandwidth and sub-carrier spacing for the downlink and uplink. The nominal intra-cell guard bands in Table 5.3.3-2 are applicable when the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6] for the uplink and downlink are not provided, as specified in [12] clause 7.

Table 5.3.3-2: Nominal intra-cell guard bands for wideband operation

| SCS (kHz) | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- |
| 15 | 105-6-105 (216) | N/A | N/A |
| 30 | 50-6-50 (106) | 50-6-50-6-50 (162) | 50-6-50-5-50-6-50 (217) |
| 60 | 23-5-23 (51) | 23-5-23-5-23 (79) | 23-5-23-5-23-5-23 (107) |
| NOTE 1: The intra-cell guard band is denoted TBW0-GB0-…-GBN_RBset-2-TBWN_RBset-1 for N_RBset > 1 number of RB-sets with TBWr the maximum transmission bandwidth (PRB) of RB-set r and GBr the guard band (PRB) above the upper edge of RB-set r. The RB-set 0 is starting at the first common resource block (CRB) of the carrier as indicated by offsetToCarrier. The total transmission bandwidth configuration (size of resource grid) including guard bands is given in between parentheses. |  |  |  |

For a UE that supports shared spectrum channel access, there are no uplink or downlink intra-cell guard bands for operation with 10 MHz and 20 MHz channel bandwidths; the maximum transmission bandwidth configurations for these channel bandwidths are in accordance with clause 5.3.2.

For each UE channel bandwidth and sub-carrier spacing given by Table 5.3.3-2, the maximum transmission bandwidth configuration of the carrier including intra-cell guard bands, if configured for the uplink and downlink by the respective IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6], and corresponding RB-set(s) shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1 for the uplink and downlink. Minimum requirements specified for wideband operation in Clause 6 and Clause 7 also apply for intra-cell guard bands larger than the nominal sizes in Table 5.3.3-2 as listed in Table 5.3.3-3 for each sub-carrier spacing; each guard band in order of CRB index must be larger than or equal to the corresponding nominal guard band specified in Table 5.3.3-2 for each channel bandwidth.

Table 5.3.3-3: Applicable intra-cell guard bands for wideband operation

| Parameter | Unit | SCS |  |
| --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz |
| Intra-cell guard band (size) | PRB | 6,7 | 5,6,7 |
| Transmission bandwidth (size) of RB-set | PRB | 104,105 | 49,50,51 |

If the UE is configured with zero width intra-cell guard bands for the uplink and downlink by the IE intraCellGuardBandsUL-List and intraCellGuardBandsDL-List [6] on a carrier greater than 20 MHz, the maximum transmission bandwidth configuration for the uplink and downlink shall be in accordance with clause 5.3.2 with a minimum inter-cell guard band of the UE channel bandwidth as specified in Table 5.3.3-1.

5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in TS 38.211 [8], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [6] and will fulfil the minimum UE guard band requirement specified in Clause 5.3.3.

5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the TX and RX path.

Table 5.3.5-1: Channel Bandwidths for each NR band

| NR Band | SCS (kHz) |  | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 | 35 |  | 40 | 45 | 50 | 60 | 706 | 80 | 906 | 100 |
| n1 | 15 |  | 5 |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 | 4512 | 506 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 | 4512 | 506 |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 | 4512 | 506 |  |  |  |  |  |
| n2 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
| n3 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 | 35 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 | 35 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 | 35 |  | 40 | 45 | 50 |  |  |  |  |  |
| n5 | 15 |  | 5 |  | 10 |  | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n7 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
| n8 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  | 353 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  | 353 |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n1411 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | 15 |  | 5 |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 406 |  |  |  |  |  |  |  |
| n26 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 15 |  | 5 |  | 10 |  | 15 | 209 |  | 306,9 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 209 |  | 306,9 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n34 | 15 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n3811 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
| n39 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  |  |  |  |  |  |  |
| n40 | 15 |  | 57 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 |  | 80 |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 |  | 80 |  |  |
| n41 | 15 |  |  |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
| n46 | 15 |  |  |  | 107 |  |  | 20 |  |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 107 |  |  | 20 |  |  |  |  | 40 |  |  | 60 |  | 80 |  |  |
|  | 60 |  |  |  | 107 |  |  | 20 |  |  |  |  | 40 |  |  | 60 |  | 80 |  |  |
| n4711 | 15 |  |  |  | 10 |  |  | 20 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  | 20 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  |  | 20 |  | 30 |  |  | 40 |  |  |  |  |  |  |  |
| n48 | 15 |  | 57 |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 508 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 508 | 608 | 708 | 808 | 908,10 | 1008 |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  | 30 |  |  | 40 |  | 508 | 608 | 708 | 808 | 908,10 | 1008 |
| n50 | 15 |  | 57 |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  | 50 | 60 |  | 803 |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  | 50 | 60 |  | 803 |  |  |
| n51 | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n53 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 |  | 5 |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 256 | 306 |  |  | 40 |  |  |  |  |  |  |  |
| n70 | 15 |  | 5 |  | 10 |  | 15 | 203 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 203 | 253 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 203 | 253 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n75 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| n76 | 15 |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 15 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 7010 | 80 | 9010 | 100 |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 7010 | 80 | 9010 | 100 |
| n78 | 15 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 7010 | 80 | 90 | 100 |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 7010 | 80 | 90 | 100 |
| n7911 | 15 |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 | 60 |  | 80 |  | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  | 40 |  | 50 | 60 |  | 80 |  | 100 |
| n80 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
| n81 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n82 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n83 | 15 |  | 5 |  | 10 |  | 15 | 20 |  | 309 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  | 309 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n84 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
| n86 | 15 |  | 5 |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 | 20 |  |  |  |  | 40 |  |  |  |  |  |  |  |
| n95 | 15 |  | 5 |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n96 | 15 |  |  |  |  |  |  | 20 |  |  |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  | 20 |  |  |  |  | 40 |  |  | 60 |  | 80 |  |  |
|  | 60 |  |  |  |  |  |  | 20 |  |  |  |  | 40 |  |  | 60 |  | 80 |  |  |
| n97 | 15 |  | 5 |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  |  | 10 |  | 15 | 20 | 25 | 30 |  |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
| n99 | 15 |  | 5 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: For test configuration tables from the transmitter and receiver tests in Section 6 and 7 that refer to this table for test SCS, the Lowest SCS refers to lowest supported SCS per channel bandwidth, Highest SCS refers to highest supported SCS per channel bandwidth. NOTE 5: For test configuration tables from the transmitter and receiver tests in Section 6 and 7 that refer to this table and list and list the test SCS as Mid or any other value; if that value is not supported by the UE in UL and/or DL, select the closest SCS supported by the UE in both UL and DL. NOTE 6: This UE channel bandwidth is optional in R15. NOTE 7: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 8: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 9: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 10: This UE channel bandwidth is optional in R16. NOTE 11: For this band, UE channel bandwidths which are applicable to sidelink operation are specified in Table 5.3E.1-1. NOTE 12: This UE channel bandwidth is optional in R17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

5.5A	Configurations for CA

5.5A.0	General

The configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for CA operating bands with Band n90 are not listed in the tables below but are covered by this specification. For BCS4 and BCS5 combinations with n41, the n90 equivalents also include 5 MHz.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for each NR carrier of intra-band contiguous and non-contiguous CA configurations.

For a CA configuration with one or more operating band supporting asymmetric channel bandwidths as specified in sub-clause 5.3.6, requirements are defined for an asymmetric UL and DL channel bandwidth combination of a supported asymmetric channel bandwidth combination set for an operating band of the CA configuration when the said UL and DL channel bandwidths are also contained in a supported bandwidth combination set of the CA configuration.

<<Unchaged sections skipped>>

5.5A.2	Configurations for intra-band non-contiguous CA

Table 5.5A.2-1: NR CA configurations and bandwidth combination sets defined for intra-band non-contiguous CA

| NR CA Configuration | Uplink CA Configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n48(2A) |  | 10, 15, 20, 40, 50, 60 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1402 | 0 |
|  |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1402 | 1 |
| CA_n66(2A) | - | 5 (note), 10, 15, 20 | 5 (note), 10, 15, 20, 40 | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 2 |
| CA_n71(2A) | - | 5,10, 15, 20 | 5,10,15, 20 | 30 | 0 |
| CA_n77(2A) | - | 20, 40, 80, 100 | 20, 40, 80, 100 | 200 | 0 |
| CA_n78(2A) |  | 10, 20, 40, 50, 60, 80, 90, 100 | 10, 20, 40, 50, 60, 80, 90, 100 | 200 | 0 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 200 | 1 |
|  |  | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 10, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 200 | 2 |
| NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: Parameter value accounts for both, the maximum frequency range of band n48 (150 MHz), and the minimum frequency gaps in between NR non-contiguous component carriers. |  |  |  |  |  |

<<End of changes>>