3GPP TSG-RAN5 Meeting #96-e	       R5-224143
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5411 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Band 103 for UE category NB1 and NB2 in transmit power test cases in clause 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Puloli |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_upper_700MHz_A-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Details for Band 103 need to be added to relevant clause 6.2 transmit power test cases for UE category NB1 and NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added minimum requirements and test requirements for the following tramitter test cases: 6.2.2F and 6.2.2FA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of UEs supporting UE category NB1 or NB2 and band 103 not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2F and 6.2.2FA |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.2.2F.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2F.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2.2F.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2 | 20 | ±2 |
| 2 | 23 | ±2 | 20 | ±2 |
| 3 | 23 | ±2 | 20 | ±2 |
| 4 | 23 | ±2 | 20 | ±2 |
| 5 | 23 | ±2 | 20 | ±2 |
| 7 | 23 | ±2 | 20 | ±2 |
| 8 | 23 | ±2 | 20 | ±2 |
| 11 | 23 | ±2 | 20 | ±2 |
| 12 | 23 | ±2 | 20 | ±2 |
| 13 | 23 | ±2 | 20 | ±2 |
| 14 | 23 | ±2 | 20 | ±2 |
| 17 | 23 | ±2 | 20 | ±2 |
| 18 | 23 | ±2 | 20 | ±2 |
| 19 | 23 | ±2 | 20 | ±2 |
| 20 | 23 | ±2 | 20 | ±2 |
| 21 | 23 | ±2 | 20 | ±2 |
| 25 | 23 | ±2 | 20 | ±2 |
| 26 | 23 | ±2 | 20 | ±2 |
| 28 | 23 | ±2 | 20 | ±2 |
| 31 | 23 | ±2 | 20 | ±2 |
| 41 | 23 | ±2 | 20 | ±2 |
| 42 | 23 | ±2 | 20 | ±2 |
| 43 | 23 | ±2 | 20 | ±2 |
| 65 | 23 | ±2 | 20 | ±2 |
| 66 | 23 | ±2 | 20 | ±2 |
| 70 | 23 | ±2 | 20 | ±2 |
| 71 | 23 | ±2 | 20 | ±2 |
| 72 | 23 | ±2 | 20 | ±2 |
| 73 | 23 | ±2 | 20 | ±2 |
| 74 | 23 | ±2 | 20 | ±2 |
| 85 | 23 | ±2 | 20 | ±2 |
| 87 | 23 | ±2 | 20 | ±2 |
| 88 | 23 | ±2 | 20 | ±2 |
| 103 | 23 | ±2 | 20 | ±2 |

The normative reference for this requirement is TS 36.101 clause 6.2.2F.

[End of change]

#### 6.2.2F.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2F.5-1.

Table 6.2.2F.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 1 | 23 | ±2.7 | 20 | ±2.7 |
| 2 | 23 | ±2.7 | 20 | ±2.7 |
| 3 | 23 | ±2.7 | 20 | ±2.7 |
| 4 | 23 | ±2.7 | 20 | ±2.7 |
| 5 | 23 | ±2.7 | 20 | ±2.7 |
| 7 | 23 | ±2.7 | 20 | ±2.7 |
| 8 | 23 | ±2.7 | 20 | ±2.7 |
| 11 | 23 | ±2.7 | 20 | ±2.7 |
| 12 | 23 | ±2.7 | 20 | ±2.7 |
| 13 | 23 | ±2.7 | 20 | ±2.7 |
| 14 | 23 | ±2.7 | 20 | ±2.7 |
| 17 | 23 | ±2.7 | 20 | ±2.7 |
| 18 | 23 | ±2.7 | 20 | ±2.7 |
| 19 | 23 | ±2.7 | 20 | ±2.7 |
| 20 | 23 | ±2.7 | 20 | ±2.7 |
| 21 | 23 | ±2.7 | 20 | ±2.7 |
| 25 | 23 | ±2.7 | 20 | ±2.7 |
| 26 | 23 | ±2.7 | 20 | ±2.7 |
| 28 | 23 | ±2.7 | 20 | ±2.7 |
| 31 | 23 | ±2.7 | 20 | ±2.7 |
| 41 | 23 | ±2.7 | 20 | ±2.7 |
| 42 | 23 | ±3.0 | 20 | ±3.0 |
| 43 | 23 | ±3.0 | 20 | ±3.0 |
| 65 | 23 | ±2.7 | 20 | ±2.7 |
| 66 | 23 | ±2.7 | 20 | ±2.7 |
| 70 | 23 | ±2.7 | 20 | ±2.7 |
| 71 | 23 | ±2.7 | 20 | ±2.7 |
| 72 | 23 | ±2.7 | 20 | ±2.7 |
| 73 | 23 | ±2.7 | 20 | ±2.7 |
| 74 | 23 | ±2.7 | 20 | ±2.7 |
| 85 | 23 | ±2.7 | 20 | ±2.7 |
| 87 | 23 | ±2.7 | 20 | ±2.7 |
| 88 | 23 | ±2.7 | 20 | ±2.7 |
| 103 | 23 | ±2.7 | 20 | ±2.7 |

[Start of change]

6.2.2FA.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2FA.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2.2FA.3-1: UE Power Class 6

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NA | NA | NA | NA | 14 | ±2.5 |
| 2 | NA | NA | NA | NA | 14 | ±2.5 |
| 3 | NA | NA | NA | NA | 14 | ±2.5 |
| 4 | NA | NA | NA | NA | 14 | ±2.5 |
| 5 | NA | NA | NA | NA | 14 | ±2.5 |
| 7 | NA | NA | NA | NA | 14 | ±2.5 |
| 8 | NA | NA | NA | NA | 14 | ±2.5 |
| 11 | NA | NA | NA | NA | 14 | ±2.5 |
| 12 | NA | NA | NA | NA | 14 | ±2.5 |
| 13 | NA | NA | NA | NA | 14 | ±2.5 |
| 14 | NA | NA | NA | NA | 14 | ±2.5 |
| 17 | NA | NA | NA | NA | 14 | ±2.5 |
| 18 | NA | NA | NA | NA | 14 | ±2.5 |
| 19 | NA | NA | NA | NA | 14 | ±2.5 |
| 20 | NA | NA | NA | NA | 14 | ±2.5 |
| 21 | NA | NA | NA | NA | 14 | ±2.5 |
| 25 | NA | NA | NA | NA | 14 | ±2.5 |
| 26 | NA | NA | NA | NA | 14 | ±2.5 |
| 28 | NA | NA | NA | NA | 14 | ±2.5 |
| 31 | NA | NA | NA | NA | 14 | ±2.5 |
| 41 | NA | NA | NA | NA | 14 | ±2.5 |
| 42 | NA | NA | NA | NA | 14 | ±2.5 |
| 43 | NA | NA | NA | NA | 14 | ±2.5 |
| 65 | NA | NA | NA | NA | 14 | ±2.5 |
| 66 | NA | NA | NA | NA | 14 | ±2.5 |
| 70 | NA | NA | NA | NA | 14 | ±2.5 |
| 71 | NA | NA | NA | NA | 14 | ±2.5 |
| 72 | NA | NA | NA | NA | 14 | ±2.5 |
| 73 | NA | NA | NA | NA | 14 | ±2.5 |
| 74 | NA | NA | NA | NA | 14 | ±2.5 |
| 85 | NA | NA | NA | NA | 14 | ±2.5 |
| 87 | NA | NA | NA | NA | 14 | ±2.5 |
| 88 | NA | NA | NA | NA | 14 | ±2.5 |
| 103 | NA | NA | NA | NA | 14 | ±2.5 |

The normative reference for this requirement is TS 36.101 clause 6.2.2F.

[End of change]

[Start of change]

#### 6.2.2FA.5	Test requirements

Same test requirements as in clause 6.2.2F.5 with the following exceptions:

-	Instead of Table 6.2.2F.5-1  use Table 6.2.2FA.5-1.

Table 6.2.2FA.5-1: UE Power Class test requirement for Power Class 6

| EUTRA band | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 14 | ±3.2 |
| 2 | 14 | ±3.2 |
| 3 | 14 | ±3.2 |
| 4 | 14 | ±3.2 |
| 5 | 14 | ±3.2 |
| 7 | 14 | ±3.2 |
| 8 | 14 | ±3.2 |
| 11 | 14 | ±3.2 |
| 12 | 14 | ±3.2 |
| 13 | 14 | ±3.2 |
| 14 | 14 | ±3.2 |
| 17 | 14 | ±3.2 |
| 18 | 14 | ±3.2 |
| 19 | 14 | ±3.2 |
| 20 | 14 | ±3.2 |
| 21 | 14 | ±3.2 |
| 25 | 14 | ±3.2 |
| 26 | 14 | ±3.2 |
| 28 | 14 | ±3.2 |
| 31 | 14 | ±3.2 |
| 41 | 14 | ±3.2 |
| 42 | 14 | ±3.5 |
| 43 | 14 | ±3.5 |
| 65 | 14 | ±3.2 |
| 66 | 14 | ±3.2 |
| 70 | 14 | ±3.2 |
| 71 | 14 | ±3.2 |
| 72 | 14 | ±3.2 |
| 73 | 14 | ±3.2 |
| 74 | 14 | ±3.2 |
| 85 | 14 | ±3.2 |
| 87 | 14 | ±3.2 |
| 88 | 14 | ±3.2 |
| 103 | 14 | ±3.2 |

[End of change]