3GPP TSG-RAN5 Meeting #96-e	R5-225342

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2492 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap default test channel bandwidth for signaling MFBI test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1,Max bandwidth for RedCap UE is 20MHz for FR1 band, and set initial BWP for RedCap in SIB1 will impact TTCN implement so reducing the bandwidth of Cell is needed for RedCap signalling test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Reduce the n77,n78 CBW to 20Mhz only for RedCap UE. 2. New tables are added for RedCap UE and text also update to introduce these new tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE could not test n77,n78 MFBI test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-224726r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2.3.5	Test frequencies for MFBI signalling testing

For signalling test cases, the mapping of MFBI frequency ranges to NR test frequencies are as follows: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3).

The test frequencies, subcarrier spacing, default channel bandwidth, SS/PBCH block and CORESET#0 parameters for signalling are specified in Table 6.2.3.5-1 and Table 6.2.3.5-1A(for RedCap UE).

Table 6.2.3.5-1: Test frequencies for MFBI NR bands in FR1 for non-RedCap UE

| NR Band | MFBI overlapping NR Band | SCS [kHz] | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | n25 | 15 | 10 | 52 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n25 | n2 | 15 | 10 | 52 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.2 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n38 | n41 | 15 | 10 | 52 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.38 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n41 | n38 | 15 | 10 | 52 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.38 for bandwidth=10 MHz and SCS=15 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n77 | n78 | 30 | 100 | 273 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n78 | n77 | 30 | 100 | 273 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=100 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-1A: Test frequencies for MFBI NR bands in FR1 for RedCap UE

| NR Band | MFBI overlapping NR Band | SCS [kHz] | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | n25 | Same values as in Table 6.2.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n25 | n2 | Same values as in Table 6.2.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n38 | n41 | Same values as in Table 6.2.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n41 | n38 | Same values as in Table 6.2.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | n78 | 30 | 20 | 51 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |
| n78 | n77 | 30 | 20 | 51 | Downlink & Uplink | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.1.1.78 for bandwidth=20 MHz and SCS=30 kHz. |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>