3GPP TSG-RAN5 Meeting #96-e	R5-225017

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.918 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TR 38.918 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The technical report is incomplete and needs to be updated according to work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of test procedures A.3.3.1 and A.3.3.2 - Addition of test parameters in A.3.1 and A.3.2 - Updates to the conclusion in Chapter 6 - Addition of References in Chapter 2 - Addition of Impact of Test System Uncertainty in 5.5.1.3 - Removal of editor notes in several clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The technical report is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5.3.2, 5.4.2, 5.4.3, 5.5.1.3, 6, A.2.2.6, A.3, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5] 	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[6] 	3GPP TS 38.508-1: "5GS;User Equipment (UE) conformance specification; Part 1: Common test environment".

[7] 	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[9] 	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); stage 3".

[10]	3GPP TS 24.526: "UE policies for 5G System (5GS); Stage 3".

[11] 	3GPP TS 23.003: "Numbering, addressing and identification".

[12]	3GPP TS 38.521-4: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements".

[13]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Symbols

Void

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

< Unchanged sections omitted >

5.3	Application Simulation

5.3.1	Application Client Simulator

To enable testing an Application Client Simulator on device is used to simulate the application which configured with network slicing. The following functionality is recommended for the Application Client Simulator to support.

-	The Application Client Simulator is installed on device.

-	The Application Client Simulator may start automatically on device power-up or manually be started by the operator at the beginning of the test campaign.

-	The Application Client Simulator could receive the type of traffic descriptor to be tested and generate the corresponding traffic descriptor which conform to Network Slicing Configurations defined in clause 5.2.3.

-	The Application Client Simulator could generate data packets matching the traffic descriptor in Network Slicing Configurations and transfer data packets to operating system and communication unit on device.

-	The Application Client Simulator could directly send the generated data packets to application server simulator via cable connection.

5.3.2	Test Models

There is no specific Test Models identified at this time.

5.4	Statistical Analysis

5.4.1	Overview of Mapping Application to Network Slicing

When the mapping of applications to network slicing is tested, the fundamental goal is to verify the procedure of associating applications to PDU sessions based on URSP rules. URSP rules contain Traffic Descriptors determining which URSP rule is applicable for the specific application and a list of Route Selection Descriptors determining the corresponding PDU session.

With this knowledge, the following aspects are recommended to be checked to verify the procedure of mapping application to Network Slicing:

-	The selection of applicable URSP rule according to the Traffic descriptors in UE Route Selection Policy Rule within URSP rules and information from the specific application.

-	The selection of existing PDU session or establishment of new PDU session for specific application based on the selected Route Selection Descriptors within the applicable URSP rule.

The way to trigger URSP rule selection is UE implementation specific and driven by application. To enable testing then the UE is recommended to be equipped with the application simulation client defined in clause 5.3.1 to trigger the required selection procedures

5.4.2	Overview of Application Layer Throughput With Network Slicing

In general, the application layer throughput is impacted by variations mainly at the physical layer and MAC layer. Thus, a variety of test points should be picked across different physical layer conditions (doppler, signal-to-noise ratio, antenna configurations, etc.).

There can be multiple usages of network slicing with different needs such as mission critical ultra-reliable low latency slice, massive machine to machine type communication slice, extreme high throughput mobile broadband slice. Depending on the use case, channel requirements will be different and accordingly the application layer performance will also be benchmarked.

During the application layer throughput measurement, ACK, NACK and DTX can also be recorded in order to calculate the L1 payload bit throughput, although this is not the main target of the test procedure. The measured throughput and its comparison to an expected throughput value can be evaluated. However, the parameters for this measurement are not as controlled as in the case of L1 throughput.

For fixed reference channel testing, the UE is compared against a predefined limit at the physical layer. Therefore, statistics can be derived to determine the minimum number of samples for a given confidence level for the pass/fail decision. While the application layer data throughput is of a statistical nature, in case of fixed reference channel testing, it is possible to set a lower bound for the achievable throughput by computing the overhead due to upper layer payload headers.

5.4.3	Overview of Application Layer Latency With Network Slicing

In order to measure the application layer latency for a Standardized Slice Type (SST) or a non-standardized slice type, some important factors which needs to be considered may involve network node delay in time, jitter experienced based on channel bandwidth and network propagation conditions. Each such conditions can be evaluated against the QoS requirements and delay sensitivity of the data being transmitted. The end goal here is to optimize the utilization of the allocation of network resources and the quality of service metrics.

For fixed or variable reference channel and an SST, the UE latency for each data traffic session per slice needs to be captured and evaluated against the required maximum delay in reception of the desired data packet. Based on the measured packet delay and jitter experienced e2e evaluation of the layer latency can be derived and benchmarked. Important aspect here to consider would be to adjust the tolerance +/- of the latency and jitter in percentage terms to define a final evaluation criterion as per testing needs and network traffic type usage.

5.5	Test System Uncertainty and Test Tolerance

5.5.1	Test System Uncertainty and Test Tolerance for FR1 testing

5.5.1.1	Recommended Uncertainty of Test System

For service performance test procedure in Annex A, the test system should fulfil the 3GPP test system uncertainty values specified in Annex F of TS 38.521-4 [12]. If a test system cannot fulfil the 3GPP test system uncertainty requirements, then the test system vendor shall declare its test system uncertainty values.

5.5.1.2	Test Tolerances

Since there are no absolute minimum requirements nor PASS/FAIL requirements in tests specified in the present TR the test tolerances are not defined which should be understood as the applicable test tolerance being set to zero in all tests. If PASS/FAIL requirements are recommended, appropriate analysis of test tolerance shall be considered.

5.5.1.3	Impact of Test System Uncertainty on Test Results

Test system uncertainties play a big role in application layer throughput results. The tighter the uncertainty requirements are the more re-producible and comparable the results are.

In TS 38.521-4 [12] applicable test system uncertainty has been specified. Test System Uncertainty is a measure how accurately tester can setup the certain parameter/signal level to the specified level. In5G NR UE Full Stack Testing for Network Slicing, for service performance test procedure the most meaningful test system uncertainties are listed in Annex F of 38.521-4[12].

These specified test system uncertainties are very tight requirements for test systems. Typically the specified uncertainty values are the best that test system vendors can achieve when their test systems are fully calibrated. Full calibration means that each individual device, signal route and cable has to be calibrated. Hence the calibration costs take quite a big share of total costs of 3GPP compliant test systems.

6	Conclusions

The UE full stack testing for network slicing study item was initiated by RAN5 to include UE full stack network slicing capability. The following aspects were included as part of the present document:

Definition of UE Full Stack Testing for Network Slicing

Test configurations

Application Simulation

Statistical analysis

Test uncertainty and test tolerance

The following items have been identified at the conclusion of the study item

Definition of UE Full Stack Testing for Network Slicing and identification

Identification of the test configuration to include test equipment, test equipment connection diagrams, network slicing configurations, message and information elements and UE specific items

Identification of the application client simulator

Completion of the analysis for mapping between application to network slicing, application throughput and application latency with network slicing

Consideration about the impact of test system uncertainty on test results and confirmation of the Test System Uncertainty Recommendations

Identification of the test procedures for URSP Configuration and application mapping to network slicing

Identification of the test procedure for service performance testing with network slicing

The present document includes a set of recommended test procedures for UE supporting network slicing that is in-line with the study item objectives identified in clause 4.

The test procedures are contained in Annex A.

< Unchanged sections omitted >

A.2.2.6	5G NR / Mapping Application to Network Slicing / URSP Update

A.2.2.6.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.2.2.6.2	Test Purpose

To verify that UE could support the update of URSP rules and the mapping of applications to right network slicing based on the updated URSP rules.

A.2.2.6.3	Test Parameters

Unless otherwise stated, refer to the test frequency and common test parameters for signalling conformance testing defined in TS 38.508-1 [6].

A.2.2.6.4	Test Description

A.2.2.6.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.2.2.6.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.1.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

7.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

8.	The UE transmits an UL NAS TRANSPORT message including a MANAGE UE POLICY COMPLETE message.

9.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

10.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

11.	Observe that the data received in Application Server Simulator via PDU session is consistent with the data generated by Application Client Simulator.

Table A.2.2.6.4.2-1: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | 1 |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

Table A.2.2.6.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 10)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | 1 |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 10 |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.6.4.2-3: MANAGE UE POLICY COMMAND (step 7)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000003 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.2.2.6.4.2-4: UL NAS Transport (step 9)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | 1 |  |  |
| SD | 0x000003 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

A.3	5G NR / Service Performance Testing with Network Slicing

A.3.1	5G NR / Service Performance / Single Network Slicing

A.3.1.1	5G NR / Service Performance / Single Application with Single Network Slicing

A.3.1.1.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.1.2	Test Purpose

To measure the performance of single service in application layer of the 5G NR UE with single network slicing.

A.3.1.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.1.1.4	Test Description

A.3.1.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.1.4.2	Test Procedure

1.	UE is switched on.

2.	Steps 2 to 15 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed.

3.	Using the Application Client Simulator, begin uplink data transfer. Wait for 15 seconds and then start to measure the throughput result and latency in [transport layer]. (This is iteration 1) Continue data transfer for 1 minutes.

4.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

5.	Calculate and record the average application layer data throughput and latency across three iterations.

6.	Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed.

7.	UE is brought back to operation.

8.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the MANAGE UE POLICY COMMAND message is defined in Table A.3.1.1.4.2-1.

9.	Using the Application Client Simulator, generate traffic matching DNN value in Traffic descriptor in URSP rule provisioned to the UE in step 8.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

13.	Wait for 15 seconds and then start to measure the throughput result and latency in [transport layer] in parallel (This is iteration 1). Continue data transfer for 1 minute.

14.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

15. 	Calculate and record the average application layer data throughput and latency across three iterations.

16.	Observe that the throughput in step 15 is not lower than the throughput benchmark in step 5 and the latency in step 15 is not higher than the latency benchmark in step 5.

Table A.3.1.1.4.2-1: MANAGE UE POLICY COMMAND (step 8)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| Route selection descriptor 2 |  |  |  |  |
| Precedence value of route selection descriptor |  | 1 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000100'B |  | DNN type |
| Route selection descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

A.3.1.2	5G NR / Service Performance / Multiple Applications with Single Network Slicing

A.3.1.2.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with single network slicing.

A.3.1.2.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. For single APP A at step 5-8 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block. For APP A and APP B running simultaneously at step 9-11 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.1.2.4	Test Description

A.3.1.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  MANAGE UE POLICY COMMAND message is defined in Table A.3.2.1.4.2-2.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching DNN value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in application layer(This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

10.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator.

11.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

12.	Observe that the throughput in step 11 is not lower than the throughput benchmark in step 8.

Table A.3.1.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.1.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | 1 |  |  |
| SD | 0x000001 |  |  |  |

Table A.3.1.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | 1 |  |
| SD |  | 0x000001 |  |  |

A.3.2	5G NR / Service Performance / Multiple Network Slicing

A.3.2.1	5G NR / Service Performance / Multiple Applications with Multiple Network Slicing

A.3.2.1.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.2.1.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with multiple network slicing.

A.3.2.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. For single APP A or APP B at step 8-15 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block. For APP A and APP B running simultaneously at step 16-19 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.2.1.4	Test Description

A.3.2.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.2.1.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  REGISTRATION ACCEPT message is defined in Table A.3.2.1.4.2-1 and MANAGE UE POLICY COMMAND message is defined in Table A.3.2.1.4.2-2.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching DNN value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP B, generate traffic matching DNN value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

6.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI and DNN value are selected.

7.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

8.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator.

9.	Wait for 15 seconds and then start to measure the throughput result in application layer (This is iteration 1). Continue data transfer for 1 minute.

10.	Repeat step 9 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

11.	Calculate and record the average application layer data throughput across three iterations.

12.	Stop the data transfer with simulated APP A.

13.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator.

14.	Wait for 15 seconds and then start to measure the latency in application layer (This is iteration 1). Continue data transfer for 1 minute.

15.	Repeat step 14 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

16.	Calculate and record the average application layer latency across three iterations.

17.	Restart the data traffic of APP A. APP A and APP B generate data traffic simultaneously.

18.	Repeat steps 9 to 11 to get the average application layer data throughput of the data traffic of simulated APP A

19.	Repeat steps 14 to 16 to get the average application layer latency of the data traffic of simulated APP B in parallel with step 18.

20.	Observe that the throughput in step 18 is not lower than the throughput benchmark in step 11 and the latency in step 19 is not higher than the latency benchmark in step 16.

Table A.3.2.1.4.2-1: REGISTRATION ACCEPT (step 1)

| Derivation path: TS 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result value |  | '001'B | 3GPP access |  |
| Allowed NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | default S-NSSAI |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |
| Configured NSSAI |  |  |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 1 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | S-NSSAI value 2 |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| S-NSSAI IEI |  |  | default S-NSSAI |  |
| Length of S-NSSAI contents |  | '00000100'B |  |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.2.1.4.2-2: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B | DNN Type |  |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B | DNN Type |  |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.2.1.4.2-3: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B | eMBB |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific |  |  |  |

Table A.3.2.1.4.2-4: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B | eMBB |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |

Table A.3.2.1.4.2-5: UL NAS Transport (step 6)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-2 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000010'B | URLLC |  |  |
| SD | 0x000001 |  |  |  |
| DNN |  |  |  |  |
| DNN length | Set to the actual length of 'DNN value' in bytes |  |  |  |
| DNN value | pc_APN_ID_Specific_2nd |  |  |  |

Table A.3.2.1.4.2-6: PDU SESSION ESTABLISHMENT ACCEPT (step 7)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B | URLLC |  |
| SD |  | 0x000001 |  |  |
| DNN |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific_2nd |  |  |

A.3.3	5G NR / Service Performance / URSP Update

A.3.3.1	5G NR / Service Performance / Single Network Slicing / URSP update

A.3.3.1.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.1.2	Test Purpose

To measure the performance of single services in application layer of the 5G NR UE with the change of network slicing.

A.3.3.1.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block for step 5-7. For APP A and APP B running simultaneously at step 16-19 configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for step 12-13.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.1.4	Test Description

A.3.3.1.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.1.4.2	Test Procedure

- Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1.

2.	Using the Application Client Simulator, generate traffic matching OS App Id in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in transport layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator, begin uplink data transfer from UE to the Application Server Simulator.

13.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

14.	Observe that the throughput in step 13 is not lower than the throughput benchmark in step 8.

Table A.3.3.1.4.2-1: UL NAS Transport (step 3, 10)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  |  |  |
| SD | 0x000001 |  | step 3 |  |
|  | 0x000003 |  | step 10 |  |

Table A.3.3.1.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 11)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 11 |

Table A.3.3.1.4.2-3: MANAGE UE POLICY COMMAND (step 9)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000003 |  |  |

A.3.3.2	5G NR / Service Performance / Multiple Network Slicing / URSP update

A.3.3.2.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with the change of network slicing.

A.3.3.2.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.2.4	Test Description

A.3.3.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.2.2.4.2	Test Procedure

1.     Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  REGISTRATION ACCEPT message is defined in Table A.3.2.1.4.2-1 and MANAGE UE POLICY COMMAND message is defined in Table A.3.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

6.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

7.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

8.	Using the simulated APP A and APP B, begin uplink data transfer from UE to the Application Server Simulator.

9.	Wait for 15 seconds and then start to measure the throughput result of simulated APP A and latency result of simulated APP B in application layer (This is iteration 1). Continue data transfer for 1 minute.

10.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

11.	Calculate and record the average application layer data throughput of simulated APP A and latency  of simulated APP B across three iterations.

12.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

13.	Repeat step 2-4 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

14.	Repeat step 5-7 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

15.	Repeat step 8-11 to get the average application layer data throughput of simulated APP A and latency of simulated APP B in parahell .

16.	Observe that the throughput in step 15 is not lower than the throughput benchmark in step 11 and the latency in step 15 is not higher than the latency benchmark in step 11.

Table A.3.2.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.2.2.4.2-2: UL NAS Transport (step 3, step 6)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  | step 3 |  |
|  | '00000010'B |  | step 6 |  |
| SD | 0x000001 |  |  |  |

Table A.3.2.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 7)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  | step 4 |
|  |  | '00000010'B |  | step 7 |
| SD |  | 0x000001 |  |  |

Table A.3.2.2.4.2-4: MANAGE UE POLICY COMMAND (step 12)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  |  |

Table A.3.2.2.4.2-5: UL NAS Transport (step 13, step 14)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  |  |  |
| SD | 0x000001 |  | step 13 |  |
|  | 0x000002 |  | step 14 |  |

Table A.3.2.2.4.2-6: PDU SESSION ESTABLISHMENT ACCEPT (step 13, step 14)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  | step 13 |
|  |  | 0x000002 |  | step 14 |

Annex B:
Applicability

Void

<End of modified section>