3GPP TSG-RAN5 Meeting #96-e	R5-224201

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3059 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current Prose, at step 22A of the test case, Prach configuration index and RaResponse window in the system information are set to 157 and sl40 respectively in case of TDD (Table 8.1.1.4.1.3.3-14) Howevever, according to 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. It is proposed (in line with abovespec reference) that RaResponse window be limited to sl20. According to current Prose, at Step 22A prach-ConfigurationIndex is set as 157 , for which Prach is expected to be received in the subframe = 4 ( every SFN ). If Prach is received in Subframe =4 and slot = 1 on even SFN , no issue is observed as there is no interference from SSB in the next SFN ( sf = 0 slot = 0 ) But If RACH is received in subframe = 4 & Slot = 1 of Odd SFN , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 (Table 8.1.1.4.1.3.3-14) for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 8.1.1.4.1.3.3-14 updated the following: - Ra-Response window is set as sl20 prach-ConfigurationIndex is set as 91 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s220929 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.1.4.1	RRC resume / Suspend-Resume / RNA update / Success / Short message for SI update

8.1.1.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state, with stored shortI-RNTI and useFullResumeID is not signalled in SIB1 }

ensure that {

when { UE receives a Paging message to resume RRC connection }

then { UE sends RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value }

}

(2)

with { UE in NR RRC_INACTIVE state, with stored fullI-RNTI and useFullResumeID is signalled in SIB1 }

ensure that {

when { UE receives a Paging message to resume RRC connection }

then { UE sends RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value }

}

(3)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { UE performs cell reselection and enters an RNA not belonging to the configured ran-NotificationAreaInfo }

then { UE shall initiate RRC connection resume procedure with cause value set to rna-Update }

}

(4)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { UE receives a short message transmitted on PDCCH using P-RNTI indicating a systemInfoModification }

then { UE re-acquires and applies the new system information about the correct prach-ConfigurationIndex in random access to resume RRC connection }

}

8.1.1.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.2.2.2.2, 5.2.2.3.1, 5.2.2.4.2, 5.3.8.3, 5.3.13.2, 5.3.13.3, 5.3.13.8 and 6.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.2.2.2.2]

A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS and positioning assistance data) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period. SI change indication is not applicable for SI messages containing posSIBs.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

…

If the UE receives a Short Message, the UE shall:

…

1>	if the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification period.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

2>	if the UE is unable to acquire the MIB;

3>	perform the actions as specified in clause 5.2.2.5;

2>	else:

3>	perform the actions specified in clause 5.2.2.4.1.

…

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

…

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

…

1>	else:

…

2>	if the UE supports the bandwidth of the initial uplink BWP and of the initial downlink BWPs indicated in the locationAndBandwidth fields in uplinkConfigCommon:

3>	select the first frequency band in the frequencyBandList which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

3>	forward the cellIdentity to upper layers;

3>	if trackingAreaCode is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	if intraFreqReselection is set to notAllowed:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20];

4>	else:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20];

3>	else:

4>	forward the trackingAreaCode to upper layers;

3>	forward the PLMN identity to upper layers;

3>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

4>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

5>	initiate an RNA update as specified in 5.3.13.8;

[TS 38.331, clause 5.3.8.3]

The UE shall:

…

1>	if the RRCRelease includes suspendConfig:

2>	apply the received suspendConfig;

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the C-RNTI with the temporary C-RNTI in the cell the UE has received the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

4>	replace the suspendConfig with the current suspendConfig;

2>	else:

3>	store in the UE Inactive AS Context the configured suspendConfig, the current KgNB and KRRCint keys, the ROHC state, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and all other parameters configured except with ReconfigurationWithSync;

…

[TS 38.331, clause 5.3.13.2]

The UE initiates the procedure when upper layers or AS (when responding to RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to true;

4>	the procedure ends;

…

1>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

1>	apply the default SRB1 configuration as specified in 9.2.1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	set the variable pendingRnaUpdate to false;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

[TS 38.331, clause 5.3.13.3]

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

…

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

…

[TS 38.331, clause 5.3.13.8]

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

[TS 38.331, clause 6.5]

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

8.1.1.4.1.3	Test Description

8.1.1.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell and NR Cell 2 is a suitable neighbour intra-frequency cell.

-	Cell power levels are selected according to 38.508-1 [4] Table 6.2.2.1-3 and NR Cell 2 is switched on after UE has registered on NR Cell 1.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 2N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 2N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.1.4.1.3.2	Test procedure sequence

Table 8.1.1.4.1.3.2-1 for FR1 and table 8.1.1.4.1.3.2-2 for FR2 illustrate the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.1.4.1.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -94 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3 |

Table 8.1.1.4.1.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
|  | Qrxlevmin | dBm | -110+Delta(NRf1) | -110+Delta(NRf1) |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -91 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2 |
|  | Qrxlevmin | dBm | - | -110+Delta(NRf1) |  |

Table 8.1.1.4.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise | - | - | - | - |
| 1 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 3 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 4 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? | - | - | 1 | - |
| 6 | The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 7 | The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. | - | - | - | - |
| 8 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 9 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 10 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 11 | Check: Does the UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? | --> | NR RRC: RRCResumeRequest1 | 2 | P |
| 12 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 13 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 14 | The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS adjusts cell levels according to row T1 of table 8.1.1.4.1.3.2-1 for FR1 and table 8.1.1.4.1.3.2-2 for FR2. | - | - | - | - |
| - | The following messages are to be observed on NR Cell 2 unless explicitly stated otherwise. | - | - | - | - |
| 16 | Check: Does the UE transmit an RRCResumeRequest message with resumeCause set to rna-Update on NR Cell 2 within 60s? | --> | NR RRC: RRCResumeRequest | 3 | P |
| 17 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 18 | The UE transmit an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 19 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? | - | - | 3 | - |
| 20 | The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 21-22 | Void | - | - | - | - |
| 22A | The SS changes the prach-ConfigurationIndex and ra-responseWindow in the system information. | - | - | - | - |
| 22B | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 23 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 24 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | - | NR RRC: Paging | - | - |
| 25 | Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 22? | - | - | 4 | P |
| 26 | The UE transmits an RRCResumeRequest message. | --> | RRCResumeRequest | - | - |
| 27 | The SS transmits an RRCResume message. | <-- | RRCResume | - | - |
| 28 | The UE transmits an RRCResumeComplete message. | --> | RRCResumeComplete | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

8.1.1.4.1.3.3	Specific message contents

Table 8.1.1.4.1.3.3-1: Paging (step 1, Table 8.1.1.4.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.1.4.1.3.3-2: RRCRelease (step 6, 13, Table 8.1.1.4.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 8.1.1.4.1.3.3-3: SIB1 (step 7, Table 8.1.1.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| useFullResumeID | true |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-4: Paging (step 10 and 24, Table 8.1.1.4.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.1.4.1.3.3-5: RRCResumeRequest (step 16, Table 8.1.1.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | rna-Update |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-6: RRCResume (step 3, 12, Table 8.1.1.4.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-17

Table 8.1.1.4.1.3.3-7: Void

Table 8.1.1.4.1.3.3-8: RRCResume (step 17, Table 8.1.1.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| masterCellGroup := SEQUENCE { |  | CellGroupConfig | Table 8.1.1.4.1.3.3-10 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.1.4.1.3.3-9: Void

Table 8.1.1.4.1.3.3-10: CellGroupConfig (Table 8.1.1.4.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition RESUME |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-11: ServingCellConfig (Table 8.1.1.4.1.3.3-10: CellGroupConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition RESUME |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-12: BWP-DownlinkDedicated (Table 8.1.1.4.1.3.3-11: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-13: PDSCH-Config (Table 8.1.1.4.1.3.3-12: BWP-DownlinkDedicated)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| dmrs-DownlinkForPDSCH-MappingTypeA | Not present |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 1 entry |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | 0 |  |  |
| qcl-type1 SEQUENCE { |  |  |  |
| referenceSignal CHOICE { |  |  |  |
| ssb | SSB index #n | SSB index of NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.4.1.3.3-14: SIB1 (step 22A, Table 8.1.1.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 91 |  | FR1 |
|  | 156 |  | FR2 |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |