3GPP TSG-RAN5 Meeting #96-e	R5-224648

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1918 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Starting PRB index for ULBWP.1.3 in A.8.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A.8.2 is not aligned with TS 38.133. The note number for RBa in ULBWP.1.3 is 2 in TS 38.133 A.3.9 but is 1 in TS 38.533 A.8.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Core spec alignment: Note for RBa in ULBWP.1.3 was corrected to Note 2 in Table A.8.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance Test cannot be correctly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

A.8	Reference BWP configurations

This clause provides the typical BWP configurations used for RRM test cases defined in this test specification. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.8.1 and for uplink BWP, dedicated BWP configurations are specified in clause A.8.2. Parameters not listed in this clause can be derived from the configuration of each test.

A.8.1	Downlink BWP configurations

Table A.8.1-1 defines the different downlink initial BWP configurations. Table A.8.1-2 defines the different downlink dedicated BWP configurations.

Table A.8.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth |  | Same as RF channel defined in each test | same as RMSI CORSET(CORESET #0) defined in each test |  |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |  |  |

Table A.8.1-2: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 | DLBWP.1.2 |  |  |  | DLBWP.1.3 |  |  |  | DLBWP.1.4 |  |
| Starting PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |

A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  |  | ULBWP.1.3 |  |  |  | ULBWP.1.4 |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |

<<End of change>>