3GPP TSG-RAN5 Meeting #96-e	R5-225366

# Online, , 15th August 2022 - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2428 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE Power Limit Messages: Option 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Message type value for POWER LIMIT test functions is not aligned with definition in TS 38.509 according to R5-224088 and R5-224089. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Redefine message type value for POWER LIMIT test functions. - Redefine parameters for ACTIVATE POWER LIMIT REQUEST message to also consider unequal bandwidth among all component carriers within the aggregated bandwidth. - Remove editor’s notes in sections 4.7A.7 and 4.9.32 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL-CA tests blocked from execution due to SCell Drop Issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A.7, 4.9.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-224078, R5-224084, R5-224085, R5-224088 and R5-224089. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-224093 and the outcome of 3 revisions. |  |  |  |  |  |  |  |  |

<Unchanged Sections skipped>

#### NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

### 4.7A.7	UE Power Limit Messages

#### -	ACTIVATE POWER LIMIT REQUEST

This Message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 0 |  | TOT NR AGG BW 100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 1 |  | TOT NR AGG BW 150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 0 |  | TOT NR AGG BW 200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 1 |  | TOT NR AGG BW 250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 0 |  | TOT NR AGG BW 300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 1 |  | TOT NR AGG BW 350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 0 |  | TOT NR AGG BW 400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 1 |  | TOT NR AGG BW 450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 0 |  | TOT NR AGG BW 500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 1 |  | TOT NR AGG BW 550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 0 |  | TOT NR AGG BW 600 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 1 |  | TOT NR AGG BW 650 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 0 |  | TOT NR AGG BW 700 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 1 |  | TOT NR AGG BW 750 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 0 |  | TOT NR AGG BW 800 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 1 |  | TOT NR AGG BW 850 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 0 |  | TOT NR AGG BW 900 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 1 |  | TOT NR AGG BW 950 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 0 |  | TOT NR AGG BW 1000 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 1 |  | TOT NR AGG BW 1050 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 0 |  | TOT NR AGG BW 1100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 1 |  | TOT NR AGG BW 1150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 0 |  | TOT NR AGG BW 1200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 1 |  | TOT NR AGG BW 1250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 0 |  | TOT NR AGG BW 1300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 1 |  | TOT NR AGG BW 1350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 0 |  | TOT NR AGG BW 1400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 1 |  | TOT NR AGG BW 1450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 0 |  | TOT NR AGG BW 1500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 1 |  | TOT NR AGG BW 1550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 1 0 0 0 0 0 |  | TOT NR AGG BW 1600 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 0 1 |  | PCELL NR BW 50 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 1 0 |  | PCELL NR BW 100 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 1 0 0 |  | PCELL NR BW 200 |
| PCELL NR BANDWIDTH | 0 0 0 0 1 0 0 0 |  | PCELL NR BW 400 |

| Condition | Explanation |
| --- | --- |
| TOT NR AGG BW 100 | Total NR aggregated Bandwidth equal to 100 MHz |
| TOT NR AGG BW 150 | Total NR aggregated Bandwidth equal to 150 MHz |
| TOT NR AGG BW 200 | Total NR aggregated Bandwidth equal to 200 MHz |
| TOT NR AGG BW 250 | Total NR aggregated Bandwidth equal to 250 MHz |
| TOT NR AGG BW 300 | Total NR aggregated Bandwidth equal to 300 MHz |
| TOT NR AGG BW 350 | Total NR aggregated Bandwidth equal to 350 MHz |
| TOT NR AGG BW 400 | Total NR aggregated Bandwidth equal to 400 MHz |
| TOT NR AGG BW 450 | Total NR aggregated Bandwidth equal to 450 MHz |
| TOT NR AGG BW 500 | Total NR aggregated Bandwidth equal to 500 MHz |
| TOT NR AGG BW 550 | Total NR aggregated Bandwidth equal to 550 MHz |
| TOT NR AGG BW 600 | Total NR aggregated Bandwidth equal to 600 MHz |
| TOT NR AGG BW 650 | Total NR aggregated Bandwidth equal to 650 MHz |
| TOT NR AGG BW 700 | Total NR aggregated Bandwidth equal to 700 MHz |
| TOT NR AGG BW 750 | Total NR aggregated Bandwidth equal to 750 MHz |
| TOT NR AGG BW 800 | Total NR aggregated Bandwidth equal to 800 MHz |
| TOT NR AGG BW 850 | Total NR aggregated Bandwidth equal to 850 MHz |
| TOT NR AGG BW 900 | Total NR aggregated Bandwidth equal to 900 MHz |
| TOT NR AGG BW 950 | Total NR aggregated Bandwidth equal to 950 MHz |
| TOT NR AGG BW 1000 | Total NR aggregated Bandwidth equal to 1000 MHz |
| TOT NR AGG BW 1050 | Total NR aggregated Bandwidth equal to 1050 MHz |
| TOT NR AGG BW 1100 | Total NR aggregated Bandwidth equal to 1100 MHz |
| TOT NR AGG BW 1150 | Total NR aggregated Bandwidth equal to 1150 MHz |
| TOT NR AGG BW 1200 | Total NR aggregated Bandwidth equal to 1200 MHz |
| TOT NR AGG BW 1250 | Total NR aggregated Bandwidth equal to 1250 MHz |
| TOT NR AGG BW 1300 | Total NR aggregated Bandwidth equal to 1300 MHz |
| TOT NR AGG BW 1350 | Total NR aggregated Bandwidth equal to 1350 MHz |
| TOT NR AGG BW 1400 | Total NR aggregated Bandwidth equal to 1400 MHz |
| TOT NR AGG BW 1450 | Total NR aggregated Bandwidth equal to 1450 MHz |
| TOT NR AGG BW 1500 | Total NR aggregated Bandwidth equal to 1500 MHz |
| TOT NR AGG BW 1550 | Total NR aggregated Bandwidth equal to 1550 MHz |
| TOT NR AGG BW 1600 | Total NR aggregated Bandwidth equal to 1600 MHz |
| PCELL NR BW 50 | PCC NR Bandwidth equal to 50 MHz |
| PCELL NR BW 100 | PCC NR Bandwidth equal to 100 MHz |
| PCELL NR BW 200 | PCC NR Bandwidth equal to 200 MHz |
| PCELL NR BW 400 | PCC NR Bandwidth equal to 400 MHz |

#### ACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 1 |  |  |

#### DEACTIVATE POWER LIMIT REQUEST

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 1 0 0 0 0 |  |  |

#### DEACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 1 0 0 0 1 |  |  |

<Unchanged sections skipped>

### 4.9.32	Test procedure to activate UE Power Limit Function (UPLF)

4.9.32.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state with UE power limit test function activated.

4.9.32.2	Procedure

Table 4.9.32.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to activate UE Power Limit Function | <-- | ACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is activated | --> | ACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.32.3	Specific Message contents

- Table 4.9.32.3-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 0 |  | TOT NR AGG BW 100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 |  | TOT NR AGG BW 150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 0 |  | TOT NR AGG BW 200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 1 |  | TOT NR AGG BW 250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 0 |  | TOT NR AGG BW 300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 1 |  | TOT NR AGG BW 350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 0 |  | TOT NR AGG BW 400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 1 |  | TOT NR AGG BW 450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 0 |  | TOT NR AGG BW 500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 1 |  | TOT NR AGG BW 550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 0 |  | TOT NR AGG BW 600 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 1 |  | TOT NR AGG BW 650 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 0 |  | TOT NR AGG BW 700 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 1 |  | TOT NR AGG BW 750 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 0 |  | TOT NR AGG BW 800 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 1 |  | TOT NR AGG BW 850 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 0 |  | TOT NR AGG BW 900 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 1 |  | TOT NR AGG BW 950 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 0 |  | TOT NR AGG BW 1000 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 1 |  | TOT NR AGG BW 1050 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 0 |  | TOT NR AGG BW 1100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 1 |  | TOT NR AGG BW 1150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 0 |  | TOT NR AGG BW 1200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 1 |  | TOT NR AGG BW 1250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 0 |  | TOT NR AGG BW 1300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 1 |  | TOT NR AGG BW 1350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 0 |  | TOT NR AGG BW 1400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 1 |  | TOT NR AGG BW 1450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 0 |  | TOT NR AGG BW 1500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 1 |  | TOT NR AGG BW 1550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 1 0 0 0 0 0 |  | TOT NR AGG BW 1600 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 0 1 |  | PCELL NR BW 50 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 1 0 |  | PCELL NR BW 100 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 1 0 0 |  | PCELL NR BW 200 |
| PCELL NR BANDWIDTH | 0 0 0 0 1 0 0 0 |  | PCELL NR BW 400 |

| Condition | Explanation |
| --- | --- |
| TOT NR AGG BW 100 | Total NR aggregated Bandwidth equal to 100 MHz |
| TOT NR AGG BW 150 | Total NR aggregated Bandwidth equal to 150 MHz |
| TOT NR AGG BW 200 | Total NR aggregated Bandwidth equal to 200 MHz |
| TOT NR AGG BW 250 | Total NR aggregated Bandwidth equal to 250 MHz |
| TOT NR AGG BW 300 | Total NR aggregated Bandwidth equal to 300 MHz |
| TOT NR AGG BW 350 | Total NR aggregated Bandwidth equal to 350 MHz |
| TOT NR AGG BW 400 | Total NR aggregated Bandwidth equal to 400 MHz |
| TOT NR AGG BW 450 | Total NR aggregated Bandwidth equal to 450 MHz |
| TOT NR AGG BW 500 | Total NR aggregated Bandwidth equal to 500 MHz |
| TOT NR AGG BW 550 | Total NR aggregated Bandwidth equal to 550 MHz |
| TOT NR AGG BW 600 | Total NR aggregated Bandwidth equal to 600 MHz |
| TOT NR AGG BW 650 | Total NR aggregated Bandwidth equal to 650 MHz |
| TOT NR AGG BW 700 | Total NR aggregated Bandwidth equal to 700 MHz |
| TOT NR AGG BW 750 | Total NR aggregated Bandwidth equal to 750 MHz |
| TOT NR AGG BW 800 | Total NR aggregated Bandwidth equal to 800 MHz |
| TOT NR AGG BW 850 | Total NR aggregated Bandwidth equal to 850 MHz |
| TOT NR AGG BW 900 | Total NR aggregated Bandwidth equal to 900 MHz |
| TOT NR AGG BW 950 | Total NR aggregated Bandwidth equal to 950 MHz |
| TOT NR AGG BW 1000 | Total NR aggregated Bandwidth equal to 1000 MHz |
| TOT NR AGG BW 1050 | Total NR aggregated Bandwidth equal to 1050 MHz |
| TOT NR AGG BW 1100 | Total NR aggregated Bandwidth equal to 1100 MHz |
| TOT NR AGG BW 1150 | Total NR aggregated Bandwidth equal to 1150 MHz |
| TOT NR AGG BW 1200 | Total NR aggregated Bandwidth equal to 1200 MHz |
| TOT NR AGG BW 1250 | Total NR aggregated Bandwidth equal to 1250 MHz |
| TOT NR AGG BW 1300 | Total NR aggregated Bandwidth equal to 1300 MHz |
| TOT NR AGG BW 1350 | Total NR aggregated Bandwidth equal to 1350 MHz |
| TOT NR AGG BW 1400 | Total NR aggregated Bandwidth equal to 1400 MHz |
| TOT NR AGG BW 1450 | Total NR aggregated Bandwidth equal to 1450 MHz |
| TOT NR AGG BW 1500 | Total NR aggregated Bandwidth equal to 1500 MHz |
| TOT NR AGG BW 1550 | Total NR aggregated Bandwidth equal to 1550 MHz |
| TOT NR AGG BW 1600 | Total NR aggregated Bandwidth equal to 1600 MHz |
| PCELL NR BW 50 | PCC NR Bandwidth equal to 50 MHz |
| PCELL NR BW 100 | PCC NR Bandwidth equal to 100 MHz |
| PCELL NR BW 200 | PCC NR Bandwidth equal to 200 MHz |
| PCELL NR BW 400 | PCC NR Bandwidth equal to 400 MHz |

- Table 4.9.32.3-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 1 1 1 1 |  |  |