3GPP TSG RAN Meeting #97-e							RP-222106

Electronic Meeting, September 12 – 16, 2022

# Source:	Ericsson

# Title:		New WID on high power for FR1 for DC_R18_xBLTE_yBNR_zDLnUL with power class PC2 and PC1.5 (x= 1, 2, 3, 4; y=1, 2)

# Document for:	Approval

Agenda Item:	9.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	High power for FR1 for DC_R18_xBLTE_yBNR_zDLnUL with power class PC2 and PC1.5 (x= 1, 2, 3, 4; y=1, 2)

## Acronym: DC_R18_xBLTE_yBNR_zDLnUL

## Unique identifier: 	xxxxx

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| DC_R18_xBLTE_yBNR_zDLnUL | RAN4 | xxxxxx | High power for FR1 for DC_R18_xBLTE_yBNR_zDLnUL with power class PC2 and PC1.5 (x= 1, 2, 3, 4; y=1, 2) |

## 3	Justification

This Work Item will focus on power class 1.5 (PC1.5) and power class 2 (PC2) EN-DC band combinations with 26dBm maximum output power, in which configurations for x LTE bands and y NR (xLTE+yNR) band DL with 1 LTE UL and 1TDD NR (1LTE+1TDD NR) band UL will be defined under this WI, where

- The downlink x is 2, 3 or 4 and y is 1, or,

- The downlink x is 1, or 2 and y is 2

- The uplink is 1 LTE and 1 TDD NR

The preconditions shall ensure that the constituent of PC3 EN-DC configurations shall be completed and specified before to propose PC1.5 and PC2 EN-DC configurations.

Example 1: If DC_1A-2A-n3A.n78A is proposed,

- EN-DC Band 1, 2, n3 and n78 requirements shall be completed and specified in advanced.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- PC1.5 and PC2 EN-DC band combinations introduced by this WI will be introduced starting with REL-18.

- Specify the band-combination specific RF requirements for all listed NR EN-DC combinations for

  - 1 (1LTE+1NR) different bands DL with 2 (1LTE+1TDD NR) bands UL, or

  - 3 (2LTE+1NR) different bands DL with 2 (1LTE+1TDD NR) bands UL, or

  - 4 (3LTE+1NR) different bands DL with 2 (1LTE+1TDD NR) bands UL, or

  - 3 (1LTE+2NR) different bands DL with 2 (1LTE+1TDD NR) bands UL, or

  - 4 (2LTE+2NR) different bands DL with 2 (1LTE+1TDD NR) bands UL.

- including at least

  - Applicable frequencies

  - Applicable bandwidths and bandwidth sets

- Analyze combinations that have self-desensitization due to following reasons:

  - TX Harmonic overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB and ∆RIB

  - Reference sensitivity excerptions

  - UL RB restrictions for REFSENS test

- Add conformance testing in RAN5 specifications (to follow at a later stage) of all Rel-18 EN-DC combinations that fall into the category defined by the WI title.

Note：the uplink band combination includes at least one TDD band. And, the uplink FDD+TDD band combinations could support LTE 23dBm + NR 23dBm and LTE 23dBm + NR 26dBm, and the TDD+TDD band combinations support only LTE 23dBm + NR 23dBm.

An overview table of these DC configurations is provided in the appended Excel sheet.

### 4.2	Objective of Performance part WI

Specify the necessary performance requirements such as release independence in TS 38.307.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | TBD | High power for FR1 for DC_R18_xBLTE_yBNR_zDLnUL with power class PC2 and PC1.5 (x= 1, 2, 3, 4; y=1, 2) |  | TSG#102 | Core part |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-3 | Add PC1.5 and PC2 EN-DC to User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios | RAN#102 | Core part |
| 38.307 | Add PC1.5 and PC2 EN-DC Requirements on User Equipment (UEs) supporting a release-independent frequency band | RAN#102 | Perf. part |

## 6	Work item Rapporteur(s)

Per Lindell

Company:	Ericsson

Email:	per.lindell@ericsson.com

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

AT&T

Verizon

Nokia

Qualcomm

Samsung

LGE

Mediatek