3GPP TSG RAN Meeting #97-e			RP-222227

Electronic Meeting, September 12th – 16th, 2022

Agenda item:	9.3.1.5

Source:	KT Corp.

Title:	Discussion on Release-18 NR Sidelink evolution

Document for:	Discussion

# Introduction

Normative work on sidelink evolution is on-going focusing on sidelink operations in Unlicenced band for both mode 1 and mode 2 operation. In this meeting, RAN plenary needs to decide whether to work on following 2 objectives [1]:

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#97) - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication) - The work is limited to FR1 licensed spectrum and ITS band in FR1. - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (This part of the work is put on hold until further checking in RAN#97) - Update evaluation methodology for commercial deployment scenario - Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution, above two objectives are discussed.

# Market Needs for Sidelink Enhancement

While operators are yet to find a dominant business model and/or service model for FR2, enhanced DL/UL throughput performance in FR2 remains valuable for 5G-Advanced. In addition to this, competence in low latency can also be plus for operators utilizing FR2 spectrum. In fact, many services which is currently supported by fibre based fixed broadband can be replaced with devices supporting FR2. For example, VR theme parks, which are being supported using fixed broadband, can be beneficial by having fast wireless devices. Even if standardization work on XR enhancement is currently on-going, operators clearly see market demands for higher throughput and low latency devices which can be fitted into limited form factors with less thermal dissipations.

Further more FR2 sidelink communication between devices can help reducing the network load where heavy data transfer is needed between two devices. By supporting this feature, it will help operators to reduce network load, which will also result reducing energy consumptions from gNBs.

Observation: There are market needs for sidelink with higher data rate support

In order to achieve above needs it is good time for RAN to make decision on starting normative work for NR sidelink CA support and sidelink operation on FR2 to gain the full benefits of 5G-Advanced. As RAN plenary had been agreed to discuss whether to start working on these two objectives, KT proposes to start normative work on NR sidelink CA and sidelink operation on FR2.

Proposal: Start normative work on NR sidelink CA and sidelink operation on FR2 from RAN1#111-e

# Conclusion

Based on the discussion in the above section, the following proposal is suggested.

Proposal: Start normative work on NR sidelink CA and sidelink operation on FR2 from RAN1#111-e

# Reference

- RP-221798, WID Revision: NR Sidelink Evolution, RAN#96