3GPP TSG RAN Meeting #97-e						RP-222392

Electronic Meeting, September 12 - 16, 2022

Agenda item:	9.3.2.1

Source: 	CATT

Title: 	Discussion on Objective #3 in the R18 Mobility Enhancement WID

Document for:	Discussion and decision

1 Introduction

In [2], there is one open issue on the objective #3, i.e.

3.	To specify CHO including target MCG and target SCG [RAN3, RAN2].

Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e.

This results from some discussions in the last RAN meeting, and there were some discussions after that in the RAN2 and RAN3 WGs.

This contribution reviews the latest situation and suggests to adjust this objective so that it has a more focused work scope. The discussions are in section 2 and the proposal is summarized in section 3.

2 Discussions

RAN2 progresses

Details could be found in [2].

First of all, it is concluded in RAN2 that the objective #3 is supported by RAN2 specification.

Observation: Current RAN2 Stage-3 specifications can support CHO including target MCG and target SCG in Rel-17.

Then there was suggestion to capture further information to the stage 2 specification (TS 37.340) regarding the overall procedure of CHO with SCG (e.g., CHO including target MCG and target SCG). This is of course a useful attempt, but in RAN2 discussions there are views that this does not impact the functionality that has already been supported by stage-3, so perhaps not critical. The latest status is that a post meeting email discussion is planned to discuss how to capture missing signalling procedure for CHO with SCG in TS 37.340. So it is expected such stage 2 addition should be able to conclude already as part of Rel-17 in the next RAN2 meeting.

So it is clear from the above that from RAN2 point of view the current objective #3 can be completed as part of Rel-17.

RAN3 progresses

Details could be found in [3].

Firstly, similar as in RAN2, RAN3 already completes most of the work for CHO including target MCG and target SCG in Rel-17. The following conclusions were reached in this RAN3 meeting.

(1) There is a need for SN (or S-SN) to inform MN of the execution of Rel-16 intra-SN CPC or legacy SCG reconfiguration in order for MN to avoid configuration mismatch with UE (i.e. either release conditional reconfigurations in the NW side that are auto-released by the UE or update conditional reconfigurations in the NW side) due to the change of current SCG configuration.

For (1), whether to inform MN about the execution of Rel-16 intra-SN CPC depends on RAN2 conclusion.

For (1), SN-initiated SN modification procedure is used and enhanced with a dedicated indicator, where SN MOD REQD message carries the updated SCG configuration to the MN.

RAN3 works out detailing stage-2/3 in the CRs once RAN2 progresses, on the basis of the CRs proposed in R3-224268, R3-224248, and R3-224320.

The in-meeting discussion and post meeting email discussion solve the remaining signalling as mentioned above. There might be some further discussions in the next RAN3 meeting on potential remaining details. It is expected that no more RAN2 involvement is needed. But again that should be done as part of Rel-17.

So we come to similar observation as RAN2 that from RAN3 point of view the current objective #3 can complete as part of Rel-17.

What is next?

Then the next question is can we just remove the objective #3 from the WID [1]? The answer seems to be NO. The main reason is that RAN3 conclude that there is a need to enhance the data forwarding procedure in Rel-18. The following is the related conclusion that we reproduced from [3].

In Rel.18, RAN3 will continue the work on the CHO with SCG at the target. The scope will be limited to the data forwarding optimizations.

The main issue seems to be that when MN prepares the CHO with SCG, it is not aware whether the target SCG ‘behind’ the multiple candidate target MNs may be exactly the same SCG. If they are the same the data forwarding to multiple target MNs and then further to the same target SN is perhaps not optimal way.

We believe the main work regarding this part is in RAN3.

Based on all above, it seems reasonable to update the objective in the following

3.	To specify data forwarding enhancements, if needed, for CHO including target MCG and target SCG [RAN3].

Essentially, we propose to focus this objective to the potential data forward enhancements that RAN3 suggests and therefore limit the work in RAN3 only (i.e., no RAN2 work required.)

3 Conclusions

In this contribution, we discussed the objective #3 in the Rel-18 mobility enhancement WID. The following is proposed:

Proposal 1 	Objective #3 in RP-221799 is updated to

3.	To specify data forwarding enhancements, if needed, for CHO including target MCG and target SCG [RAN3].

Reference

[1] RP-221799, Revised WID on Further NR mobility enhancements

[2] Chair Notes of RAN2#119-e, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_119-e/Inbox/Chairs_Notes/R2_119-e%20Chair%20Notes%20EOM.docx

[3] Chair Notes of RAN3#117-e, https://www.3gpp.org/ftp/tsg_ran/WG3_Iu/TSGR3_117-e/Inbox/Drafts/Chairs_Notes/RAN3_117-e_agenda_20220825_EOM1.zip