3GPP TSG RAN WG1 Meeting #110	R1-2208190

# Toulouse, France, August 22 – Augest 26, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0321 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on aperiodic CSI-RS for tracking for fast SCell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), Qualcomm, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Subclause 5.2.1.5.3 of TS38.214, the first slot of the first CSI-RS burst is m1th SCell slot and the second slot of the second CSI-RS burst (if any) is m2th SCell slot where m1 and m2 are provided by the MAC-CE and higher layer configuration. However, it is not clear which higher layer configurations are referred to. In RAN2#118-e meeting, a new RRC parameter aperiodicTriggeringOffsetL2-r17 in NZP-CSI-RS-ResourceSet was agreed as an indication of triggering offset for efficient activation of the SCell. It should be clear that m1 is provided by aperiodicTriggeringOffsetL2-r17 in NZP-CSI-RS-ResourceSet and m2 is provided by gapBetweenBursts that are associated with the CSI-RS burst(s) triggered by the MAC-CE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that m1 and m2 are provided by parameters aperiodicTriggeringOffsetL2 in NZP-CSI-RS-ResourceSet and gapBetweenBursts, respectively, which are associated with the CSI-RS burst(s) triggered by the MAC-CE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are potential misunderstanding between gNBs and UEs on the slots that the first CSI-RS burst and/or the second CSI-RS burst start. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on the slot indication of aperiodic CSI-RS for tracking for fast SCell activation. Since the CR is in line with RAN1 agreements, it is not expected to impact on gNB/UE implementation by the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.1.5.3	Aperiodic CSI-RS for tracking for fast SCell activation

When the UE receives an Enhanced Scell Activation/Deactivation MAC-CE that triggers one or two CSI-RS bursts for fast SCell activation for a (set of) deactivated SCell(s),

-	if the MAC-CE indicates that the first CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the first CSI-RS burst for SCell activation is present in that SCell. The first slot of the first CSI-RS burst starts at the m1th SCell slot after the last SCell slot coinciding with the reference slot n+k, as defined in clause 4.3 of [6, TS38.213].

-	if the MAC-CE indicates that the second CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the second CSI-RS burst for SCell activation is present in that SCell. The first slot of the second CSI-RS burst starts at the m2th SCell slot after the end of the first CSI-RS burst. The CSI-RS of the second burst shall have the same antenna port index, OFDM symbol allocations in a slot, same PRB allocation location as the CSI-RS of the first burst.

-	Where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots in clause 5.1.6.1.1.1, and m1 and m2 are provided by aperiodicTriggeringOffsetL2 in NZP-CSI-RS-ResourceSet and gapBetweenBursts, respectively, associated with the CSI-RS burst(s) triggered by the MAC-CE.

<Unchanged parts are omitted>