3GPP TSG RAN meeting #97-e									RP-222423

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | TBA |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220977 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:5% RAN4: 5% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#104e meeting

The following tdocs have been agreed in RAN4#104e meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2214345 | WF on R18 eFeRRM | Apple |
| R4-2211851 | Work plan for R18 Even Further RRM enhancement for NR and MR-DC WI | Apple, OPPO |

Topic #2: RRM core requirements for FR2 SCell activation delay reduction (11.9.2)

Sub-topic 2-1 General

Issue 2-1-1: Baseline scope for FR2 SCell activation enhancement

Agreement:

- 1st phase: RAN4 starts the discussion of the R18 unknown FR2 SCell activation enhancement based on baseline FR2 SCell activation requirement in TS38.133 section 8.3.2

  - Discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS

- 2nd phase: the other SCell activation cases, e.g., multiple SCell activation, direct SCell activation, and PUCCH SCell activation, to be discussed after baseline case (i.e. single FR2 SCell activation) is completed.

  - 2nd phase will start from RAN4 #107 meeting.

Issue 2-1-2: Prioritization for initial phase

Agreement:

- RAN4 to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI.

  - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

  - The extension of the enhancement solutions to FR1 can also be discussed.

Sub-topic 2-2 L3 part enhancement for FR2 SCell activation (AGC, cell synchronization, cell L3 measurement, cell T/F tracking and etc.)

Beam related enhancement for L3 part

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- FFS on beam sweeping factor reduction during unknown FR2 SCell activation.

  - Option 1a (CMCC): The agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or positioning enhancement can be considered as baseline.

  - Option 1b (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2

  - Option 1c (Ericsson):

    - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms].

    - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping.

    - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before

  - Other option is not precluded

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

- FFS whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction.

AGC/Cell measurement/synchronization sample number related enhancement for L3 part

Issue 2-2-3: Cell measurement/synchronization sample number related enhancement of FR2 unknown SCell activation

- FFS to align the understanding of cell measurement/synchronization/AGC in the existing FR2 unknown SCell activation delay requirement.

- FFS on all options for issue 2-2-3

  - Option 1 (Apple, OPPO, CMCC, LGE, MTK, NTT DCM): RAN4 to study sample number reduction for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition)

    - Option 1a(Apple, CMCC): RAN4 to study if we can remove or reduce L3 T/F tracking time (8 Trs)

  - Option 2 (ZTE): Based on the assumption of scaling factor 8, whether the processing latency of AGC can be reduced, should be further discussed.

RS related enhancement for L3 part

Issue 2-2-4: RS related enhancement of FR2 unknown SCell activation

- FFS: AP-CSI-RS and/or A-TRS based fast SCell activation may also apply to unknown FR2 SCell activation enhancement

Sub-topic 2-3 L1 part enhancement for FR2 SCell activation (L1-RSRP, TCI determination, CQI RS configuration/activation, CQI reporting and etc.)

Enhancement for L1-RSRP

Issue 2-3-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- FFS on beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation.

  - Option 1a (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 for FR2

  - Option 1b (Ericsson):

    - RAN4 to discuss the solutions or pre-conditions to reduce the scaling factor N1 in FR2 SCell activation.

    - RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X=1280ms].

    - If UE performed a full RX beam sweeping for a procedure, next procedures or steps fall within RX beam constant time do not need to perform RX beam sweeping.

    - UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before

  - Option 2 (LG, MTK, HW, ZTE): Consider reporting (rough) best Tx beam during cell detection to reduce L1-RSRP measurement delay

Issue 2-3-2: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation related with WI of FR2 multi-Rx chain DL reception

Agreement:

- Conclusion of issue 2-2-2 is applied for this issue 2-3-2

Issue 2-3-3: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

- FFS: the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown SCell activation requirement.

Issue 2-3-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

FFS:

- Proposal 1 (Huawei, vivo, Apple): study whether L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and whether it should be performed in non-DRX mode even DRX is configured.

  - Proposal 1a (Huawei): agree that L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and it should be perform in non-DRX mode even DRX is configured.

Issue 2-3-5: Other enhancement for L1-RSRP measurement of FR2 unknown SCell activation

FFS:

- Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell

- Proposal 2 (Apple, OPPO, ZTE, Xiaomi, OPPO, Ericsson, Nokia, NTT DCM): RAN4 to study skipping L1-RSRP measurement

  - Proposal 2a (Apple, OPPO, LGE, NTT DCM, ZTE): use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs

  - Proposal 2b (ZTE, NTT DCM, ZTE): For the case of IBM disabled, maybe the UE would receive DL transmissions from different serving cells with the same Rx beam assumption. In such case, not any L1-RSRP measurement and report is necessary for this to-be-activated SCells if one active serving cell existing.

  - Proposal 2c (Huawei, Apple, LGE, OPPO, NTT DCM, ZTE): UE can report the beam information based on cell search and skip the L1-RSRP procedure.

TCI related enhancement for L1 part

Issue 2-3-6: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation

- FFS: whether and how to skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused.

Issue 2-3-7: TCI activation enhancement during FR2 unknown SCell activation

FFS:

- Proposal 1 (Intel, Apple, Xiaomi,): further discuss the UE beam assumption after L1-RSRP report to reduce MAC CE based TCI activation delay.

  - Proposal 1a(Huawei, Apple, Intel, Xiaomi, OPPO, MTK, Ericsson, NTT DCM): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

- Proposal 2 (ZTE): to consider whether the TCI state indication can be acquired by UE before SCell activation or not, which would impact the total latency.

- Proposal 3 (ZTE): to consider whether and how to identify TCI state without L1-RSRP measurement and report, which is the core issue for latency reduction for unknown case.

Aperiodic RS related enhancement for L1 part

Issue 2-3-8: Aperiodic RS related enhancement during FR2 unknown SCell activation

FFS:

- Proposal 1 (Apple, Ericsson): RAN4 to discuss: use AP RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement

- Proposal 2a(Apple, Xiaomi, MTK, Ericsson, ZTE): RAN4 to discuss: use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command

- Proposal 2b(Xiaomi): If there is no any QCL information of reference signal for target SCell, A-TRS is configured for fine timing tracking, and the A-TRS is QCL-ed with the selected SSB index.

- Proposal 3: Any other solutions to enable A-TRS usage to unknown SCell activation is not precluded.

SP CSI-RS and periodic CSI-RS related enhancement for L1 part

Issue 2-3-9: SP CSI-RS and periodic CSI-RS related enhancement during FR2 unknown SCell activation

FFS:

- Proposal 1 (Intel, MTK): Futher discuss how to reduce the semi-persistent CSI-RS activation or RRC based CSI-RS configuration delay.

Sub-topic 2-4 Other potential enhancement for FR2 SCell activation

Issue 2-4-1: Other possible enhancement of FR2 unknown SCell activation

FFS:

- Proposal 1 (LG, Nokia, ZTE): Consider long term measurement or aperiodic measurement to detect target SCell before SCell activation command

- Proposal 2 (Nokia, LGE, Ericsson):

  - Proposal 2a: The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be revisited to reduce the FR2 SCell activation delay.

  - Proposal 2b: Some alignment on the up-to-date known/unknown status of the to-be-activated SCell shall be considered to determine the FR2 SCell activation delay.

  - Proposal 2c (this is not only for L3 part but for the whole activation procedure): The potential to enable earlier data transmission within the activation period needs to be studied

- Proposal 3 (MediaTek):

  - Contention based random access (CBRA) can be used to activate first unknown SCell in one band.

- Proposal 4 (Huawei, Apple, CTC, ZTE):

  - Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced (e.g., Tactivation_time=3ms).

- Proposal 5 (ZTE, CTC):

  - To realize energy saving, similar as scellWithoutSSB defined for intra-band CA in Rel-16, the SCell without SSB or SSB-less can be considered for inter-band CA case. The corresponding SCell activation procedure needs some update for such SCell withou SSB or SSB-less.

Topic #3: RRM core requirements for FR1-FR1 NR-DC (11.9.3)

Sub-topic 3-1 General

Issue 3-1-1: How to reflect the applicability of the existing requirement for FR1+FR1 NR-DC

Agreement:

- Rel-18 RRM requirements need to remove the clarification or limitation of only supporting FR1-FR2 NR-DC

- FFS

  - Proposal 1: Apart from the requirements in R16/R17 mentioned in the WID, RAN4 to discuss the applicability rules of other R16/R17 requirements for FR1+FR1 NR-DC with/without changing on existing requirements.

  - Proposal 2:

    - RAN4 to firstly agree on the list of features that shall be included in the table1.

    - RAN4 to agree on the baseline requirement clause and formulation based on the agreed list of features that included in the table1.

    - RAN4 shall have separate applicability discussion of the Scheduling availability, CSSF impacted based on the agreed list of features that included in the table1.

Issue 3-1-2: Other features with FR1+FR1 NR-DC

Agreement:

- The impact of features such as NR-U, MG-enhancement and Redcap are not expected unless the WI scope was extended.

Sub-topic 3-2 RRM requirement for FR1+FR1 NR-DC

Number of serving carriers

Issue 3-2-1: Number of serving carriers

Agreement:

- RRM requirements for supported number of serving carriers for NR-DC need be updated  for FR1-FR1 NR-DC according to RF specification.

  - FFS on exact number of serving carriers for NR-DC.

    - Option 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell.

    - Option 2: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

PSCell addition/release delay and conditional PSCell addition delay

Issue 3-2-2: PSCell addition delay requirement for FR1+FR1 NR-DC

Agreement:

- For NR PSCell in FR1: Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms.

- For NR PSCell in FR1: Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3* Trs ms.

Issue 3-2-3: PSCell release delay requirement for FR1+FR1 NR-DC

Agreement:

- For FR1+FR1 NR-DC case, existing PSCell release delay requirement can be reused (section 8.9.3 in TS38.133).

  - Whether or not the interruption requirement stated in section 8.9.3 of TS38.133 can be reused for FR1+FR1 NR-DC case is up to issue 3-2-4.

Issue 3-2-4: Interruption requirement for PSCell addition/release for FR1+FR1 NR-DC

- FFS

  - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC): For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused.

  - Option 2(Nokia, Ericsson, vivo): NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC.

    - Option 2a(vivo): When defining the interruption requirements, for the number of SCells for FR1-FR1 NR-DC, up to 3 SCell(s) in FR1 of PCell and up to 3 SCell(s) in FR1 of PSCell are configured, deconfigured, activated or deactivated.

Issue 3-2-5: conditional PSCell addition delay requirement for FR1+FR1 NR-DC

- FFS

  - Option 1(Apple, Xiaomi, vivo, Intel, OPPO, MTK, QC, Nokia, HW, vivo): For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133)

  - Option 2 (Ericsson): existing Conditional Pscell addition requirement (section 8.9A.2 TS38.133) can be used as a baseline for starting the discussion

    - Option 2a: study the potential enhancement for TUE_preparation

PSCell change delay and conditional PSCell change delay

Issue 3-2-6: PSCell change delay requirement for FR1+FR1 NR-DC

Agreement:

- For FR1+FR1 NR-DC case, existing PSCell change requirement can be reused. (Section 8.11 TS38.133)

Issue 3-2-7: conditional PSCell change delay requirement for FR1+FR1 NR-DC

Agreement:

- For FR1+FR1 NR-DC case, existing conditional PSCell change requirement can be reused. (Section 8.11B TS38.133)

Scheduling availability

Issue 3-2-8: Scheduling availability requirement for RLM/BFD/CBD/L1-RSRP in FR1+FR1 NR-DC

Agreement:

- Scheduling restrictions for UE performing RLM/BFD/CBD/L1-RSRP in case of FR1-FR1 NR-DC needs to be clarified/updated in the specification.

- FFS on option 1a and 2 and 3.

  - Option 1a(Apple, OPPO, Intel, Xiaomi, QC, vivo): For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

  - Option 2(MediaTek, QC): Add inter-band NR-DC scenario in scheduling availability for both FR1+FR1 and FR1+FR2.

  - Option 3(Nokia): Other solutions are not excluded.

Issue 3-2-9: Scheduling availability requirement for L3 measurement in FR1+FR1 NR-DC

FFS:

- Option 1(Apple, OPPO, Intel, Xiaomi, MTK, HW):

  - For intra-frequency measurement without MG,

    - the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

  - For inter-frequency measurement without MG,

    - No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Measurement restriction

Issue 3-2-10: Measurement restriction in FR1+FR1 NR-DC

FFS:

- Option 1(MediaTek): Add inter-band NR-DC scenario in measurement restriction for both FR1+FR1 and FR1+FR2.

CSSF

Issue 3-2-11: CSSF outside gap for FR1+FR1 NR-DC

Agreement:

- For CSSF, add the scenario of FR1 PCell + FR1 PSCell in CSSFoutside_gap,i.

- FFS on option 1a and 2.

  - Option 1a(Apple, Xiaomi, OPPO, vivo, MTK, Ericsson, HW): CSSF outside gap for FR1+FR1 NR-DC is:

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 PCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

  - Option 2(Intel, QC): For FR1+FR1 NR-DC, further discuss whether similar rule to derive CSSFoutside_gap and CSSFwithin_gap based on FR1+FR2 NR DC can be re-used.

Issue 3-2-12: CSSF inside gap for FR1+FR1 NR-DC

Agreements:

- For FR1+FR1 NR-DC case, no change is needed on existing CSSF within MG requirement.

HO with PSCell

Issue 3-2-13: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC)

FFS:

- Option 1(Apple, Xiaomi, OPPO, vivo, Intel, MTK, QC, HW, vivo): the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC.

- Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

Issue 3-2-14: HO with PSCell for FR1+FR1 NR-DC (FR1-FR2 NR-DC to FR1-FR1 NR-DC)

FFS:

- Option 1(Apple, OPPO, Intel, Xiaomi, QC), HW: the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused

- Option 2 (vivo, MTK, QC): Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

- Option 3(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

Issue 3-2-15: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC)

FFS:

- Option 1(Apple, OPPO, vivo, Intel, Xiaomi, MTK, QC, HW):

  - Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

- Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

SCG activation/deactivation

Issue 3-2-16: RACH based PSCell activation for FR1+FR1 NR-DC

Agreements:

- SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC

- FFS on option 1a and 1b

  - Option 1a(Apple, Xiaomi, OPPO, MediaTek, vivo, Intel, Ericsson, HW): if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

  - Option 1b(Ericsson): further study the potential enhancement in T∆

Issue 3-2-17: Known condition for PSCell activation for FR1+FR1 NR-DC

FFS:

- Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC, HW, vivo): In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

MTTD/MRTD

Agreement:

- For MRTD/MTTD requirements of FR1-FR1 NR-DC, the existing requirements can be reused.

#### 2.4.2	Remaining Open issues

As summarized in section 2.4.1 (open options in the agreement).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2211769 | Discussions on FR2 SCell activation | NTT DOCOMO, INC. |
| R4-2211851 | Work plan for R18 eFeRRM | Apple |
| R4-2211852 | Preliminary discussion on FR2 SCell activation enhancement | Apple |
| R4-2211853 | Preliminary discussion on RRM for FR1-FR1 NR-DC | Apple |
| R4-2211941 | Discussion on FR2 Scell activation delay reduction | CMCC |
| R4-2211964 | Discussion on FR2 SCell activation delay reduction | Xiaomi |
| R4-2211967 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2212063 | Initial discussion on FR2 Scell activation delay reduction | OPPO |
| R4-2212064 | Initial discussion on RRM requiremetns for FR1-FR1 NR-DC | OPPO |
| R4-2212125 | Discussion on FR2 SCell activation delay reduction | Intel Corporation |
| R4-2212126 | Discussion on FR1-FR1 NR-DC | Intel Corporation |
| R4-2212217 | Discussion on FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2212277 | Discussion on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2212513 | Discussion on FR2 SCell activation delay reduction | MediaTek Inc. |
| R4-2212514 | Discussion on FR1-FR1 NR-DC | MediaTek Inc. |
| R4-2212861 | discussion on FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2212972 | Discussion on FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2212973 | Discussion RRM requirements for FR1-FR1 NR-DC | Huawei, HiSilicon |
| R4-2213039 | Discussion on FR2 Scell activation delay reduction requirements | vivo |
| R4-2213040 | Discussion on FR1-FR1 NR-DC requirements | vivo |
| R4-2213286 | Discussion on RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2213869 | Discussion on RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2213902 | Discussion on NRDC FR1 FR1 RRM requirements | Ericsson |
| R4-2213958 | On FR2 SCell activation delay reduction | Ericsson |
| R4-2214154 | Email Discussion Summary for [104-e][234] NR_RRM_enh3 | Moderator (Apple) |
| R4-2214283 | Email Discussion Summary for [104-e][234] NR_RRM_enh3 | Moderator (Apple) |
| R4-2214345 | WF on R18 eFeRRM | Apple |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template