3GPP TSG RAN meeting #97-e									RP-222424

Electronic Meeting, September 12-16, 2022

## Status Report to TSG

Agenda item:			9.5.4.7

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh2 |  |  |  |  |
| Unique ID | 890157 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213067 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 03/2022 | Performance part: 09/2022 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 100% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple Corporation |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Qiuge Guo |
|  | Company | CATT |
|  | Email | guoqiuge@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #104-e meeting

(1) SRS antenna port switching

Following tdocs were agreed and endorsed.

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2214328 | WF on R17 FeRRM - SRS antenna port switching | Apple |
| R4-2214682 | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | Intel Corporation |
| R4-2214516 | 38133 CR on interruptions at SRS antenna port switching in MR-DC | Nokia, Nokia Shanghai Bell |
| R4-2214690 | draftCR on TC1 on SRS antenna switching in EN-DC | Nokia, Nokia Shanghai Bell |
| R4-2214696 | Draft CR on test case for NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | vivo |
| R4-2212948 | CR on SRS antenna port switching requirements 36.133 | Huawei, HiSilicon |
| R4-2214708 | Draft CR on TC for NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | Huawei, HiSilicon |

Sub-topic 3-1: Test configuration principles

Issue 3-1-1: General configuration – SCS and BW

Agreements in GTW (Aug 22)

- for configuration of test cases for SRS antenna port switching, to consider following cases:

  - 15 kHz SSB SCS with 10 MHz bandwidth for FDD

  - 15 kHz SSB SCS with 10 MHz bandwidth for TDD

  - 30 kHz SSB SCS with 40 MHz bandwidth for TDD

- Above configurations apply for both aggressor cell and victim cell, and such configuration may be different between aggressor cell and victim cell. The configuration of victim and aggressor cells can be chosen independently in the test.

- Add a note in the test requirements that the UE is required to test only one of the supported combinations.

Issue 3-1-2: SRS configuration for scenario 1 sync case (symbol-level interruption)

Agreements in GTW (Aug 22)

- Define test cases for scenario 1 sync case by allocating the SRS resource before the last two symbols or more of slot i.e., at the 10th symbol of a slot and the interruption on victim CCs should be no longer than 1 slot (subframe for E-UTRA) when the SCS of aggressor CC and victim CC is 15/30 KHz.

Sub-topic 3-2: testing configuration details

Issue 3-2-1: Special slot configuration to accommodate 6 SRS symbols

Agreements in GTW (Aug 22)

For both Scenario 1 and Scenario 2 in SRS antenna port switching test cases, new TDD configurations are introduced to have 6UL symbols in the special slot.

| SCS | Special slot configuration |
| --- | --- |
| 15kHz | S = ‘6DL: 2GP: 6UL’ |
| 30kHz | S = ‘4DL: 4GP: 6UL’ |

(2) HO with PSCell

Following tdocs were agreed and endorsed.

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2214329 | WF on FR1+FR2 test cases for HO with PSCell | vivo |
| R4-2211634 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT |
| R4-2214670 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple |
| R4-2214509 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi |
| R4-2214677 | draft CR on TC2 for HO with PSCell from NR SA to EN-DC with parallel processing | OPPO |
| R4-2214683 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation |
| R4-2214697 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo |
| R4-2214698 | Correction on HO with PSCell test cases | Nokia, Nokia Shanghai Bell |
| R4-2214709 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, HiSilicon |
| R4-2214526 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson |
| R4-2214527 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson |
| R4-2214732 | TC for EN-DC to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2214733 | TC for NR SA to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2215108 | Draft CR on TC for HO with PSCell from EN-DC with FR1 PSCell to EN-DC with FR2 PSCell | Nokia, Nokia Shanghai Bell |

Topic #1: HO with PSCell test cases

Agreement:

Specify FR1+FR2 test cases for HO with PSCell as below

•	FR1+FR2 NR-DC to FR1+FR2 NR-DC

•	NR-SA FR1 to EN-DC with FR2 PSCell

•	EN-DC with FR1 PSCell to EN-DC with FR2 PSCell

Introduce the cases with applicability rule based on the testability study and update the applicability when testability study progress.

(2) PUCCH SCell activation

Following tdocs were agreed and endorsed.

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2214330 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2214331 | draftCR on TC 1-7 PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell | Xiaomi |
| R4-2214506 | Completing PUCCH SCell activation requirement | CATT |
| R4-2214663 | TC1-6 for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell | CATT |
| R4-2214671 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | Apple |
| R4-2214672 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | Apple |
| R4-2214688 | draft CR of TC 1-4 and 1-10 (FR2 unknown PUCCH SCell Activation) | Qualcomm |
| R4-2214517 | 38133CR on PUCCH SCell activation delay | Nokia |
| R4-2214691 | draftCR on TC1-5 and TC2-6 PUCCH SCell activation in FR2 inter-band | Nokia |
| R4-2214694 | Draft CR on TC for PUCCH SCell activation and deactivation delay of FR1 unknown cell | MTK |
| R4-2214695 | Draft CR on FR2 TC for PUCCH SCell activation and deactivation delay of known PUCCH SCell and one unknown SCell with PSCell | MTK |
| R4-2214710 | Draft CR on TC for PUCCH SCell activation and deactivation | Huawei |
| R4-2214734 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | Ericsson |
| R4-2214735 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | Ericsson |

Topic #1: PUCCH SCell activation/deactivation core requirements maintenance

Sub-topic 1-1 PUCCH SCell activation delay requirements

Issue 1-1-1: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

Way forward:

- Option 1:

      - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

- Option 2: (Nokia)

      - Pathloss estimation for UL Tx power control is based on RS being configured for L1 measurement using narrow beam.

        - a)       PL-RS is based on the M L1-RSRP measurement samples.

      - How many samples (M) do we need to acquire an accurate L1 measurement of PL-RS?

        - a)        M=1/3 has been used in L1-RSRP measurement. M=1 is used in SCell activation delay for unknown DL SCell.

        - b)      Would M=[3] be sufficient to do L1 measurement in the same as in L1-RSRP?

      - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

Issue 1-1-2: Update TFirst_available_CSI and TCSI_reporting_after in the PUCCH SCell activation delay requirements?

Way forward:

- Option 1: (Ericsson)

      - Existing requirements:

        - -	TFirst_available_CSI: the delay uncertainty in acquiring the first available downlink CSI reference resource.

        - -	TCSI_reporting_after: the delay uncertainty in acquiring the first available CSI reporting resource after T3

      - Update to:

        - TLast_Valid_CSI: the delay uncertainty in acquiring the downlink CSI reference resource which is the last CSI reference resource associated with the first available CSI report resource after T1+T2+T3.

        - TCSI_reporting_after is the delay uncertainty in acquiring the first available CSI reporting resource after T3

- Option 2:

      - Keep the existing requirements.

Issue 1-1-3: The conditions where the cross PUCCH group CSI reporting UE capability is required?

Agreements:

- The conditions where the cross PUCCH group CSI reporting UE capability is required shall be aligned with RAN4#101e agreement (only required when the beam information is needed to be indicated).

Issue 1-1-4: Clarification on previous working assumption on PL-RS?

Agreements:

- Based on RAN2 spec, there is no case that when pathlossReferenceRSs is not provided but PUCCH-SpatialRelationInfo is provided in FR2.

- The above agreement has no impact on existing RAN4 requirements.

- FFS: In FR1, when pathlossReferenceRSs is not provided, the PL-RS is assumed as the SSB for fine timing tracking during SCell activation. Thus, no uncertainty for PL-RS to be considered.

Issue 1-1-5: Clarification on the relation between the associated RS for TCI state, PL-RS and spatial relation indication?

Agreements:

- No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication in core requirements.

Issue 1-1-6: Clarification on the condition for PUCCH SCell activation with multiple Downlink Scells?

Agreements:

- Updated the condition to:  any to-be-activated unknown non-PUCCH Scell in a different band from to-be-activated PUCCH Scell has active serving cell(s) or known to-be-activated non-PUCCH Scell(s) on the same band.

Issue 1-1-7: Clarification on interruption length on UL transmission when colliding with PRACH transmission on PUCCH SCell?

Way forward:

- Option 1: (Huawei)

      - No need to define the interruption length of PRACH transmission when UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH.

- Option 2: (Ericsson)

      - RAN4 to define 4 different interruption groups with each group having different interruption length.

      - For preambles format A1, A2, A3, B1, B2, B3, B4, C0 and C1, the interruption length is 0.5ms.

      - For preambles format 0 and 3, the interruption length is 1ms.

      - For preambles format 1, the interruption length is 3ms.

      - For preambles format 2, the interruption length is 5ms.

Topic #2: PUCCH Scell activation/deactivation performance requirements

Sub-topic 2-1 Test cases and test configuration

Issue 2-1-1: Whether to define test case for intra-band CA

Agreements:

- RAN4 to not define test cases for intra-band CA between dual PUCCH groups, i.e. PUCCH SCell should not be in the same band as the other PUCCH group.

Issue 2-1-2: For EN-DC test, whether PUCCH SCell and  PSCell can be in the same FR

Agreements:

- For EN-DC test, PUCCH SCell and PSCell cannot be in the same FR.

Issue 2-1-3: Whether to define test case for FR1 PCell/PSCell + FR2 PUCCH SCell

Agreements:

- Define test case for FR1 PCell/PSCell + FR2 PUCCH SCell

- TE determines the starting point of the test based on the activation command of which the HARQ ACK is successfully received

- For FR1 PCell/PSCell + FR2 PUCCH SCell, consider known PUCCH SCell with valid TA

- Introducing the unknown PUCCH SCell and invalid TA cases with applicability rule based on the testability study

- Updating the applicability when testability study progress

Issue 2-1-4: Test configuration for PUCCH SCell activation with multiple DL SCells in FR2 (SA TC 1-10: FR2 PUCCH SCell+FR2 SCell+FR2 PCell)

Agreements:

- For SA TC 1-10: FR2 PUCCH SCell+FR2 SCell+FR2 PCell

    - Configures a PCell and two SCells (one PUCCH Scell and one normal SCell) in different FR2 bands

    - The two SCells are intra-band contiguous to each other.

    - The two SCells belong to the same PUCCH group, i.e. the secondary PUCCH group

Issue 2-1-4a: whether to define test for the case when non-PUCCH SCell is in same PUCCH group as PCell, i.e. Primary PUCCH group

Agreements:

- For PUCCH SCell activation with multiple DL SCells in FR2, define test for the case when non-PUCCH SCell is in the same PUCCH group as PCell, i.e. Primary PUCCH group.

    - Configures a PCell and one PUCCH SCell in different FR2 bands

    - Configure a normal SCell on the same band as PCell.

    - The PCell and normal SCells belong to the same PUCCH group, i.e. the Primary PUCCH group

#### 2.4.2	Remaining Open issues

PUCCH SCell activation

Issue 1-1-1: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

Issue 1-1-2: Update TFirst_available_CSI and TCSI_reporting_after in the PUCCH SCell activation delay requirements?

Issue 1-1-7: Clarification on interruption length on UL transmission when colliding with PRACH transmission on PUCCH SCell?

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2211600 | SRS antenna switching requirement | Qualcomm, Inc. |
| R4-2211600 | SRS antenna switching requirement | Qualcomm, Inc. |
| R4-2211632 | Discussion on configuration for test case for SRS antenna port switching | CATT |
| R4-2211632 | Discussion on configuration for test case for SRS antenna port switching | CATT |
| R4-2211841 | On testing configuration for SRS antenna port switching | Apple |
| R4-2211841 | On testing configuration for SRS antenna port switching | Apple |
| R4-2212032 | draft CR on TC6 for NR FR1-E-UTRAN interruptions at NR SRS antenna port switching | OPPO |
| R4-2212032 | draft CR on TC6 for NR FR1-E-UTRAN interruptions at NR SRS antenna port switching | OPPO |
| R4-2212122 | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | Intel Corporation |
| R4-2212122 | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | Intel Corporation |
| R4-2212266 | 38133CR on SRS antenna switching - Resubmission | Nokia, Nokia Shanghai Bell |
| R4-2212266 | 38133CR on SRS antenna switching - Resubmission | Nokia, Nokia Shanghai Bell |
| R4-2212268 | 38133CR on SRS antenna switching in MR-DC | Nokia, Nokia Shanghai Bell |
| R4-2212268 | 38133CR on SRS antenna switching in MR-DC | Nokia, Nokia Shanghai Bell |
| R4-2212269 | Test configuration for SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2212269 | Test configuration for SRS antenna port switching | Nokia, Nokia Shanghai Bell |
| R4-2212270 | draftCR on TC1 on SRS antenna switching in EN-DC | Nokia, Nokia Shanghai Bell |
| R4-2212270 | draftCR on TC1 on SRS antenna switching in EN-DC | Nokia, Nokia Shanghai Bell |
| R4-2212658 | Discussion on test cases for R17 SRS antenna switching interruption requirements | vivo |
| R4-2212658 | Discussion on test cases for R17 SRS antenna switching interruption requirements | vivo |
| R4-2212659 | Draft CR on test case for NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | vivo |
| R4-2212659 | Draft CR on test case for NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | vivo |
| R4-2212948 | CR on SRS antenna port switching requirements 36.133 | Huawei, HiSilicon |
| R4-2212948 | CR on SRS antenna port switching requirements 36.133 | Huawei, HiSilicon |
| R4-2212951 | Discussion on performance requirements for SRS antenna port switching | Huawei, HiSilicon |
| R4-2212951 | Discussion on performance requirements for SRS antenna port switching | Huawei, HiSilicon |
| R4-2212952 | Draft CR on TC for NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | Huawei, HiSilicon |
| R4-2212952 | Draft CR on TC for NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | Huawei, HiSilicon |
| R4-2214128 | Email Discussion Summary for [104-e][208] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2214260 | Email Discussion Summary for [104-e][208] NR_RRM_enh2_1 | Moderator (Apple) |
| R4-2214328 | WF on R17 FeRRM - SRS antenna port switching | Apple |
| R4-2214516 | 38133CR on SRS antenna switching in MR-DC | Nokia, Nokia Shanghai Bell |
| R4-2214682 | DraftCR to TS 38.133: NR FR1 interruptions at NR SRS antenna port switching with more than 1 SRS symbol in NR-CA | Intel Corporation |
| R4-2214690 | draftCR on TC1 on SRS antenna switching in EN-DC | Nokia, Nokia Shanghai Bell |
| R4-2214696 | Draft CR on test case for NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in NR-CA | vivo |
| R4-2214708 | Draft CR on TC for NR SRS antenna port switching with more than 1 SRS symbol in asynchronous EN-DC | Huawei, HiSilicon |
| R4-2211619 | RRM performance requirements for HO with PSCELL | Qualcomm Incorporated |
| R4-2211633 | Discussion on test case for handover with PSCell | CATT |
| R4-2211634 | Test case of handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1 | CATT |
| R4-2211842 | On testing configuration for HO with PSCell | Apple |
| R4-2211843 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple |
| R4-2211956 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi |
| R4-2212033 | draft CR on TC2 for HO with PSCell from NR SA to EN-DC with parallel processing | OPPO |
| R4-2212129 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation |
| R4-2212660 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo |
| R4-2212860 | Correction on HO with PSCell test cases | Nokia, Nokia Shanghai Bell |
| R4-2212953 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, HiSilicon |
| R4-2213747 | Discussion on the testability of FR1+FR2 TCs for HO with PSCell | MediaTek inc. |
| R4-2213952 | TC for EN-DC to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2213953 | TC for NR SA to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2214129 | Email Discussion Summary for [104-e][209] NR_RRM_enh2_2 | Moderator (Vivo) |
| R4-2214261 | Email Discussion Summary for [104-e][209] NR_RRM_enh2_2 | Moderator (Vivo) |
| R4-2214509 | CR on test case for Handover with PSCell from NR SA to EN-DC with sequential processing | Xiaomi |
| R4-2214670 | Draft CR on TC for HO with PSCell from NR-SA to EN-DC with parallel processing and known FR2 PSCell in TS38.133 R17 | Apple |
| R4-2214677 | draft CR on TC2 for HO with PSCell from NR SA to EN-DC with parallel processing | OPPO |
| R4-2214683 | DraftCR to TS 38.133: Handover with PSCell from NR-DC to NR-DC with sequential processing | Intel Corporation |
| R4-2214697 | draft CR on test cases for Handover with PSCell from NE-DC to NE-DC with known target PSCell | vivo |
| R4-2214698 | Correction on HO with PSCell test cases | Nokia, Nokia Shanghai Bell |
| R4-2214709 | Draft CR on TC for HO with PSCell from NR SA to EN-DC | Huawei, HiSilicon |
| R4-2214732 | TC for EN-DC to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2214733 | TC for NR SA to EN-DC Handover with PSCell using CCA with known target PSCell | Ericsson |
| R4-2215108 | Draft CR on TC for HO with PSCell from EN-DC with FR1 PSCell to EN-DC with FR2 PSCell | Nokia, Nokia Shanghai Bell |
| R4-2215109 | Draft CR on TC for HO with PSCell from EN-DC with FR2 PSCell to EN-DC with FR2 PSCell | MediaTek Inc. |
| R4-2213948 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson |
| R4-2213949 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson |
| R4-2214329 | WF on FR1+FR2 test cases for HO with PSCell | vivo |
| R4-2214526 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in EN-DC to EN-DC scenario | Ericsson |
| R4-2214527 | CR on correction of fine timing for HO with PSCell when PSCell is on CCA in NR SA to EN-DC scenario | Ericsson |
| R4-2211630 | Discussion on remaining issues for PUCCH Scell activation | CATT |
| R4-2211631 | Completing PUCCH SCell activation requirement | CATT |
| R4-2211635 | TC1-6 for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell | CATT |
| R4-2211636 | TC2-2 for PUCCH SCell activation and deactivation delay requirements of FR1 unknown cell (All NR cells in FR1) | CATT |
| R4-2211637 | Big CR on test cases of Rel-17 FeRRM - PUCCH SCell activation | CATT |
| R4-2211844 | On PUCCH SCell activation requirement and testing | Apple |
| R4-2211845 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | Apple |
| R4-2211846 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | Apple |
| R4-2212034 | draft CR on TC10 for PUCCH SCell activation and deactivation delay requirements of FR1 known cell for EN-DC | OPPO |
| R4-2212181 | Test framework for FR2 unknown PUCCH SCell Activation | Qualcomm Incorporated |
| R4-2212183 | draft CR of TC 1-4 and 1-10 (FR2 unknown PUCCH SCell Activation) | Qualcomm Incorporated |
| R4-2212267 | 38133CR on PUCCH SCell activation delay requirement - Resubmission | Nokia, Nokia Shanghai Bell |
| R4-2212271 | Open issues on PUCCH SCell activation | Nokia, Nokia Shanghai Bell |
| R4-2212272 | 38133CR on PUCCH SCell activation delay | Nokia, Nokia Shanghai Bell |
| R4-2212273 | draftCR on TC1-5 and TC2-6 PUCCH SCell activation in FR2 inter-band | Nokia, Nokia Shanghai Bell |
| R4-2212517 | Discussion on PUCCH SCell activation and deactivation | MediaTek Inc. |
| R4-2212519 | Draft CR on TC for PUCCH SCell activation and deactivation delay of FR1 unknown cell | MediaTek Inc. |
| R4-2212520 | Draft CR on FR2 TC for PUCCH SCell activation and deactivation delay of known PUCCH SCell and one unknown SCell with PSCell | MediaTek Inc. |
| R4-2212949 | Discussion on maintenance on PUCCH SCell activation requirements | Huawei, HiSilicon |
| R4-2212950 | CR on interruption of PUCCH SCell activation | Huawei, HiSilicon |
| R4-2212954 | Discussion on performance requirements for PUCCH SCell activation | Huawei, HiSilicon |
| R4-2212955 | Draft CR on TC for PUCCH SCell activation and deactivation | Huawei, HiSilicon |
| R4-2213460 | draft CR for TC for PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell | vivo |
| R4-2213950 | Remaining issues in SCell activation/deactivation with PUCCH | Ericsson |
| R4-2213951 | Maintenance CR on SCell activation/deactivation with PUCCH | Ericsson |
| R4-2213954 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | Ericsson |
| R4-2213955 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | Ericsson |
| R4-2213956 | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | Ericsson |
| R4-2214130 | Email Discussion Summary for [104-e][210] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2214262 | Email Discussion Summary for [104-e][210] NR_RRM_enh2_3 | Moderator (CATT) |
| R4-2214330 | WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements | CATT |
| R4-2214331 | draftCR on TC 1-7 PUCCH SCell activation and deactivation delay requirements of FR1 known PUCCH SCell and one FR1 unknown SCell | Xiaomi |
| R4-2214506 | Completing PUCCH SCell activation requirement | CATT |
| R4-2214517 | 38133CR on PUCCH SCell activation delay | Nokia, Nokia Shanghai Bell |
| R4-2214663 | TC1-6 for PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with inter-band FR2 PCell | CATT |
| R4-2214671 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) | Apple |
| R4-2214672 | Draft CR on TC for PUCCH SCell activation and deactivation delay requirements of FR2 unknown PUCCH SCell and one FR2 unknown SCell with FR2 PSCell | Apple |
| R4-2214688 | draft CR of TC 1-4 and 1-10 (FR2 unknown PUCCH SCell Activation) | Qualcomm Incorporated |
| R4-2214691 | draftCR on TC1-5 and TC2-6 PUCCH SCell activation in FR2 inter-band | Nokia, Nokia Shanghai Bell |
| R4-2214694 | Draft CR on TC for PUCCH SCell activation and deactivation delay of FR1 unknown cell | MediaTek Inc. |
| R4-2214695 | Draft CR on FR2 TC for PUCCH SCell activation and deactivation delay of known PUCCH SCell and one unknown SCell with PSCell | MediaTek Inc. |
| R4-2214710 | Draft CR on TC for PUCCH SCell activation and deactivation | Huawei, HiSilicon |
| R4-2214734 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known cell with FR1 PCell | Ericsson |
| R4-2214735 | TC for PUCCH SCell activation and deactivation delay requirements of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell | Ericsson |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template