3GPP TSG RAN meeting #97e										       RP-222477

Electronic Meeting, September 12 – 16, 2022

## Status Report to TSG

Agenda item:			9.5.1.1

| WI / SI Name | Extending current NR operation to 71GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ext_to_71GHz |  |  |  |  |
| Unique ID | 860041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213540 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2022 | Performance part: 12/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 60% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun (RAN1); Aida Vera Lopez (RAN4) |
|  | Company | Qualcomm; Intel Corporation |
|  | Email | jingsun@qti.qualcomm.com; aida.l.vera.lopez@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

### 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

RAN1 completed the work. Remaining details will be completed in maintenance phase.

### 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN2 completed the work. Remaining details will be completed in maintenance phase.

### 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

### 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RF

General aspects and system parameters

UE feature list – Improved ON/ON transient period

- Agreement:

  - Further discuss in Rel-18; either in existing WI or TEI XXX

LS reply to RAN1 on minimum guard period

- LS reply was approved (R4-2214890)

Answer from RAN4: RAN4 has considered the absolute switching time evaluated for FR2-1 in R15, i.e., the antenna switching time is 15 µsec with a view to reusing this value for FR2-2. The detailed evaluation results for the switching scenarios considered are included in the R15 reply LS R4-1710048 [1]. RAN4 will further consider this result and its feasibility for SRS transmission used for antenna switching and whether the switching time assumed could be improved for 480 and 960 kHz operation.

FR2 unlicensed operation

The following note for band n263 will be included in Table 5.2-1: NR operating bands in FR2 of TS 38.101-2

- Agreement:

  - NOTE 1: This band is for unlicensed operation and subject to regional and/or country specific regulatory requirements

Operating bands for CA

- Agreement:

  - Add NR CA band CA_n263 to Table 5-2A.1-1 in TS 38.101-2 together with a note clarifying that only contiguous CA is applicable for this band.

| NR CA Band | NR Band |
| --- | --- |
| CA_n257 | n257 |
| CA_n258 | n258 |
| CA_n259 | n259 |
| CA_n260 | n260 |
| CA_n261 | n261 |
| CA_n263 (Note) | n263 |
| NOTE: In this release of the specification, only contiguous CA is applicable for this operating band. |  |

NR CA bandwidth class

- Agreement:

  - No new bandwidth classes are specified for n*100 MHz

  - The discussion on the new channel bandwidth classes for FR2-2 is allowed in the maintenance of this WI

Channel spacing for CA

- Agreement:

  - Channel spacing for adjacent NR carriers in FR2-2

Nominal Channel spacing = ceil((BWChannel(1) + BWChannel(2))/100.8 MHz) * 50.4 MHz

  - Channel spacing for CA in FR2-2

Nominal Channel spacing = ceil((BWChannel(1) + BWChannel(2))/100.8 MHz) * 50.4 MHz

Configurations for intra-band contiguous CA

- Agreement:

  - This is related to NR CA bandwidth classes and therefore can also be discussed in maintenance stage

FR1+FR2-2 DC/CA band combinations

- Agreement:

  - Specify n48 + n263 within this work item and if n48 + n263 is completed, then the WI is viewed as completed.

    - Band combinations for n48 + n263 were introduced in endorsed draft CR R4-2214431

- Agreement:

  - RAN4 targets completing one example band combination FR2-2 DC/CA with an anchor in FR1 within the maintenance phase of the WI

  - Additional band combinations can be added in a release-independent manner as part of a Release 18 basket WI

  - To specify the example band combination FR2-2 DC/CA with an anchor in FR1, RAN4 could take a corresponding FR2-1 DC/CA combination with anchor in FR1 as a baseline in this meeting, with square brackets around the requirements, and aim to remove the brackets in the maintenance phase

Minimum guard band

- Agreement:

  - Remove square brackets in Table 5.3.3-1

UE Tx agreements

Multi-band relaxation (MBR)

- Agreement for PC3:

  - Remove the brackets on the multi-band relaxation factors (∆MBP,n and ∆MBS,n) of band n263 and confirm both values are 1.0 dB.

MPR

- Agreement - power class 1

Table 6.2.2.1-3 MPRWT for power class 1, BWchannel = 100 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 100 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [5.5] | [0.0] | ≤ [3.5] |
|  | QPSK | ≤ [6.5] | [0.0] | ≤ [3.5] |
|  | 16 QAM | ≤ [7.0] | ≤ [2.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [8.0] | ≤ [8.0] | ≤ [8.0] |
| CP-OFDM | QPSK | ≤ [8.0] | ≤ [1.5] | ≤ [3.5] |
|  | 16 QAM | ≤ [8.0] | ≤ [3.5] | ≤ [4.0] |
|  | 64 QAM | ≤ [9.5] | ≤ [9.5] | ≤ [9.5] |

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [6.0] | ≤ [1.0] | ≤ [3.5] |
|  | QPSK | ≤ [6.0] | ≤ [1.0] | ≤ [4.0] |
|  | 16 QAM | ≤ [4.5] | ≤ [3.0] | ≤ [3.0] |
|  | 64 QAM | ≤ [8.0] | ≤ [8.0] | ≤ [8.0] |
| CP-OFDM | QPSK | ≤ [6.0] | ≤ [1.5] | ≤ [3.5] |
|  | 16 QAM | ≤ [6.0] | ≤ [4.0] | ≤ [5.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

- Agreement - power class 3

Table 6.2.2.3-1a MPRWT for power class 3, BWchannel = 100 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 100 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ [0.5] |
|  | QPSK | 0.0 | ≤ [0.5] |
|  | 16 QAM | ≤ [3.0] | ≤ [3.0] |
|  | 64 QAM | ≤ [8.5] | ≤ [8.5] |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [1.5] |
|  | 16 QAM | ≤ [4.0] | ≤ [4.0] |
|  | 64 QAM | ≤ [10.0] | ≤ [10.0] |

Table 6.2.2.3-2b MPRWT for power class 3, BWchannel = 400 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.0] | ≤ 3.0 |
|  | QPSK | ≤ [1.0] | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ [9.5] | ≤ [9.0] |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 |

Table 6.2.2.3-2c MPRWT for power class 3, BWchannel >= 800 MHz, FR2-2

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.0] | ≤ 4.0 |
|  | QPSK | ≤ [1.0] | ≤ 4.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ [9.5] | ≤ [9.0] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 8.0 |
|  | 64 QAM | ≤ 10.5 | ≤ 10.5 |

A-MPR

- Agreement:

  - No A-MPR requirement needed for the EN 303753 emissions mask.

Minimum output power

- Agreement:

  - PC1 Pmin to be 4 dBm. PC2 and PC3 Pmin to be -13 dBm.

TX OFF power

- Agreement:

  - Reuse FR2-1 requirements for minimum output power and OFF power, specifically remove the [] from this table

EVM PTRS processing

- Agreement:

  - The DMRS based channel estimate shall utilize CPE-corrected DMRS symbols

  - The PTRS extraction and correction stage is used as the final refinement of the received signal.

  - For CP-OFDM, all non-DMRS symbols in a slot must be equipped with PTRS, and frequency density of PTRS tones maximized.

  - For DFT-s-OFDM, PTRS is specified with 4 symbols per group, and the groups are configured in a ‘head and tail’ configuration.

  - For DFT-s-OFDM, the number of PTRS groups is maximized so the ratio of PUSCH symbols to PTRS symbols stays at 1 or higher.

EVM compliance power levels

- Agreement (CCBW = 100 MHz Power class 1 and 2)

  - EVM compliance levels for FR2-2 CBW=100MHz in PC2 and PC1 are the same as FR2-1

- Agreement (CCBW ≥ 400 MHz Power class 1 and 2)

  - The numbers in the table of the proposal are agreeable, but further checking is needed.

- Agreement (CCBW = 100 MHz Power class 3)

  - EVM compliance levels for FR2-2 CBW=100MHz in PC3 is the same as FR2-1

- Agreement (CCBW ≥ 400 MHz Power class 3)

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | [ -16 -13] | [ -13 -11] | [ -10 -8] | [ -7 -5] | [ -6 -4] |
| UE EIRP for UL 16 QAM | dBm | [ -13 -10] | [ -10 -8] | [ -7 -5] | [ -4 -2] | [ -3 -1] |
| UE EIRP for UL 64 QAM | dBm | [ -9 -6] | [ -6 -4] | [ -3 -1] | [ 0 2] | [ 1 3] |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Carrier leakage

- Agreement (PC1 carrier leakage for n263)

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

- Agreement (PC3 carrier leakage for n263)

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9 dBm ≤ EIRP ≤ -1.9 dBm | -20 |

In-band emissions

- Agreement for power class 1

| IQ Image | dB | -25 | Output power > 23.4 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 23.4 dBm |
| Carrier leakage | dBc | -25 | Output power > 13.4 dBm |
|  |  | -20 | 0.4 dBm ≤ Output power ≤ 13.4 dBm |

- Agreement for power class 2

| IQ Image | dB | -25 | Output power > 19 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 19 dBm |
| Carrier leakage | dBc | -25 | Output power > 9 dBm |
|  |  | -20 | -13 dBm ≤ Output power ≤ 9 dBm |

- Agreement for power class 3

| IQ Image | dB | -25 | Output power > 8.1 dBm |
| --- | --- | --- | --- |
|  |  | -20 | Output power ≤ 8.1 dBm |
| Carrier leakage | dBc | -25 | Output power > -1.9 dBm |
|  |  | -20 | -14.9 dBm ≤ Output power ≤ 1.9 dBm |

Improved ON/ON transient period

- Agreement:

  - Further discussion in Rel-18. Either in existing WI or TEI.

Beam correspondence

- Agreement:

  - Specify the requirement of beam correspondence for bit-1. FFS on bit-0 in the next meeting.

Tx CA agreements

Maximum output power for CA

- Agreement:

  - FR2-2 PC1 and PC3 power classes for CA are the same as for FR2-2 single carrier. Note this is the same approach as in FR2-1.

MPR for CA

- Agreement - PC1

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ [7.0] | ≤ [5.0] | ≤ [2.0] | ≤ [2.0] |
| QPSK | ≤ [8.0] | ≤ [6.0] | ≤ [3.0] | ≤ [3.0] |
| 16 QAM | ≤ [8.0] | ≤ [6.0] | ≤ [4.0] | ≤ [4.0] |
| 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

- Agreement – PC3

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] | ≤ [1.0] |
| QPSK | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] | ≤ [2.0] |
| 16 QAM | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] |
| 64 QAM | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] | ≤ [10.0] |

A-MPR for CA

- Agreement:

  - No CA A-MPR needed for the EN 303753 emissions mask.

Carrier leakage in CA

- Agreement for PC1 - n263

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > 13.4 dBm | -25 |
| 0.4 dBm ≤ EIRP ≤ 13.4 dBm | -20 |

- Agreement for PC2 - n263

  - For n263 PC2 use the same value as in FR2-1 since the min peak EIRP values are nearly the same

- Agreement for PC3 - n263

| Parameters | Relative Limit (dBc) |
| --- | --- |
| EIRP > -1.9 dBm | -25 |
| -14.9 dBm ≤ EIRP ≤ -1.9 dBm | -20 |

In-band emissions in CA

- Agreement for PC1 and PC3:

  - Re-use the FR2-1 CA in-band emissions method for PC1 and PC3 CA with the same output power values we are proposing for FR2-2 single carrier.

SEM for CA

- Agreement:

  - Re-use the FR2-1 CA SEM requirements for FR2-2.

ACLR for CA

- Agreement:

  - Use the FR2-2 single carrier 15 dB ACLR value for CA

  - Common understanding in RAN4 is that OBW requirement is more stringent than ACLR for FR2-2

UE Rx agreements

Uplink configuration for REFSENS

- Agreement:

Table 7.3.2.1-2: Uplink configuration for reference sensitivity

| Operating band | NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz | SCS | Duplex mode |
| n257 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n262 | 32 | 64 | 128 | 256 | N/A | N/A | N/A | 120 kHz | TDD |
| n263 | N/A | 64 | N/A | 256 | N/A | N/A | N/A | 120 kHz | TDD |
|  | N/A | N/A | N/A | [64] | 120 | [240] | N/A | 480 kHz |  |
|  | N/A | N/A | N/A | [32] | [60] | [120] | 144 | 960 kHz |  |

Adjacent channel selectivity

- Agreement:

  - Use 21 dB for CCBW ≤ 400 MHz and 20 dB for > 400 MHz

In-band blocking

- Agreement:

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |  |
| PInterferer for band n263 | dBm | N/A | REFSENS + 33.5 dB | N/A | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB |  |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 | ≤ -1600 & ≥ 1600 NOTE 5 | ≤ -3200 & ≥ 3200 | ≤ -4000 & ≥ 4000 |  |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 | FDL_low + 400 to FDL_high - 400 | FDL_low + 800 to FDL_high - 800 | FDL_low + 1600 to FDL_high - 1600 |  |

Spurious emissions

- Agreement:

  - Note 2 and Note 3 have been added to spurious emission requirements table

Table 7.9-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm (NOTE 2) | 1 |
| 1GHz f 2nd harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm (NOTE 3) |  |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: This maximum level does not apply for Band n263 for which -36 dBm applies. NOTE 3: This maximum level does not apply for Band n263 for which -30 dBm applies. |  |  |  |

Rx CA agreements

EIS relaxation for intra-band contiguous CA

- Agreement:

  - Use the same values as in FR2-1. For 1600-2000 MHz FR2-2 EIS relaxation dB value as [1.5].

Maximum input level for intra-band contiguous CA

- Agreement:

  - Use the FR2-1 max input for intra-band contiguous requirement for FR2-2.

Adjacent channel selectivity for intra-band contiguous CA

- Agreement:

Table 7.5A.1-1: Adjacent channel selectivity for intra-band contiguous CA

| Operating band | Units | Adjacent channel selectivity / CA bandwidth class |
| --- | --- | --- |
|  |  | All CA bandwidth class |
| n263 | dB | 21 for BWChannel_CA ≤ 400 MHz 20 for BWChannel_CA > 400 MHz |

Table 7.5A.1-2: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 1

| Rx Parameter | Units | All CA bandwidth Classes |
| --- | --- | --- |
| PInterferer for band n263 | dBm | Aggregated power + 19.5 for BWChannel_CA ≤ 400 MHz Aggregated power + 18.5 for BWChannel_CA > 400 MHz |

Table 7.5A.1-3: Adjacent channel selectivity test parameters for intra-band contiguous CA, Case 2

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, aggregated power for band n263 | dBm | -44.5 for BWChannel_CA ≤ 400 MHz -43.5 for BWChannel_CA > 400 MHz |

In-band blocking - CA

- Agreement:

Table 7.6A.2.1-1: In band blocking minimum requirements for intra-band contiguous CA

| Rx Parameter | Units | All CA bandwidth classes |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB |
| Pinterferer for band n257, n258, n261 | dBm | Aggregated power + 21.5 |
| Pinterferer for band n260, n262 | dBm | Aggregated power + 20.5 |
| Pinterferer for band n263 | dBm | Aggregated power + 19.5 |

BS RF agreements

In this meeting, RAN4 completed the core Tx and Rx requirements. Multiple CRs were endorsed finalizing TAE Tx requirements and ICS Rx requirements. Ongoing discussions regard conformance testing requirements.

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2211806 | Draft CR to TS 38.104 on correction of TAE requirements | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214422 | WF on system parameters for FR2-2 | Intel Corporation | Approved |
| R4-2214430 | Draft CR for TS 38.101-2 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214431 | Draft CR for TS 38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2 | Charter Comm Inc. | Endorsed |
| R4-2214480 | Draft CR for TS 38.104 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2214883 | Draft CR to 38.101-2 on band n263 Tx aspects | Qualcomm Inc., Apple Inc. | Endorsed |
| R4-2214884 | Draft CR to 38.101-2 on band n263 Rx aspects | Qualcomm Inc., Ericsson | Endorsed |
| R4-2214890 | Reply LS for the minimum guard period between two SRS resources for antenna switching | CATT | Approved |
| R4-2215142 | Draft CR for n263 RMC | Huawei, HiSilicon | Endorsed |
| R4-2215255 | Big CR of TS38.101-2 for FR2-2 UE requirements | Intel Corporation | Agreed |
| R4-2215256 | Big CR of TS38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2 | Intel Corporation | Agreed |
| R4-2215257 | Big CR of TS38.104 on system parameter updates for FR2-2 | Nokia, Nokia Shanghai Bell | Agreed |

##### 2.4.1.2	Demod

RAN4 completed this work prior to RAN#96

##### 2.4.1.3	RRM

RAN4 completed this work prior to RAN#96

#### 2.4.2	Remaining Open issues

None

### 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

### 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-e

- R4-2211626, Draft CR to 38.101-2 on band n263 Tx aspects, Qualcomm Inc., Apple Inc.

- R4-2211627, Draft CR to 38.101-2 on band n263 Rx aspects, Qualcomm Inc., Ericsson

- R4-2211628, 60GHz UE TX, Qualcomm Inc.

- R4-2211629, 60GHz UE RX, Qualcomm Inc.

- R4-2211644, Discussion on the BS RX RF requirements for 52.6-71GHz, CATT

- R4-2211697, Reply LS for the minimum guard period between two SRS resources for antenna switching, CATT

- R4-2211806, Draft CR to TS 38.104 on correction of TAE requirements, Nokia, Nokia Shanghai Bell

- R4-2211807, Proposals on finalizing BS receiver requirements for extending current NR operation to 71 GHz, Nokia, Nokia Shanghai Bell

- R4-2211808, Draft CR to TS 38.104 on finalization of ICS interfering signal RB, Nokia, Nokia Shanghai Bell

- R4-2211873, Remaining issues on system parameters for NR operation in 52.6GHz - 71GHz, Apple Inc.

- R4-2211950, Multi-band relaxation for FR2-2 handheld UE, Murata Manufacturing Co Ltd.

- R4-2212118, UE feature list and general aspects for NR ext. to 71GHz, Intel Corporation

- R4-2212119, Open issues for UE Tx requirements in FR2-2, Intel Corporation

- R4-2212278, Draft CR to 38.101-2 on minimum peak EIRP for PC3 band n263, Murata Manufacturing Co Ltd.

- R4-2212372, Discussion on minimum UE EIRP scaling for FR2-2, Apple Inc.

- R4-2212373, CR for TS 38.101-2: Correcting oversight with UE EIRP CBW scaling for FR2-2, Apple Inc.

- R4-2212845, Contiguous DL CA system parameters for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2213203, draftCR on the UE RX requirement for band n263, Xiaomi

- R4-2213220, On UE Tx RF aspects for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2213221, On UE Rx RF aspects for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2213231, UE Rx requirements for band n263, Apple Inc.

- R4-2213232, SSB side conditions for band n263, Apple Inc.

- R4-2213366, Draft CR for n263 RF Tx requirements, Huawei, HiSilicon

- R4-2213367, Draft CR for n263 RF Rx requirements, Huawei, HiSilicon

- R4-2213368, Draft CR for n263 RMC, Huawei, HiSilicon

- R4-2213369, On remaining RF requirements on n263, Huawei, HiSilicon

- R4-2213370, Draft reply LS on the minimum guard period between two SRS resources for antenna switching, Huawei, HiSilicon

- R4-2213466, CR on FR2-2 PC3 MPR, LG Electronics Finland

- R4-2213573, Discussion on MPR values for FR2-2 PC3, LG Electronics Finland

- R4-2213744, UE Tx requirements for band n263, Apple Inc.

- R4-2214088, Email discussion summary for [104-e][110] NR_ext_to_71GHz_Part_1, Moderator (Intel)

- R4-2214089, Email discussion summary for [104-e][111] NR_ext_to_71GHz_Part_2, Moderator (Qualcomm)

- R4-2214168, Email Discussion Summary for [104-e][310] NR_exto71GHz_BSRF, Moderator (Nokia)

- R4-2214228, Email discussion summary for [104-e][110] NR_ext_to_71GHz_Part_1, Moderator (Intel)

- R4-2214229, Email discussion summary for [104-e][111] NR_ext_to_71GHz_Part_2, Moderator (Qualcomm)

- R4-2214299, Email Discussion Summary for [104-e][310] NR_exto71GHz_BSRF, Moderator (Nokia)

- R4-2214422, WF on system parameters for FR2-2, Intel Corporation

- R4-2214430, Draft CR for TS 38.101-2 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214431, Draft CR for TS 38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2, Charter Comm Inc.

- R4-2214480, Draft CR for TS 38.104 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

- R4-2214883, Draft CR to 38.101-2 on band n263 Tx aspects, Qualcomm Incorporated, Apple Inc.

- R4-2214884, Draft CR to 38.101-2 on band n263 Rx aspects, Qualcomm Incorporated, Ericsson

- R4-2214890, Reply LS for the minimum guard period between two SRS resources for antenna switching, CATT

- R4-2215142, Draft CR for n263 RMC, Huawei, HiSilicon

- R4-2215255, Big CR of TS38.101-2 for FR2-2 UE requirements, Intel Corporation

- R4-2215256, Big CR of TS38.101-3 to add new NR_CADC 2BDL_xBUL combinations containing FR1 + FR2-2, Intel Corporation

- R4-2215257, Big CR of TS38.104 on system parameter updates for FR2-2, Nokia, Nokia Shanghai Bell

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template