3GPP TSG RAN meeting #97-e									       RP-222493

Electronic Meeting, September 12-16, 2022

Status Report to TSG

Agenda item:	9.5.2.4

| WI / SI Name | Further Multi-RAT Dual-Connectivity enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_NR_DC_enh2-Perf |  |  |  |  |
| Unique ID | 860249 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201040 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: | Performance Part: 100% | Testing part: |  |

Source:

| Leading WG |  | RAN WG 2 |
| --- | --- | --- |
| Rapporteur | Name | David Lecompte |
|  | Company | Huawei |
|  | Email | david.lecompte@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN4

2.1.1	Agreements

The following test cases were endorsed and the big-CR to 38.133 for MR-DC enh RRM (Rel-17) was email approved in [R4-2215227].

Temporary RS based SCell activation

- R4-2212982: Introduce Aperiodic CSI-RS for SCell activation configurtion for fast SCell activation delay.

- R4-2211901: Introduce test case for fast SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle.

- R4-2212420: Introducing test case for Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle in EN-DC

- R4-2212984: Introduce the following test cases for fast SCell activation delay:

- Fast SCell Activation of SCell in FR2 intra-band (EN-DC tests with NR cell in FR2)

- Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle (NR standalone tests with NR cell in FR1)

- Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle(NR standalone tests with NR cell in FR1)

- Fast SCell Activation for SCell in FR2 inter-band (NR standalone tests with NR cell in FR2)

Efficient activation/de-activation mechanism for one SCG

- R4-2214937: Introduce the test case for SCG activation/deactivation for EN-DC with NR cell in FR1.

- R4-2212155: Introduce the test requirements for the interruption due to RRM measurements and RLM/BFD configured on the deactivated PSCell when under EN-DC.

- R4-2214604: Introduction Test for PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2.

- R4-2214988: Introduce Test case for PSCell RACH-less activation and deactivation delay (EN-DC tests with NR cell in FR2)

Conditional PSCell change and addition

- R4-2212986: Introduce test for conditional PSCell addition delay requirements in FR2 EN-DC.

- R4-2213020: Introduce the test cases on conditional addition and release delay of PSCell in FR1 EN-DC.

- R4-2213021: Introduce the test cases on conditional addition and release delay of PSCell in FR2 SA.

2.1.2	Remaining Open issues

None for performance part.

3.	References

- R4-2212982, ATRS configurations for fast SCell activation, Huawei, HiSilicon

- R4-2211901, Test case: Fast SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle, Apple

- R4-2212420, Test case for Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle in EN-DC, MediaTek

- R4-2212984, Test cases on fast SCell activation, Huawei, HiSilicon

- R4-2214937, DraftCR on TC11 for FR1 PSCell activation and deactivation delay in EN-DC tests, OPPO

- R4-2212155, EN-DC test cases for interruptions due to RRM measurements on deactivated NR PSCell in FR1, Intel

- R4-2214604, DraftCR TC#13 PSCell activation and deactivation delay, Nokia, Nokia Shanghai Bell

- R4-2214988, Test case on efficient activation/de-activation mechanism for one SCG, Huawei, HiSilicon

- R4-2212986, Test case on Conditional PSCell addition and release, Huawei, HiSilicon

- R4-2213020, Draft CR on test case for conditional addition and release of PSCell in FR1 EN-DC, vivo

- R4-2213021, Draft CR on test case for conditional addition and release of PSCell for FR2 SA, vivo