3GPP RAN TSG Meeting #97		RP-222555

Electronic meeting, 12 – 16th September 2022

Agenda item:	9.2.1

Source:	Apple Inc.

Title:	Regulatory update for the 6GHz frequency range

WI/SI:	FS_6GHz_LTE_NR

Release:	Rel-17

Document for:	Approval

# 1	Introduction

Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations is the RAN level study item, which aims at capturing the latest information and status of the regulatory decisions for the 6GHz frequency range. In this discussion paper we present a text proposal for the corresponding TR 37.890 that captures the following information:

-	Regulatory decisions in the ITU region 3 (New Zealand, Japan, Malaysia).

# 2	Text proposal

-------------------------------------------------- TP BEGIN --------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	RP-172804: “Feasibility Study on 6 GHz for LTE and NR”, Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]					ITU-R Radio Regulations, Articles, Edition 2016;

[4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017;

[5]	FCC 17-104, Notice of Inquiry, “Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz”;

[6]	Comments of IEEE 802, in GN Docket No. 17-183;

[7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183;

[8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183;

[9]	Comments of Ericsson, in GN Docket No. 17-183;

[10]	Comments of T-Mobile USA, in GN Docket No. 17-183;

[11]	Comments of Verizon, in GN Docket No. 17-183;

[12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183;

[13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183;

[14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183;

[15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183;

[16]	Comments of CTIA, in GN Docket No. 17-183;

[17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183;

[18] 	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183;

[19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations;

[20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	ETSI TR 103 524 System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”, v1.1.1, October 2018

[23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

[24]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, May 2019.

[25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018

[26]	ETSI TR 103 612, "IMT cellular networks; Mobile/Fixed Communication Network (MFCN) in the frequency range 6 425 - 7 125 MHz", v1.1.1, December 2019

[27]	ETSI TR 103 631, "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz", v1.1.1, March 2019

[28]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz”

[29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

[30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[31]				Report and order and further notice of proposed rulemaking, FCC 20-51

[32]	ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz”, 21 May 2020

[33]	CEPT Report 075, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation. , November 2020

[34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)”, November 2020

[35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz"

[36]	EN 303 687, “"6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

[37]	doc 2.1_LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits), [to be updated]

[38]	CEPT Report 073, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

[39]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

[40]	Ofcom,  "Improving spectrum access for Wi-Fi; Spectrum use in the 5 GHz and 6 GHz bands", July 2020, URL: https://www.ofcom.org.uk/__data/assets/pdf_file/0036/198927/6ghz-statement.pdf

[41]	Communications & Information Technology Commission, "Spectrum Outlook for Commercial and Innovative Use 2021- 2023", January 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/Spectrum%20Outlook%20for%20Commercial%20and%20Innovative%20(2021-2023).pdf

[42]	Innovation, Science and Economic Development Canada, "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band", May 2021, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11698.html

[43]	National Telecommunications Agency (ANATEL), "ACT NO. 1306", February 2021, URL: https://sei.anatel.gov.br/sei/modulos/pesquisa/md_pesq_documento_consulta_externa.php?eEP-wqk1skrd8hSlk5Z3rN4EVg9uLJqrLYJw_9INcO7uvjUt3vSOwT_4Z5fukj9yIzPErY4KWH5cpE9W_9hcTZkCG-vLPIdpXyuhgMG-L9M-uBLoSdAAXO0clb3SIt1i

[44]	Ministerio de Transportes y Comunicaciones, "Resolución Ministerial N° 373-2021-MTC/01", April 2021, URL: https://cdn.www.gob.pe/uploads/document/file/1861732/Resolución%20Ministerial%20nro%20373-2021-MTC/01.pdf

[45]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1985 EXENTA", October 2020, URL: https://www.bcn.cl/leychile/navegar?idNorma=1109333&idParte=9841504&idVersion=&r_c=6

[46]	RP-210957, Liaison statement to 3GPP TSG RAN on inclusion of the frequency band 6425-7125 MHz in 3GPP specification for 5G-NR/IMT-2020 systems, Regional Commonwealth in the filed of Communications.

[47]		COMMISSION IMPLEMENTING DECISION (EU) 2021/1067, on the harmonised use of radio spectrum in the 5 945-6 425 MHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs), 17 June 2021.

[48]		The National Telecommunications Commission (CONATEL) of Honduras, "Resolution NR 003/21", March 2021, URL: http://www.conatel.gob.hn/doc/Regulacion/resoluciones/2021/NR003-21.pdf

[49]		The Superintendencia de Telecomunicaciones (SUTEL), "DECRETO EJECUTIVO N° 42924-MICITT", April 2021, URL: https://www.imprentanacional.go.cr/pub/2021/04/30/ALCA87_30_04_2021.pdf

[50]	Communications & Information Technology Commission, "Radio Spectrum Allocation and Use Regulation for WLAN Application", June 2021, URL: https://www.citc.gov.sa/en/new/publicConsultation/Documents/144207-en.pdf

[51]	Communications & Information Technology Commission, “Public Consultation on Spectrum Light Licensing”, August 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/EN_PublicConsultationonLightLicensing-144301.pdf

[52]	Communications & Information Technology Commission, “WLAN regulations”, January 2022, URL: https://www.citc.gov.sa/en/mediacenter/pressreleases/Pages/2022022101.aspx

[53]	UAE Telecommunication Regulatory Authority, "Ultra-Wide Band and Short Range Devices", Version 4.0, December 2020, URL: https://tdra.gov.ae/-/media/About/regulations-and-ruling/EN/UWB-and-SRD-Regs-V4-0-pdf.ashx

[54]	National Telecommunications Regulatory Agency (ANRT), "Decision ANRT/DG/N°07/21", May 2021, URL: https://www.anrt.ma/sites/default/files/decision_a2fp_-vf-_mod_07.05.2021.pdf

[55]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1325 EXENTA", July 2021, URL: https://www.bcn.cl/leychile/navegar?idNorma=1162978&idParte=10255091

[56]	Agencia Nacional del Espectro (ANE), "Por medio de la cual se modifica la Resolución 105 de 2020 y se actualiza el Cuadro Nacional de Atribución de Bandas de Frecuencia", October 2021, URL: https://www.ane.gov.co/Documentos%20compartidos/ArchivosDescargables/Normatividad/Planeacion_del_espectro/RESOLUCIÓN%20No%20000105%20DE%2027-03-2020(1)%20(1).pdf

[57]	African Telecommunication Union, "ATU-R Recommendation 005-0", July 2021, URL: https://www.atuuat.africa/wp-content/uploads/2021/08/En_ATU-R-Recommendation-005-0.pdf

[58]	African Telecommunication Union, "2nd ATU Preparatory Meeting for WRC-23 (APM23-2); Agenda Item 1.2", September 2021, URL: https://docs.google.com/document/d/1u0Oz6o3gnH6oTGY-sE6_BTarNf2RUuSx/edit

[59]		RCC Recommendation 1/21 “Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions” (RP-213605).

[60]	Recommendation ITU-R SM.329-12, Unwanted emissions in the spurious domain, 09/2012.

[61]	The Australian Communications and Media Authority, "Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1)", March 2022, URL: https://www.legislation.gov.au/Details/F2022L00249/Html/Text

[62]	HKCA 1081, " PERFORMANCE SPECIFICATION FOR RADIOCOMMUNICATIONS APPARATUS OPERATING IN THE 6 GHz BAND FOR WIRELESS LOCAL AREA NETWORK", April 2022, URL: https://www.ofca.gov.hk/filemanager/ofca/en/content_401/hkca1081.pdf

[63]	Radio Spectrum Management, "WLAN use in the 6 GHz Band: Outcomes and Other Updates", August 2022, URL: https://www.rsm.govt.nz/projects-and-auctions/completed-projects/wlan-use-in-the-6-ghz-band/

[64]	Ministry of Internal Affairs and Communications, "Technical conditions for introduction of 6GHz band wireless LAN", April 2022, URL: https://www.soumu.go.jp/menu_news/s-news/01kiban12_02000142.html

[65]	Malaysian Communications and Multimedia Commission (MCMC), "Class Assignment No. 1 of 2022", January 2022, URL: https://www.mcmc.gov.my/skmmgovmy/media/General/CA-No-1-of-2022_-signed_19012022.pdf

-------------------------------------------------- next section --------------------------------------------------

## 4.3	ITU Region 3

### 4.3.1	China

In April 2019, CCSA established a new project on technical report for the feasibility study of IMT system using 5925-7125 MHz frequency band [29].

This CCSA study includes identifying IMT parameters in 5925-7125MHz in order to develop coexistence between IMT system and incumbent services and applications.

Editor’s note: This work is expected to be finalized in 2020.

### 4.3.2	South Korea

In June 2020, Korea’s Ministry of Science and ICT (MSICT) issued an amendment of technical standards that also includes proposed rules for the 5925–7125MHz frequency band, which was finally approved in October 2020 [39]. The key points are as follows:

-	The whole range of 5925–7125MHz is allocated for the license-exempt low power indoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 24dBm;

-	the maximum power spectral density is 2dBm/MHz;

-	a device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply;

-	this frequency range cannot be used for indoor automotive or airplane deployments;

-	usage inside moving vehicles is prohibited.

-	The 5925-6425MHz frequency range is allocated for the license-exempt very low power indoor and outdoor:

-	the occupied channel bandwidth should be less or equal 160MHz;

-	the EIRP limit is 14dBm;

-	the maximum power spectral density is 1dBm/MHz;

-	this frequency range cannot be used for drones;

-	automotive embedded systems shall use 6085-6425MHz.

-	Unwanted emissions at frequencies outside the operational frequency range shall be below -27dBm/MHz.

-	Unwanted emissions at frequencies outside the operational frequency of 5925-6445MHz (very low power under the conditions for that range described above) shall be below -34dBm/MHz.

-	Transmitter and receiver spurious emission limits should be as follows:

| Frequency Range | Maximum level (average) | Measurement bandwidth |
| --- | --- | --- |
| f<1GHz | -54dBm | 100kHz |
| f>1GHz | -47dBm | 1MHz |

### 4.3.3	Hong Kong

In April 2022, the Office of the Communication Authority (OFCA) published HKCA 1081 [62] for devices operating in the 6GHz band. This specification defines the minimum performance requirements for radiocommunications devices operating in the 6 GHz band for wireless local area networks. The apparatus shall operate in the 5.925 – 6.425 GHz frequency range and shall meet the technical requirements specified in the standard EN 303 687. According to [62], the RF output power limit of 23 dBm EIRP and the power spectral density limit of 10 dBm/MHz EIRP as specified in clauses 4.3.2 and 4.3.3 of EN 303 687 are allowed to be increased by 1 dB to 24 dBm EIRP and 11 dBm/MHz EIRP respectively

### 4.3.4	Australia

In March 2022, The Australian Communications and Media Authority makes the following variation under subsection 132(1) of the Radiocommunications Act 1992 [61]:

-	Low power indoor transmitters:

-	Operating frequency 5925-6425MHz;

-	The transmitter must only be used indoors;

-	Maximum EIRP of 250mW;

-	The power spectral density of the transmitter must not exceed 12.5mW EIRP per MHz;

-	Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented.

-	Very low power indoor/outdoor transmitters:

-	Operating frequency 5925-6425MHz;

-	Maximum EIRP of 25mW;

-	The power spectral density of the transmitter must not exceed 1.25mW EIRP per MHz;

-	Contention-based protocols for multiple access, such as Carrier Sense Multiple Access (CSMA) or Multiple Access Collision Avoidance (MACA), must be implemented.

### 4.3.5	New Zealand

In August 2022, the New Zealand Radio Spectrum Management (RSM) body agreed to make the 5925 – 6425 MHz frequency band available for Wireless Local Area Network (WLAN) under the Low Power Indoor (LPI) use and the Very Low Power (VLP) Indoor and Outdoor with the following technical conditions [63]:

-	Low power indoor:

-	Indoor use only within a building or within an enclosed space having attenuation characteristics at least equivalent to those of a building;

-	Maximum EIRP of 24dBm (250mW);

-	Maximum power density of 11dBm/MHz (12.6mW/Hz).

-	Very low power indoor/outdoor:

-	Maximum EIRP of 14dBm (25.11mW);

-	Maximum power density of 1dBm/MHz (1.26mW/Hz).

### 4.3.6	Japan

In April 2022 Japanese Ministry of Internal Affairs and Communications published the document describing technical conditions for the 6GHz band wireless LAN:

-	LPI and VLP mode in the frequency range of 5925-6425MHz;

-	Maximum EIRP should be less than 200mW in LPI mode and less than 25mW in VLP mode;

-	For the LPI mode frequency use is limited to indoors;

-	Operating channel bandwidth size of 20, 40, 80, and 160MHz with the following centre frequencies:

-	20MHz channel: 5955, 5975, 5995, 6015, 6035, 6055, 6075, 6095, 6115, 6135, 6155, 6175, 6195, 6215, 6235, 6255, 6275, 6295, 6315, 6335, 6355, 6375, 6395, 6415 MHz;

-	40MHz channel: 5965, 6005, 6045, 6085, 6125, 6165, 6205, 6245, 6285, 6325, 6365, 6405 MHz;

-	80MHz channel: 5985, 6065, 6145, 6225, 6305, 6385 MHz;

-	160MHz channel: 6025, 6185, 6345 MHz;

-	Adjacent channel leakage power:

-	20MHz channel: -25dBc/20MHz for the 20MHz offset and -40dBc/20MHz for the 40MHz offset;

-	40MHz channel: -25dBc/40MHz for the 40MHz offset and -40dBc/40MHz for the 80MHz offset;

-	80MHz channel: -25dBc/80MHz for the 80MHz offset and -40dBc/80MHz for the 160MHz offset;

-	160MHz channel: -25dBc/160MHz for the 160MHz offset and -40dBc/160MHz for the 320MHz offset MHz;

### 4.3.7	Malaysia

In January 2022, Malaysian Communications and Multimedia Commission (MCMC) published a new class assignment document, which in particular specifies unlicensed operation in the 5925-6425MHz frequency range with the following technical conditions:

-	The maximum EIRP for indoor use only is limited to 200mW;

-	The radiated PSD for indoor devices is limited to 10mW/MHz;

-	The maximum EIRP for indoor and outdoor devices is limited to 25mW;

-	The radiated PSD for outdoor and indoor devices is limited to 1.25mW/Hz;

-	The radiated PSD for narrowband usage (channel bandwidth below 20MHz) for outdoor and indoor devices is limited to 10mW/MHz;

-------------------------------------------------- next section --------------------------------------------------

## 4.5	Regulatory parameters comparative for license-exempt

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.3.6) |  | 14dBm |  |  |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |

-------------------------------------------------- TP END --------------------------------------------------