3GPP TSG-RAN Meeting #97-e	R4-222620

Electronic Meeting, 12-16 September, 2022

Title: 	TP for TR38.893 Band Subsets: Other issues regarding regulatory certification

Agenda Item:	9.2.2

Source: 	Qualcomm Incorporated

Document for:	Approval

- Introduction

This is a text proposal for the TR for Study on UE support of regionally-defined subsets of an NR band, (new RAN SI: FS_NR_subset_band_support). This TP proposes text for a new section titled ‘Other issues’ to be inserted prior to the ‘Possible solutions’ section.

- Text Proposal

<start change>

# 4	Root cause

# 5	Other Issues

## 5.1	Roaming UEs

In some countries it is neither the operator nor the base station that takes responsibility for the UE meeting regulations.  In the USA UE vendors can only allow their devices to operate in parts of the band where there are FCC regulations at the time of device certification.

UE vendors may choose to pursue certification for some countries but not others. In view of the regional regulatory differences described above, we recognize that there may be ambiguity with regards to exactly which regulatory requirements are supported in any given UE implementation.   It is important to understand how this ambiguity could affect regulatory compliance when UEs roam in countries in which they are not certified. Two scenarios are identified for clarification for feasibility of roaming:

The first scenario is if a UE is certified to operate only in part of a band (e.g. band n77 cases UE in Canada/US).  In this scenario, the UE is only allowed to operate within the subset of the band for which it is certified in the country where this certification is necessary.  Because the UE in addition to the above regulatory certification is also compliant to 3GPP requirements (e.g., for Band n77), it is required that the UE can support the entire band.  Therefore, this UE when roaming to a country which does not have any regional regulatory requirements shall be able to operate on the entirety of the band.  If this country does have regional requirements, the UE shall abide by them to operate in this country.

The second scenario is if a UE can support the frequency range of a band based on the 3GPP requirements but not certified for additional regional requirements in a subset of the band in a certain foreign country. This aspect has to be evaluated for each country based on prevailing regulation.  Whether the roaming UE is allowed to operate without certification from the country’s regulator is subject to each country’s laws.

# 6	Possible solutions

## 6.1	Solution 1

## 6.2	Solution 2

# 7	Conclusions and recommendations

<end change>