3GPP TSG-RAN WG1 Meeting #111	R1-2212753

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0373 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UE TEG margin value report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (CATT), ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4#104e meeting, RAN4 discussed UE/TRP TEG framework and provided the following feedback: Issue #6: Questions on UE Rx/RxTx TEG margins RAN4 feedback: - UE Rx/RxTx TEG margins are provided as LPP signalling parameters out of UE capability signaling. - A single timing error margin value is provided per Rx TEG/RxTx TEG type per measurement instance in a single LPP message, if it has multiple measurement instances. - The timing error margin values for an Rx TEG/RxTx TEG type in different LPP messages can be different.. Also, the TS 37355-h20 and TS 38331-h20 provided by RAN2 captures TEG timing error margins. However, currently the report of UE Tx/Rx/RxTx TEG margin value is not included in RAN1’s spec. TS 38.214 should be updated accordingly. |
| Summary of change: |  | Add description on UE Tx/Rx/RxTx TEG margin value according to RAN4’s agreement and TS 37355-h20 and TS 38331-h20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The framework to UE TEG is not completely captured in RAN1’s specification. TS 38.214 does not include the UE behavior on report of UE Tx/Rx/RxTx TEG margin value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5, 6.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.6.5 PRS reception procedure

<Unrelated part omitted>

Timing Error Group(s) (TEG(s)) at UE side are defined:

-	UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin.

-	UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

The UE may be configured to report, subject to UE capability, via high layer parameter nr-UE-RxTEG-Request, the association information of DL RSTD measurement(s) with UE Rx TEG(s) via higher layer parameter nr-UE-Rx-TEG-ID when the UE reports the DL RSTD measurement(s). The UE may report up to 4 RSTD measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

The UE may report a UE Rx TEG ID via higher layer parameter nr-UE-Rx-TEG-ID for a RSTD reference time dl-PRS-ReferenceInfo and a UE Rx TEG ID for each DL RSTD measurement, where the DL RSTD can be DL RSTD measurement in NR-DL-TDOA-MeasElement and/or NR-DL-TDOA-AdditionalMeasurementElement.

If the UE reports a UE Rx TEG ID with a DL RSTD measurement, the UE may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-DL-TDOASignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, RSTD measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs with the same dl-PRS-ReferenceInfo. The higher layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be provided with association information of DL PRS resource(s) with Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with UE RxTx TEG(s) via higher layer parameter nr-UE-RxTx-TEG-ID. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE RxTx TEG per dl-PRS-ID.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE RxTx TEG timing error margin value, via high layer parameter nr-UE-RxTxTEG-TimingErrorMargin, for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to report, via high layer parameter nr-UE-RxTxTEG-Request, subject to UE capability, the association information of UE Rx-Tx time difference measurement(s) with the UE Rx TEG(s) and UE Tx TEG(s) via the higher layer parameters of nr-UE-Rx-TEG-ID, and nr-UE-Tx-TEG-Index. The UE may report up to 4 UE Rx-Tx time difference measurements associated with different DL PRS resources per UE Rx TEG per dl-PRS-ID.

If the UE reports a UE Rx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Rx TEG timing error margin value, via high layer parameter nr-UE-RxTEG-TimingErrorMargin, for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs subject to UE capability, UE Rx-Tx time difference measurements on a PRS resource associated with a dl-PRS-ID using up to 8 different UE Rx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTEGs applies to all DL PRS positioning frequency layers.

The UE may be configured to measure and report, via high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs subject to UE capability, UE Rx-Tx time difference measurements with the same UE Tx TEG using up to 8 different UE RxTx TEGs. The high layer parameter measureSameDL-PRS-ResourceWithDifferentRxTxTEGs applies to all DL PRS positioning frequency layers.

<Unrelated part omitted>

#### 6.2.1.4 UE sounding procedure for positioning purposes

<Unrelated part omitted>

Timing Error Group (TEG) at UE side is defined:

-	UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin.

The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter nr-SRS-TxTEG-Set or ue-TxTEG-AssociationList.

The UE may report, via high layer parameter ue-TxTEG-TimingErrorMarginValue, the UE Tx TEG timing error margin value of all the UE Tx TEGs within one UEPositioningAssistanceInfo.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, as defined in clause 5.1.6.5, the UE shall report the association information of the already transmitted SRS resources configured by the higher layer parameter SRS-PosResource with the UE Tx TEG ID.

If the UE is configured with SRS resources configured by the higher layer parameter SRS-PosResource in multiple CCs, the UE should report the carrierFreq or servCellId of the SRS resources when it reports the UE Tx TEG associations.

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a Tx TEG ID.

If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation.

<Unrelated part omitted>