3GPP TSG-RAN WG3 Meeting #118	R3-226113

# Online, 14-18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0087 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Timing Error Margin |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 has introduced in RAN3#117 e-meeting the Timing Error Margin IE for the UE Tx Timing Error Margin, TRP Rx Timing Error Margin, TRP Tx Timing Error Margin and TRP RxTx Timing Error Margin. A note was captured by the CB’s moderator in the Summary of R17 Positioning Corrections SoD (R3-224993) stating that some checking is pending RAN4 progress regarding the TRP RxTx Timing Error Margin: Note: Maybe we need to revisit the values for TRP RxTx Timing Error Margin according to RAN4 progress. In RAN4’s latest LS reply to RAN1 in R3-225316, RAN4 confirms the value range of the timing error margin for RxTx TEG as follows: - (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc. We can see that these values do not match with all the codepoints in the Timing Error Margin IE. A correction is therefore necessary. We propose to correct this problem in an NBC way, considering that companies are alerted to the need for verification and that such functionality should not be implemented hastily. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changing the type of the TRP RxTx Timing Error Margin IE in 9.2.80 to ENUMERATED (Tc0dot5, Tc1, Tc2, Tc4, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc64, Tc80, Tc96, Tc128, …). Impact Analysis: Impact assessment towards the previous version of the specification (same release). This CR has NBC change affecting the TRP TEG ID Information IE in 9.2.80 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-alignment with RAN4 specs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.80, 9.2.84, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1054 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: resubmission to RAN3#118 |  |  |  |  |  |  |  |  |

<Start of changes>

### 9.2.80	TRP TEG ID Information

This information element contains the TRP RxTx TEG ID group information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP TEG ID | M |  |  |  | - |  |
| >RxTx TEG |  |  |  |  | - |  |
| >>TRP RxTx TEG ID | M |  | INTEGER (0..255) |  | - |  |
| >>TRP Tx TEG ID | O |  | INTEGER (0..7) |  | - |  |
| >>TRP RxTx Timing Error Margin | O |  | ENUMERATED (Tc0dot5, Tc1, Tc2, Tc4, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc64, Tc80, Tc96, Tc128, …) | Timing error margin associated to the TRP RxTx TEG ID. | YES | ignore |
| >>TRP Tx Timing Error Margin | O |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. | YES | ignore |
| >Rx TEG |  |  |  |  | - |  |
| >>TRP Rx TEG ID | M |  | INTEGER (0..31) |  | - |  |
| >>TRP Tx TEG ID | M |  | INTEGER (0..7) |  | - |  |
| >>TRP Rx Timing Error Margin | O |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Rx TEG ID. | YES | ignore |
| >>TRP Tx Timing Error Margin | O |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. | YES | ignore |

<Next change>

### 9.2.84	Timing Error Margin

This information element contains the Timing error margin for the UE Tx TEG, TRP Rx TEG, or TRP Tx TEG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timing Error Margin | M |  | ENUMERATED(Tc0, Tc2, Tc4, Tc6, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc56, Tc64, Tc72, Tc80, …) |  |

<Next change>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

<unchanged text omitted>

TRPTEGIDInformation ::= CHOICE {

rxTx-TEG			RxTxTEG,

rx-TEG				RxTEG,

choice-extension		ProtocolIE-Single-Container { { TRPTEGIDInformation-ExtIEs} }

}

TRPTEGIDInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

RxTxTEG ::= SEQUENCE {

tRP-RxTx-TEGID		INTEGER (0..255),

tRP-Tx-TEGID		INTEGER (0..7)	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { RxTxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-TRPRxTxTimingErrorMargin		CRITICALITY ignore EXTENSION RxTxTimingErrorMargin 	PRESENCE optional }|

{ ID id-TRPTxTimingErrorMargin			CRITICALITY ignore EXTENSION TimingErrorMargin 	PRESENCE optional },

...

}

RxTxTimingErrorMargin ::= ENUMERATED {tc0dot5, tc1, tc2, tc4, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc64, tc80, tc96, tc128, ...}

RxTEG ::= SEQUENCE {

tRP-Rx-TEGID		INTEGER (0..31),

tRP-Tx-TEGID		INTEGER (0..7),

iE-extensions      ProtocolExtensionContainer { { RxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-TRPRxTimingErrorMargin			CRITICALITY ignore EXTENSION TimingErrorMargin 	PRESENCE optional }|

{ ID id-TRPTxTimingErrorMargin			CRITICALITY ignore EXTENSION TimingErrorMargin 	PRESENCE optional },

...

}

TimingErrorMargin 			::= ENUMERATED {tc0, tc2, tc4, tc6, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc56, tc64, tc72, tc80, ...}

<End of change>