3GPP TSG-RAN WG3 Meeting #118	R3-226120

# Toulouse, France, 14-18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1058 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of ASN.1 for UL RTOA Measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Extended Additional Path List, TRP Rx TEG ID, and TRP Rx Timing Error Margin are IEs added to the UL RTOA Measurement IE in R17. However, in ASN.1 of TS 38.473, these three IEs, which should have been added to the extension of UL-RTOA-Measurement-ExtIEs, are mistakenly added into the UL-RTOA-MeasurementItem-ExtIEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the three IEs Extended Additional Path List, TRP Rx TEG ID, and TRP Rx Timing Error Margin from the UL-RTOA-MeasurementItem-ExtIEs. - Add them into the UL-RTOA-Measurement-ExtIEs. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact on the protocol as it moves some IEs from UL-RTOA-MeasurementItem-ExtIEs to UL-RTOA-Measurement-ExtIEs. This CR has no functionality impact. This CR can be considered isolated. This CR is non-backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist in ASN.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1, resubmission to #118 meeting. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

UL-RTOA-Measurement ::= SEQUENCE {

uL-RTOA-MeasurementItem		UL-RTOA-MeasurementItem,

additionalPath-List			AdditionalPath-List OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UL-RTOA-Measurement-ExtIEs } }	OPTIONAL

}

UL-RTOA-Measurement-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRPRXTEGID					CRITICALITY ignore EXTENSION INTEGER (0..31)				PRESENCE optional }|

{ ID id-TRPRxTimingErrorMargin		CRITICALITY ignore EXTENSION TimingErrorMargin 			PRESENCE optional },

...

}

UL-RTOA-MeasurementItem ::= CHOICE {

k0					INTEGER (0..1970049),

k1					INTEGER (0..985025),

k2					INTEGER (0..492513),

k3					INTEGER (0..246257),

k4					INTEGER (0..123129),

k5					INTEGER (0..61565),

choice-extension			ProtocolIE-SingleContainer { { UL-RTOA-MeasurementItem-ExtIEs } }

}

UL-RTOA-MeasurementItem-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore TYPE ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRPRXTEGID					CRITICALITY ignore TYPE INTEGER (0..31)				PRESENCE optional }|

{ ID id-TRPRxTimingErrorMargin		CRITICALITY ignore TYPE TimingErrorMargin 		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>