3GPP TSG RAN meeting #98-e									RP-222900

Electronic Meeting, Dec. 2022

## Status Report to TSG

Agenda item:			9.3.4.4

Title		:	Status report for WI NR_FR2_multiRx_DL; rapporteur: Qualcomm, vivo

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR2_multiRX_DL |  |  |  |  |
| Unique ID | 950077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221753 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 09/2023 | Performance part: 03/2024 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: 25% | Performance Part: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Sumant Iyer |
|  | Company | Qualcomm Inc. |
|  | Email | sumanti@qti.qualcomm.com |
| Rapporteur | Name | Qian Yang |
|  | Company | vivo |
|  | Email | qian9.yang@vivo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Void

## 2.2	RAN2

#### Void

## 2.3	RAN3

#### Void

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.1.1	UE RF Core

##### 2.4.1.1.1	From RAN4#104-e: WF R4-2214457

- To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

- UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

- Work plan:

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Discussion on system assumptions, UE assumptions | Agree on system assumptions |
| RAN4#104-Bis | Oct. 2022 | UE implementation assumptions and requirement concept |  |
| RAN4#105 | Nov. 2022 | UE implementation assumptions and requirement concept | Agree on UE assumptions |
| RAN4#106 | Feb. 2023 | Requirement concept and simulation discussion | Agree on details pertaining to simulation effort |
| RAN4#106-Bis | Apr. 2023 | Simulation results discussion | Draft CR? |
| RAN4#107 | May 2023 | Requirement parameter details | Draft CR |
| RAN4#108 | Aug. 2023 | Requirement parameter details | Feature CR |

##### 2.4.1.1.2	From RAN4#104-Bis-e:

###### 2.4.1.1.2.1	WF R4-2217731

- ‘Panel’ is not referenced in the final UE RF requirements

- Take the following panel assumption for DL reception, noting panel is a logical construct:

- ‘Panel’ is defined as one or multiple as combination of below depending on different UE implementation:

  - Unit of antenna group to control beam independently

  - Within a panel, one beam can be selected and used for DL reception.

  - Across different panels, multiple beams [(each selected per panel)] may be used for DL reception.

  - ‘Beam’ is assumed to mean spatial filter associated with reception.

- Way forward: FFS whether the mutual interference between two receive beams need to be considered and whether /how to address it in requirement

- [The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. Whether a UE with single antenna module can satisfy the requirement or not will be an implementation issue].

- [If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on 1.2 ]

- FFS how to treat the small AoA separation condition:

  - Option 1: The requirements for FR2 multi-RX chain DL do not apply when angle separation smaller than a minimum threshold.

  - Option 2: The RF requirement for any AoA pair is defined with assumption that TRP1 uses  polarization when TRP2 uses  polarization and vice-versa (and are the angular coordinates of the test system grid).

  - Option 3: Both option 1 and option 2 can be considered.

  - Option 4: Others.

- Further study the power imbalance together with how to specify 2AoA sensitivity

- For single DCI, take the RMC in Annex 1 as starting point for further discussion. (see WF)

  - FFS whether the existing 2-layer demod RMC can be reused

- For multi-DCI, FFS the existing RMC for R15 sensitivity can be reused.

- FFS what is the key performance gain need to be verified for multi-Rx DL reception and how to construct the requirement:

  - Option 1: Coverage performance, e.g., proposal 1,2,3,4,7 in Annex2 (see WF)

  - Option 2: Simultaneous reception, e.g., proposal 5,6 in Annex2 (see WF)

  - Option 3: Others

###### 2.4.1.1.2.2	WF R4-2217732

- On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - For UEs supporting single DCI, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of singleDCI-SDM-scheme-r16

    - Support of 4L DL MIMO

  - For UEs supporting multi-DCIs, assume the following set of UE capabilities as pre-requisites:

    - Support of simultaneousReceptionDiffTypeD-r16

    - Support of multiDCI-MultiTRP-r16

    - Support of 4L DL MIMO

  - FFS if and how RAN4 aims to specify the same RF requirements to accommodate both types of UEs

  - Further consider the following options:

    - Option 1: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used.

    - Option 2: UE RF requirements shall not preclude UEs that do not support the single DCI scheme.

- For core requirement and testing, it is assumed that both SSB and CSI-RS are configured to UE for beam management

- UE panel assumption should follow implementation agonistic manner.

  - FFS on how to determine a minimum benefit that a UE must demonstrate when configured for this feature.

  - The term “panel” is not used in test configuration.

- Requirement discussions need to consider testability issue so that the defined requirement can be properly verified.

##### 2.4.1.1.3	From RAN4#105: WF R4-2220553

- ‘The UE RF requirement is based on a test system that can support multiple fixed relative AoA locations during test.’ is the starting point:

  - Multiple fixed orientation of the AoAs or single fixed orientation of AoA can be considered for test

  - Multiple fixed AoA offset values or single fixed AoA offset value can be considered for core requirement

- Candidate proposals for UE RF requirement concept (others not precluded):

  - Option 1: Requirement is based on 2AoA directional sensitivity statistics

  - Option 2: Requirement is based on the spherical coverage EIS of AoA1 (which is swept over the full sphere), and a fixed/pre-defined power level for AoA2

  - Option 3: Only verify the UE functionality (e.g., go or no-go) under two AoAs with a fixed DL power level

  - Option 4: RAN4 considers specifying the demod requirements of multi-Rx in FR2 with pre-defined side condition, instead of defining the two-AoA spherical coverage requirement

- FFS on how to specify the RF requirements to ensure the minimum benefit of two AoAs for the network.

- No more than 2AoA cases are considered in this release as specified in the WID.

- DL polarizations:

  - Option 1: Multiple aspects may be chosen:

    - Aspect 1: The RF requirement can be defined for any AoA pair with assumption that TRP1 uses  polarization when TRP2 uses  polarization and vice-versa without any limitation of angular separation between two AOAs. (R4-2219125)

    - Aspect 2: Limit the polarization combinations (from 4 possible:   and )) for the 2-DL spherical coverage test case (R4-2219852)

  - Option 2: Take the mTRP system level assumptions in TR38.802 as the baseline and further discuss whether any additional constraints on DL polarization are necessary for the RF requirement definition.

  - Other options are not precluded.

- Strive to define single set of requirements for both sDCI and mDCI.

  - Further study is needed to understand the difference between sDCI and mDCI and how to accommodate such difference in single set of requirements if feasible

- The terms ‘antenna module’ and “panel” are not referenced in the final UE RF requirement and test configuration

- The scenario where a single antenna module is used to receive two AoAs simultaneously should not be excluded. If an antenna module can be used to receive two AoAs simultaneously, it is considered to consist of at least two panels, where the understanding of “panel” is based on Proposal 1 of 1.2.11

- On “panel”,

  - ‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes

    - Within a panel, one beam can be selected and used for DL reception.

    - Across different panels, multiple beams (each selected per panel) may be used for DL reception.

    - ‘Beam’ is assumed to mean spatial filter associated with reception.

  - Confirm that a physical panel with dual polarization is assumed as two “panels”.

  - [Assuming four antenna panels per UE as the reference architecture for deriving the spherical coverage requirement of the spherical coverage requirement for the multi-Rx chain DL reception. The four panels are separated into two pairs, and each pair is composed of two antenna panels with orthogonal polarizations that are co-located.

    - Antenna panels used to receive two AoAs simultaneously is up to UE implementation ]

- For the UE supporting multi-DCI, the RMC of the single carrier can be reused for each layer

- test time optimization can be further discussed and decided in OTA SI

- Test procedures for sDCI and mDCI can be further discussed and decided in OTA SI once the requirement concept and test methodologies are confirmed. Test procedure discussion should be also aligned with requirement discussion and decision including requirement concept

- Consider AoA setup and deployment scenario as a package.

  - the mTRP assumptions achieved by RAN1 in the physical layer design (including TR38.802) can be taken as one of the references in deployment scenario discussion. Interested companies are encouraged to provide their analysis on deployment scenario and associated AoA setup in next meeting.

- The same numerology (SCS and CP) is assumed for the two AoAs in Rel-18.

#### 2.4.1.2	UE RRM Core

##### 2.4.1.2.1	From RAN4#104-e: WF R4-2214344

Number of searchers for cell detection and measurements

  - 2 searchers are assumed.

Feasibility/necessity of enhancing requirements for L1 measurements

  - Following L1 measurements related requirements are considered in the multi-Rx chain WI

    - L1-RSRP measurement

    - BFD/CBD

    - RLM

    - L1-SINR

    - TCI state switching with dual TCI

    - Scheduling/measurement restrictions

- RRM  core part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Study and initial discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of at least some of the requirements Approve the RRM work plan |
| RAN4#104-Bis | Oct. 2022 | Continue discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of all the requirements Agreements on assumptions of receive timing difference between different directions Initial agreements on the confirmed RRM requirements |
| RAN4#105 | Nov. 2022 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106 | Feb. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements Work split on the Draft CRs |
| RAN4#107 | May 2023 | Continue discussion on the remaining issues for the RRM core requirements | Initial discussion on the Draft CRs Further agreements on the confirmed RRM requirements |
| RAN4#108 | Aug. 2023 | Continue discussion on the remaining issues for the RRM core requirements | Conclude all the RRM requirements Endorse all the Draft CRs for core part of the WI |

##### 2.4.1.2.2	From RAN4#104-Bis-e:

###### 2.4.1.2.2.1	WF R4-2217246

Sub-topic 1-1: Scope and scenarios

- Joint combination of multi-Rx chain with NR-U, RedCap and NTN are not in the scope of the WI.

- RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

- FFS whether UE can be configured with multiple component carriers, including intra-band CCs and/or inter-band CCs, but multi-Rx chain is enabled on only one of the component carriers.

- Clarify the SSB and CSI-RS based simultaneous L1 measurement scenario before making conclusion on scenarios for Rel-18 multi-Rx DL reception, i.e., for simultaneous reception of different QCL type D RSs, what RSs combination(s) can be considered and which scenario(s) it is aimed at.

Sub-topic 1-2: General aspects

- UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

- Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

- Whether to define RRM requirements for activation delay from a single antenna panel to multi-antenna panels should be decided after RF conclusion.

Sub-topic 1-3: Receive timing difference

- Not to discuss UL transmission in R18 FR2 multi-Rx WI.

Sub-topic 1-4: Applicability and conditions

- RRM requirement of simultaneous DL reception from different directions shall be defined based on the applicable condition to be specified in UE RF session.

  - FFS the relation between the applicability condition and RRM requirement impacts

- To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e., no FR2-2.

###### 2.4.1.2.2.2	WF R4-2217587

- L1 RSRP/SINR Measurements:

  - The number of samples assumed to define the accuracy requirements should not be changed

- BFD/BFR:

  - CSI-RS based BFD/BFR is in the scope of the current WI

- Multi-Rx and other features:

  - Do not discuss join requirements for multi-Rx and NR-U, NTN and RedCap at least until all the requirements for multi-Rx are finalized

###### 2.4.1.2.2.3	WF R4-2217248

Sub-topic 1-1: Requirements and Scenarios

Issue 1-1-1: Requirements to be defined

Agreements:

- Define DL TCI state switch requirements. UL TCI state switching, and UL spatial relation info switch are not in the scope of the WI.

Issue 1-1-2:  Scenarios to be considered w.r.t Intra-cell/Inter-cell multi-TRP

Agreements:

- Follow conclusion from thread 211.

Issue 1-1-3:  Scenarios to be considered w.r.t single DCI/multi-DCI.

Agreements:

- Follow conclusion from thread 211.

Sub-topic 1-2: TCI state switching requirements

Issue 1-2-1: Assumptions for dual TCI state switching

Issue 1-2-1-1: Dual TCI state switching requirements shall be based on

Agreements:

- Rel-15/Rel-16 TCI framework

Issue 1-2-1-2:  Can the TCI switch is assumed to be independent on each RX chain?

Agreements:

- For each of the two TCI states, the TCI state switch is assumed to be independent

  - FFS on the definition/scope of “independency.”

Issue 1-2-2:  Switch command for dual TCI state switch

Issue 1-2-2-1: When two TCI states are switched simultaneously, assumption on the switch commands

Agreements:

- Wait for conclusion on scenarios in other thread w.r.t sDCI vs mDCI.

Candidate options for next meeting:

  - Option 1 (Vivo, Huawei): requirements are defined for following modes of switching

    - Two DCI one for each TCI state (PDSCH multiple DCI)

    - Two MAC CE one for each TCI state (PDCCH non-SFN)

    - One DCI for two TCI states (PDSCH single DCI)

    - One MAC CE for two TCI states (PDCCH SFN)

  - Other options not precluded

Issue 1-2-2-2: TCI state switch scenarios to be considered

Agreements:

- Waiting for the scenario’s conclusion in other thread w.r.t sDCI vs mDCI

- FFS on the definition of dual TCI state switching. Companies are requested bring their views to next meeting

Issue 1-2-2-3: If the proposal 1 to issue 1-2-2-2 is acceptable, can the following proposal be acceptable.

Agreements:

- Wait for conclusion on issue 1-2-2-2.

Issue 1-2-4-2: Does the cross-panel switch time needs to be defined.

Agreements:

- RRM not to define additional TCI state switching delay for cross panel TCI state switching. If RF session achieves a new conclusion on panels ON/OFF switch time, RRM session may revisit the issue if required.

##### 2.4.1.2.3	From RAN4#105:

###### 2.4.1.2.3.1	WF R4-2220742

Topic #1: Analysis of scope and scenarios

<Agreement >:

Issue 1-1-7: Assumption on supporting 4-layer MIMO with simultaneous DL reception with two different QCL TypeD RSs

- group based reporting is one of the means to enable simultaneous reception

- companies proposing to enable simultaneous reception should bring more analysis on how this can be done

<Agreement >:

Issue 1-1-4: Support of single-DCI and/or multi-DCI multi-TRP operation

- Consider both sDCI and mDCI

< Way forward >

Other open issues for scope and scenarios are captured in section 1 for Topic #1 in summary R4-2220061.

Topic #2: Analysis of general aspects

< Way forward >:

Open issues for general aspects are captured in section 2 for Topic #2 in summary R4-2220061.

Topic #3: L3 measurement specifically related

<Agreement >:

  - The following L3 related requirements enhancements should be studied and specified if necessary

    - L3 measurements in RRC Connected mode

    - Note: other enhancements are deprioritized

###### 2.4.1.2.3.2	WF R4-2220743

Overall handling of L1 measurements

Agreement: Discussion on Rx beam sweeping scaling factor, simultaneous reception of DL data and RS, and other multi-Rx related topics can be done jointly for L1-RSRP, RLM, L1-SINR and CBD/BFD.

Note: issues specific to each of the above measurements can be discussed separately.

Group based reporting

Companies are encouraged to bring analysis on what requirements are needed for group based reporting to enable simultaneous reception.

Beam sweeping factor reduction

The following options are kept for further consideration:

  - Option 3: N should be reduced under the assumption that UE has some coverage overlap between different panels, how to define the conditions is FFS

  - Option 4: N should be reduced with some other conditions/restrictions, exact conditions/constraints are still FFS, should be discussed what needs to be studied further

  - Option 6: Do not change for L1

Companies should bring more analysis on the feasibility considering the following aspects:

- relationship with enabling multi-Rx (i.e. whether this enhancement is necessary to enable multi-Rx or not)

- dependency on UE implementation

- need for UE capability and whether/how this can be used by the network

- feasibility of requirement definition and whether it can be enforced

- reduction in UE flexibility related to beam sweeping

- quantifiable performance improvements (e.g. beam management/mobility improvements and impact on system performance)

- other aspects are not precluded.

Simultaneous reception of measurement RS and data

Companies are encouraged to bring further analysis on the feasibility of simultaneous reception of measurement RS and data considering the following aspects:

- concrete examples of scenarios under which simultaneous reception is possible (e.g. which measurement RS in which measurement occasion can be received simultaneously with control/data channels)

- how is the network aware of which signals/channels can be simultaneously received by the UE

- conditions under which these signals can be received simultaneously (e.g. singals grouped together in a group report)

- need for additional signaling/capability

- throughput gain

- specification impact and relationship with enabling multi-Rx (i.e. whether this enhancement is necessary to enable multi-Rx or not)

- other aspects are not precluded.

Measurement restrictions

Companies are encouraged to bring further analysis on the feasibility of making changes to the measurement restrictions currently defined:

- concrete examples of scenarios under which restrictions are not needed

- how is the network aware that UE does not need these restrictions and how it will take advantage of it

- conditions under which the restrictions can be relaxed/not needed at all

- need for additional signaling/capability

- system level performance gain (e.g. improved mobility, improved throughput, etc)

- specification impact

- relationship with enabling multi-Rx (i.e. whether this enhancement is necessary to enable multi-Rx or not)

- other aspects are not precluded.

Scheduling restrictions

Companies are encouraged to bring further analysis on the feasibility of making changes to the measurement restrictions currently defined:

- concrete examples of scenarios under which restrictions are not needed

- how is the network aware that UE does not need these restrictions and how it will take advantage of it

- conditions under which the restrictions can be relaxed/not needed at all

- need for additional signaling/capability

- system level performance gain (e.g. improved throughput)

- specification impact

- relationship with enabling multi-Rx (i.e. whether this enhancement is necessary to enable multi-Rx or not)

- other aspects are not precluded.

Sharing Factor Reduction

Companies are encouraged to bring further analysis on the feasibility of reducing the sharing factor P:

- concrete examples of scenarios under which the sharing factor can be reduced

- whether network should be that UE has a reduced sharing factor and whether/how it will take advantage of it

- conditions under which the reduction is possible

- need for additional signaling/capability

- system level performance gain (e.g. improved mobility, improved throughput)

- specification impact

- relationship with enabling multi-Rx (i.e. whether this enhancement is necessary to enable multi-Rx or not)

- other aspects are not precluded.

###### 2.4.1.2.3.3	WF R4-2220402

Sub-topic 1-1: General scope

Issue 1-1-1: Regarding group-based reporting and simultaneous reception with different QCL type-D

- Agreement

  - group based reporting is one of the means to enable simultaneous reception

- Candidate proposals for further study

  - Option 1: Simultaneous reception is not necessarily based on group-based reception

  - Option 2: TCI switch requirements are to specify dual TCI state switching requirements for multi-Rx (not necessary to limit to only simultaneous reception)

Issue 1-1-2: As per RAN1 agreements, group-based reporting is supported for single DCI only. If group-based reporting is a prerequisite, this AI can focus on single DCI case only.

- Candidate proposals for further study

  - Companies to check this RAN1 agreement and more discussion is needed during next meeting.

Issue 1-1-3: Definition of Dual TCI state switch

  - Dual TCI state switching refers to the case when two active TCI states with QCL type D for simultaneous reception of PDCCH/PDSCH are involved before and/or after TCI switching command.

  - In other words, following are considered as dual TCI state switching to discuss the requirements

    - Single DCI to Dual TCI

    - Dual TCI to Dual TCI

    - FFS: Dual TCI to Single TCI

Sub-topic 1-2: sDCI based TCI state switching requirements.

Issue 1-2-1: TCI state switch indication method for PDCCH and PDSCH

  - PDCCH non-SFN

    - Only single PDCCH is supported. So Single MAC CE should be used

  - FFS: PDCCH SFN

    - One MAC CE is used to indicate both TCI states

  - PDSCH

    - For both SFN and non-SFN, only one DCI code point is indicated using single DCI and the code point may indicate one or two TCI states

Sub-topic 1-4: known and unknown conditions.

Issue 1-4-1: Known and unknown states combination

- Dual TCI state switching requirements are defined for simultaneous reception. It is FFS whether we need to define any new requirements for non-simultaneous reception. The dual TCI states known, or unknown are FFS.

  - Option A: known and known

  - Option B: Known and unknown

  - Option C: unknown and unknown

For all other issues listed in moderator summary, there was no consensus reached in this meeting. Companies are encouraged to bring further analysis to next meeting

#### 2.4.1.3	UE Performance

##### 2.4.1.3.1	From RAN4#104-e: WF R4-2214344

- UE RRM performance part work plan

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#106 | Feb. 2023 | Initial discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Initial discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused |  |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused Initial discussion on list of test cases |  |
| RAN4#107 | May 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Initial agreement on list of test cases |
| RAN4#108 | Aug. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Work split on the Conclude the list of test cases Work split on the Draft CRs for performance requirements and test cases |
| RAN4#108-Bis | Oct. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Initial discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#109 | Nov. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Continue discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#110 | Feb. 2024 | Continue discussion on the remaining issues for performance requirements and test cases | Conclude all the performance requirements and test cases Endorse all the Draft CRs for the performance part of the WI |

#### 2.4.2	Remaining Open issues

#### 2.4.2.1	Objective of Core part WI

Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

    - The legacy spherical coverage requirement for reception from a single direction will be kept

    - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

  - Enhanced RRM requirements:

    - The following requirements should be studied and specified if necessary:

      - L1-RSRP measurement delay

      - L3 measurement delay (both cell detection delay and measurement period can be considered)

        - The starting point is the enhancements related to L1-RSRP measurement enhancements

      - RLM and BFD/CBD requirements

      - Scheduling/measurement restrictions

      - TCI state switching delay with dual TCI

      - Receive timing difference between different directions (different QCL Type D RSs)

NOTE:

- The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

#### 2.4.2.2	Objective of Performance part WI

- Introduce necessary requirement(s) for enhanced FR2 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - RRM Performance requirements and test cases corresponding to the new core requirements

  - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused

  - UE demodulation requirements:

    - Specify performance requirements for enhanced FR2 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier.

      - Demod requirements on up to 4 DL MIMO layers

      - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook

    - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined

NOTE:

− For the dual-TCI demodulation requirement case, focus on UEs supporting “simultaneousReceptionDiffTypeD-r16” UE capability, and for the 2- or 4-layer downlink MIMO reception, focus on UEs supporting the basic mTRP CSI reporting capability (FG 23-7-1/1b of NR FeMIMO).

## 2.5	RAN5

#### Void

## 2.6	RAN6

#### Void

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

## 4.1	Discussion Tdocs from RAN4#104-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2211642 | Discussion on Rel-18 multi Rx pannel RRM | CATT |
| R4-2211768 | Views on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2211814 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2211815 | Enhanced spherical coverage requirements for multi-rx chains UEs | Nokia, Nokia Shanghai Bell |
| R4-2211882 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211883 | RRM requirements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2211940 | Discussion on RRM requirements for simultaneous DL reception from different directions | CMCC |
| R4-2211978 | On multi-RX chain | Xiaomi |
| R4-2211993 | Requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2212062 | Initial discussion on RRM requirements for simultaneous DL reception from different directions | OPPO |
| R4-2212180 | Impacts on RRM to support FR2 multi-Rx chain DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2212185 | Discussion on UE RF requirements for simultaneous DL reception with up to 4 layer MIMO | LG Electronics |
| R4-2212219 | Discussion on RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2212332 | System assumptions for multi-Rx chain DL reception | Qualcomm Incorporated |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated, vivo |
| R4-2212466 | Discussion on RRM requirements for simultaneous DL reception from different directions | Samsung |
| R4-2212512 | Discussion on simultaneous DL reception from different directions | MediaTek Inc. |
| R4-2212593 | Discussion on simultaneous multi-Rx chain DL reception | Xiaomi |
| R4-2212688 | Discussion on RRM requirements for FR2 multi-Rx chain reception | Nokia, Nokia Shanghai Bell |
| R4-2212806 | General analysis on Multi-Rx requirements | vivo |
| R4-2213053 | Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception | vivo, Qualcomm |
| R4-2213054 | Discussion on RRM requirements for multi-chain DL reception | vivo |
| R4-2213289 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213495 | Discussion on RRM impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213568 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2213737 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2213872 | Discussion on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2213957 | Discussion on general RRM issues for simultaneous DL reception from different directions | Ericsson |
| R4-2214245 | Email Discussion Summary for [104-e][134] FR2_multiRx_UERF | Moderator (Qualcomm) |
| R4-2214282 | Email Discussion Summary for [104-e][233] FR2_multiRx_RRM | Moderator (Vivo) |

## 4.2	Discussion Tdocs from RAN4#104-Bis-e:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2215581 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215621 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215781 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2216127 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2216353 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2216786 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2216253 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2216589 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216875 | On Defining Coverage Requirements for Multi-Rx Chain Downlink Reception | Lenovo |
| R4-2215579 | Proposals on consideration of UE architecture for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2215620 | UE implementation assumptions for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215701 | System assumption and UE assumption for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2215778 | Discussion for FR2 multi-Rx chain DL reception | Murata Manufacturing Co Ltd. |
| R4-2216125 | Discussion on UE implementation assumption of multi-Rx DL reception | vivo |
| R4-2216352 | Discussion on UE architecture to support simultaneous DL reception | Xiaomi |
| R4-2216445 | Considerations on the UE assumptions for FR2 multi-Rx chain DL reception | OPPO |
| R4-2217784 | Email discussion summary for [104-bis-e][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2215360 | Discussion on FR2 multi Rx chain RRM impacts and general aspects | Intel Corporation |
| R4-2215361 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2215362 | Discussion on RRM impacts for TCI state switching based on FR2 multi Rx chain | Intel Corporation |
| R4-2215462 | on the multi-RX chain general aspects | Xiaomi |
| R4-2215463 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2215464 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2215465 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2215622 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215623 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215624 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2215710 | Discussions on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2215720 | Discussion on general aspects for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215721 | Discussion on L1 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215722 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2215759 | Discussion on simultaneous DL reception from different directions for general issues | MediaTek Inc. |
| R4-2215760 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2215761 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2215762 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2215803 | Discussion on general aspects of RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215804 | Discussion on L3 measurement related RRM for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215805 | Discussion on L1 measurement related to RRM for simultaneous DL reception from different | LG Electronics Inc. |
| R4-2215806 | Discussion on TCI state switching for simultaneous DL reception from different directions | LG Electronics Inc. |
| R4-2215812 | Discussion on general requirements for FR2_multiRX_DL | OPPO |
| R4-2215813 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2215814 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2215815 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2215867 | Further analysis on RRM impacts and general aspects | vivo |
| R4-2215868 | On L3 measurement for multi-Rx chain | vivo |
| R4-2215869 | On L1 measurement for multi-Rx chain | vivo |
| R4-2215870 | On TCI state switching for multi-Rx chain | vivo |
| R4-2216277 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2216285 | Discussion on RRM general impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216286 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216287 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2216474 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216475 | Discussion on L1 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216476 | Discussion on L3 part RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216477 | Discussion on TCI state related RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2216578 | General considerations on RRM requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2216579 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216580 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216581 | Discussion on RRM TCI State Switching for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2216713 | Further Analysis of RRM requirement impacts for simultaneous DL reception from different directions | Samsung |
| R4-2216824 | Discussion on scenarios for simultaneous DL reception from different directions | Ericsson |
| R4-2216825 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2216826 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2216827 | Discussion on active TCI state requirements | Ericsson |
| R4-2216866 | Impacts on RRM to support FR2 multi-Rx chain based 4 layer DL reception from multi-TRP | Qualcomm Incorporated |
| R4-2217246 | WF on RRM impacts and general aspects for multi-Rx | Vivo |
| R4-2217248 | WF on TCI state switching for multi-RX chain DL reception | Ericsson |
| R4-2217587 | WF on L1 measurements, beam sweeping factors and simultaneous reception | Qualcomm |

## 4.3	Discussion Tdocs from RAN4#105:

| Tdoc | Title | Company |
| --- | --- | --- |
| R4-2218042 | On UE RF requirements for 2AoA FR2 DL MIMO | Qualcomm Incorporated |
| R4-2218043 | On specifying requirements for mDCI UEs | Qualcomm Incorporated |
| R4-2218165 | System parameter and UE assumption for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218166 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218329 | Proposal on UE RF System parameter assumption, UE architecture and conditions of UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218330 | Proposal on UE RF requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell |
| R4-2218528 | Discussion on UE RF requirements for simultaneous DL reception | LG Electronics |
| R4-2218556 | System and UE assumptions for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218557 | RF requirements for FR2 simultaneous DL reception from different directions | Samsung |
| R4-2218755 | Further views on multi-Rx chain DL reception in FR2 | Sony, Ericsson |
| R4-2218873 | Further consideration on system parameter of multi-Rx DL reception | vivo |
| R4-2218874 | Discussion on RF requirement for multi-Rx DL reception | vivo |
| R4-2219124 | Discussion on UE RF system assumption supporting simultaneous DL reception | Xiaomi |
| R4-2219125 | Discussion on UE RF requirements supporting simultaneous DL reception | Xiaomi |
| R4-2219192 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2219193 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2219497 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219600 | Consideration on small AoA angular separation | OPPO |
| R4-2219852 | System Parameter Assumptions for Multi AoA Rx Testing | Keysight Technologies UK Ltd |
| R4-2219868 | Discussion on RF test methods for UEs with multi-Rx | ROHDE & SCHWARZ |
| R4-2220017 | Complexity Reduction for Multi-Rx Multi-AOA Coverage Evaluation | Lenovo |
| R4-2220529 | Discussion on System parameter and UE architecture of UE RF requirements | ZTE Corporation |
| R4-2220530 | Discussion on UE RF requirements for simultaneous DL reception | ZTE Corporation |
| R4-2220111 | Topic summary for [105][132] FR2_multiRx_UERF_part1 | Moderator (Qualcomm) |
| R4-2220112 | Topic summary for [105][132] FR2_multiRx_UERF_part2 | Moderator (Apple) |
| R4-2218167 | General aspects for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218168 | On L3 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218169 | On L1 measurements for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218170 | TCI state switching for NR FR2 multi-Rx chain DL reception | Apple |
| R4-2218335 | Discussion on FR2 multi Rx chain scope and scenarios | Intel Corporation |
| R4-2218336 | Discussion on FR2 multi Rx chain general aspects | Intel Corporation |
| R4-2218337 | Discussion on RRM impacts for L1 measurement based on FR2 multi Rx chain | Intel Corporation |
| R4-2218393 | Discussion on scope and scenarios for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218394 | Discussion on L3 measurement for FR2 multi-Rx chain DL reception | CMCC |
| R4-2218408 | Analysis of scope and scenarios | Qualcomm Incorporated |
| R4-2218409 | Analysis of general aspects | Qualcomm Incorporated |
| R4-2218410 | L3 measurement specifically related | Qualcomm Incorporated |
| R4-2218586 | on the multi-RX chain scenarios | Xiaomi |
| R4-2218587 | on the multi-RX chain general aspects | Xiaomi |
| R4-2218588 | on the multi-RX chain L3 measurement | Xiaomi |
| R4-2218589 | on the multi-RX chain L1 measurement | Xiaomi |
| R4-2218590 | on the multi-RX chain TCI state switching | Xiaomi |
| R4-2218596 | Discussion on scope and scenario of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218597 | Discussion on general aspects of RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218598 | Discussion on L3 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218599 | Discussion on L1 measurement related for RRM for FR2 multi-Rx chains | LG Electronics Inc. |
| R4-2218738 | Discussion on simultaneous DL reception from different directions for scope and scenarios | MediaTek Inc. |
| R4-2218739 | Discussion on simultaneous DL reception from different directions for general aspects | MediaTek Inc. |
| R4-2218740 | Discussion on simultaneous DL reception from different directions for L3 measurement | MediaTek Inc. |
| R4-2218741 | Discussion on simultaneous DL reception from different directions for L1 measurement | MediaTek Inc. |
| R4-2218742 | Discussion on simultaneous DL reception from different directions for TCI state switching | MediaTek Inc. |
| R4-2218775 | On scope and scenarios for FR2 multi-Rx | vivo |
| R4-2218776 | On general aspects for FR2 multi-Rx | vivo |
| R4-2218777 | On L3 measurement for multi-Rx | vivo |
| R4-2218778 | On L1 measurement for multi-Rx | vivo |
| R4-2218779 | On TCI state switching for multi-Rx | vivo |
| R4-2218977 | On scope and scenarios for FR2_multiRX_DL | OPPO |
| R4-2218978 | On general requirements for FR2_multiRX_DL | OPPO |
| R4-2218979 | Discussion on L3 requirements for FR2_multiRX_DL | OPPO |
| R4-2218980 | Discussion on L1 requirements for FR2_multiRX_DL | OPPO |
| R4-2218981 | Discussion on TCI state switching for FR2_multiRX_DL | OPPO |
| R4-2219008 | Scope and scenarios for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219009 | General aspects discussions for FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219010 | Discussions for L3 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219011 | Discussions for L1 measurements on FR2 multi Rx chain DL reception | NTT DOCOMO, INC. |
| R4-2219242 | Discussion RRM requirements of TCI state switching for multi-Rx | Huawei, HiSilicon |
| R4-2219250 | Discussion on scope and scenarios for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219251 | Discussion on RRM general aspects for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219252 | Discussion on L3 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219253 | Discussion on L1 measurement impacts for R18 FR2 multi-Rx chain DL reception | Huawei, HiSilicon |
| R4-2219290 | Discussion on scope and general scenarios for FR2 multi-RX | Samsung |
| R4-2219291 | Analysis on general aspects for FR2 multi-RX | Samsung |
| R4-2219434 | Discussion on scope and scenarios for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219435 | Discussion on general aspects on RRM requirements for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219436 | Discussion on L3 measurement related aspects for simultaneous DL reception from different directions | ZTE Corporation |
| R4-2219739 | Discussion on scope and scenarios for the requirements for multi-RX RRM | Nokia, Nokia Shanghai Bell |
| R4-2219740 | Discussion on general aspects for multi Rx DL | Nokia, Nokia Shanghai Bell |
| R4-2219741 | Discussion on RRM L3 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219742 | Discussion on RRM L1 enhancements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219743 | Discussion on TCI state switching requirements for multi Rx DL in FR2 | Nokia, Nokia Shanghai Bell |
| R4-2219946 | Discussion on scope and UE capabilities | Ericsson |
| R4-2219947 | Discussion on general aspects | Ericsson |
| R4-2219948 | Discussion on L3 measurements and procedures | Ericsson |
| R4-2219949 | Discussion on L1 measurements and procedures | Ericsson |
| R4-2219950 | Discussion on active TCI state requirements | Ericsson |
| R4-2220370 | Ad-hoc meeting minutes: Rel-18 NR FR2 multi-Rx chain DL reception WI | Qualcomm |
| R4-2220402 | WF on NR FR2 multi-Rx chain DL reception – Part 3 | Ericsson |
| R4-2220742 | WF on NR FR2 multi-Rx chain DL reception – Part 1 | Vivo |
| R4-2220743 | WF on NR FR2 multi-Rx chain DL reception – Part 2 | Qualcomm |