3GPP TSG RAN Meeting #98-e								RP-223171

Electronic Meeting, December 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:45% RAN4: 45% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#104-bis-e meeting

The following tdocs have been agreed in RAN4#104-bis-e meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2215599 | Updated Work plan for R18 eFeRRM | Apple |
| R4-2217249 | WF on R18 eFeRRM - FR2 SCell activation enhancement | Apple |
| R4-2217250 | WF on RRM Core requirements for FR1-FR1 NR-DC | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 2-1-1: Scenarios/status/categories for FR2 SCell activation enhancement

Agreement:

- RAN4 to use option 4 as baseline to discuss the enhancement, and further unknown SCell scenario categorization can be discussed case-by-case in enhancement issues to determine whether and how to use such new category for enhancement.

- Option 4 (last meeting agreement): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

  - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

  - The extension of the enhancement solutions to FR1 can also be discussed.

- FFS on option 1/2/3 and other options are not precluded.

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

Agreement：

- In 1st phase, RAN4 not to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI.

- In 2nd phase, if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case, RAN4 to discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI.

Issue 3-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement:

- If L1-RSRP measurement of FR2 unknown SCell activation is not skipped, RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP

  - Details are FFS

Issue 3-1-2: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

Agreements:

- If L1-RSRP measurement is needed, the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown Scell activation requirement.

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation?

Agreements:

- RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

Agreement:

- L1-RSRP measurement is performed in non-DRX mode even DRX is configured.

- FFS on whether prioritization needs to be changed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation

Issue 3-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation

Agreement:

- A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index.

RRM core requirements for FR1-FR1 NR-DC

### Issue 1-1-1: Applicability of the existing requirement for FR1+FR1 NR-DC

<Agreement on Friday GTW>:

For the features/requirements that have already been complied with FR1+FR1 NR-DC

- keep current applicability rules as they are.

For requirements explicitly listed in the WID:

- If applicability rules are specified for FR1+FR2 NR-DC requirements in the existing spec, applicability rules for FR1+FR1 NR-DC requirements are also specified;

### Issue 1-2-1: Number of serving carriers

<Agreement>:

For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to [6] NR DL CCs in total, with [1] UL in PCell, [1] UL in PSCell and [1] UL in SCell(s).

- These values can be further updated if R18 RF spec introduces greater CC number.

### Issue 1-3-1: Interruption for PSCell addition/release and conditional PSCell addition delay

<Agreement>:

For FR1+FR1 NR-DC case

- existing interruption length requirement for NR PSCell addition/release can be reused.

- existing conditional PSCell addition requirement can be reused (section 8.9A TS38.133)

### Issue 1-3-3: Requirement applicability

<Agreement>:

For FR1+FR1 NR-DC, the introduction part of interruption requirement in section 8.2.4 of TS38.133 can be revised as:

-	up to [1] SCell in FR1 and up to [7] SCell(s) in FR2 are configured, deconfigured, activated or deactivated when operating in FR1-FR2 NR-DC, or,

-	up to [4] SCell(s) in FR1 are configured, deconfigured, activated or deactivated when operating in FR1-FR1 NR-DC

Note: The SCell number could be updated according to RF progress in R18.

### Issue 1-4-4: Applicability requirement for scheduling availability

<Agreement>:

- For applicability requirement (clause 3.6.9 for scheduling availability), current spec can be reused and no spec impact for FR1-FR1 NR-DC

### Issue 1-6-1: CSSF outside gap

<Agreement>:

CSSF outside gap for FR1+FR1 NR-DC is:

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR1 PSCC | CSSFoutside_gap,i for FR2 PSCC | CSSFoutside_gap,i for FR2 SCC whereeighborr cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 + FR1 NR-DC (FR1 pCell and FR1 PScell) | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+2xNSCC_CSIRS) | 2x(1+ NPSCC_CSIRS) Note 2 | N/A | N/A | 2x(NSCC_SSB +Y+2x NSCC_CSIRS ) |

### Issue 1-7-1: Tprocessing in PCell HO and PSCell change

<Agreement>:

FR1-FR2 NR-DC handover to FR1-FR1 NR-DC with PScell shall be the same requirement as FR1-FR1 NR-DC handover to FR1-FR2.

| Scenario for HO with PSCell | Tprocessing (ms) |
| --- | --- |
| Case 1: FR1-FR1 NR-DC to FR1-FR1 NR-DC | 25 and 30 ms for parallel and sequential case, respectively |
| Case 2: FR1-FR2 NR-DC to FR1-FR1 NR-DC | 45 and 50 ms for parallel and sequential case, respectively |
| Case 3: FR1-FR1 NR-DC to FR1-FR2 NR-DC | 45 and 50 ms for parallel and sequential case, respectively |

### Issue 1-8-1: RACH based PSCell activation for FR1+FR1 NR-DC

<Agreement>:

For SCG activation delay requirement in FR1+FR1 NR-DC, the following changes on existing requirement shall be added,

- for RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

### Issue 1-8-2: Known condition for PSCell activation for FR1+FR1 NR-DC

<Agreement>:

Existing cell known condition for FR2 in SCG activation requirements can be applied for FR1 as well

RAN4#105 meeting

The following tdocs have been agreed in RAN4#105 meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2220440 | WF on Even Further RRM enhancement for NR and MR-DC – Part 1 | Apple |
| R4-2220736 | WF on Even Further RRM enhancement for NR and MR-DC – Part 2 | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 1-1-1: new condition/classification for unknown SCell

Agreement

- FFS whether to define new sub-states for known/unknown state for FR2 SCell

- Discuss the following sub-cases to reduce FR2 SCell activation delay for unknown case

  - UE reports [a valid L3 measurement result] after SCell activation command

  - UE does not report [a valid L3 measurement result] after SCell activation command

  - Note: the categorization above does not limit potential enhancements

Issue 1-1-2: new measurement status indication or report for FR2 SCell activation enhancement

Agreement:

- for [unknown] FR2 SCell activation enhancement, NW can trigger/configure UE sends L3 measurement report with SSB index to network after SCell activation command

  - FFS on how and when to trigger/configure/report such L3 measurement results

  - FFS on necessity of L3 measurement reporting if UE has no valid measurement results

  - FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported.

  - If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available is FFS.

  - The status of [unknown] may be revised after concluding the issue 1-1-1.

Issue 1-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement:

- Specify beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation as UE capability

  - Conditions to apply beam sweeping factor enhancement and details of UE capabilities are FFS

  - Detail of solution are FFS

  - Note: if feasible conditions are not identified, then no enhancement will be introduced

Issue 1-4-1(common issue for multiple topics): whether to reuse timing and Rx beam for target unknown FR2 SCell based on QCLed type C/D information from an inter-band active serving cell

Agreement:

- For FR2 SCell activation enhancement

  - RAN4 to not consider reusing timing and Rx beam to target unknown FR2 SCell based on QCLed type C/D information from an inter-band active serving cell

- For FR1 SCell activation enhancement

  - RAN4 to discuss the feasibility of reusing timing information to target unknown FR1 SCell from an FR1 inter-band active serving cell

    - FFS the feasibility of configuring QCL type C information from FR1 inter-band active serving cell

Issue 2-4-1: enhancement based on A-TRS which is QCLed typeC/D with RS of inter-band active serving cell

- Agreement

  - As per conclusion of issue 1-4-1,

    - For FR2 unknown SCell activation enhancement, not to consider enhancement based on A-TRS which is QCLed typeC/D with RS of inter-band active serving cell.

    - For FR1 unknown SCell activation enhancement, FFS on enhancement based on A-TRS which is QCLed typeC with RS of inter-band active serving cell.

Issue 3-1-4: Applying the timing/beam information of active serving cell on FR2 inter-band

- Agreement:

  - As per conclusion of issue 1-4-1, not to consider applying the timing/beam information of active serving cell on the target FR2 inter-band SCell during the FR2 unknown SCell activation enhancement.

RRM core requirements for FR1-FR1 NR-DC

Issue 1-1: Applicability of the existing requirement for FR1+FR1 NR-DC

Agreement:

- For R16/17 and later requirement not listed in the WID,

  - not applicable to FR1-FR1 NR-DC, i.e., no UE requirement applies. Any extension of scope needs to be discussed in RAN-P.

- For R15 requirement not listed in the WID,

- not precluded in FR1-FR1 NR-DC and subject to WI stage discussion.

Issue 1-2: UE preparation time for FR1-FR1 NR-DC conditional Pscell additional delay

Agreement:

- TUE_preparation for FR1-FR1 NR-DC could keep the legacy value (10ms)

Issue 1-3-1: Scheduling availability for RLM/BFD/CBD/L1-RSRP measurements

Agreement:

- For RLM/BFD/CBD/L1-RSRP when FR1+FR1 NR-DC is performed, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to RLM/BFD/CBD/L1-RSRP performed on FR1 serving PCell or PSCell in different bands

Issue 1-3-3: Scheduling availability for intra-frequency measurement without MG

Agreement:

- The existing scheduling availability requirement for FR1 inter-band CA can be reused

- No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

- FFS whether to explicitly update the spec for FR1-FR1 NR-DC

- For UE who does not support simultaneousRxTxInterBandCA, FFS scheduling availability requirement of UE performing measurements on symbols colliding with PUSCH/PUCCH/SRS on FR1 for FR1-FR1 NR-DC scenario.

Issue 1-3-4: Scheduling availability for inter-frequency measurement without MG

Agreement:

- No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

- For UE who supports simultaneousRxDataSSB-DiffNumerology-Inter-r16, no need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Issue 1-4-2: Measurement restriction requirements

Agreement:

- There is no measurement restriction for FR1-FR1 NR-DC scenario.

Issue 1-5: CSSF within gap

Agreement:

- For CSSF within gap, update the grouping rule for FR1+FR1 NR-DC:

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurements on all positioning frequency layers is zero and the UE is configured with per UE gaps:

- 	FR1 intrafrequency measurement objects of MCG belong to group A

-      FR1 intrafrequency measurement objects of SCG belong to group B

Issue 1-6: Additional uncertainty delay

Agreement:

- Not define HO with PSCell requirements for collision between PCell RACH and PSCell RACH occasion in FR1+FR1 NR-DC

#### 2.4.2	Remaining Open issues

As summarized in approved way forward R4-2220440 and R4-2220736 (open options in the way forward).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-bis-e meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2215355 | Discussion on FR1-FR1 NR-DC | Intel Corporation |
| R4-2215356 | Discussion on L3 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2215357 | Discussion on L1 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2215456 | Discussion on FR2 SCell activation delay reduction | Xiaomi |
| R4-2215466 | Discussion on RRM core requirements for FR1-FR1 NR-DC. | Xiaomi |
| R4-2215530 | Discussion on A-TRS based unknown SCell activation | China Telecom |
| R4-2215531 | Discussion on SCell activation without SSB in inter-band scenario | China Telecom |
| R4-2215599 | Updated Work plan for R18 eFeRRM | Apple |
| R4-2215600 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2215601 | On L1 part enhancement for FR2 SCell activation | Apple |
| R4-2215602 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2215717 | Discussion on RRM requirements for FR1-FR1 NR-DC | CMCC |
| R4-2215718 | Discussion on L1 part enhancement for FR2 SCell activation | CMCC |
| R4-2215719 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2215763 | Discussion on R18 RRM for FR1-FR1 NR-DC | MediaTek Inc. |
| R4-2215785 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2215786 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2215787 | Other enhancements on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2215801 | Discussion on FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2215807 | Discussions on FR2 SCell Activation delay requirements | NTT DOCOMO, INC. |
| R4-2215809 | Discussion on L3 part enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2215810 | Discussion on L1 part enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2215811 | Discussion on RRM requirements for FR1-FR1 NR-DC | OPPO |
| R4-2215837 | discussion on FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2215864 | Further discussion on FR1-FR1 NR-DC requirement | vivo |
| R4-2215865 | Discussion on L3 part enhancement for FR2 SCell activation | vivo |
| R4-2215866 | Discussion on L1 part enhancement for FR2 SCell activation | vivo |
| R4-2216272 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2216273 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2216274 | Discussion on FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2216275 | Discussion RRM requirements for FR1-FR1 NR-DC | Huawei, HiSilicon |
| R4-2216341 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Ericsson |
| R4-2216478 | Discussion on other aspects of RRM requirements enhancement for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2216479 | Discussion on the L1 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2216480 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2216744 | Discussion on RRM requirements for FR2 unknown Scell activation delay reduction | Qualcomm Incorporated |
| R4-2216745 | Discussion on RRM requirements for remaining issues about FR1+FR1 NR-DC | Qualcomm Incorporated |
| R4-2216758 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2216759 | Discussion on L1 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2216760 | Discussion on other potential enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2216828 | Discussion on L3 part enhancement for FR2 SCell activation | Ericsson |
| R4-2216829 | Discussion on L1 part enhancement for FR2 SCell activation | Ericsson |
| R4-2216830 | Discussion on Other potential enhancement for FR2 SCell activation | Ericsson |
| R4-2216925 | Email discussion summary for 1[04-bis-e][214] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2216926 | Email discussion summary for 1[04-bis-e][215] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2217147 | Email discussion summary for 1[04-bis-e][214] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2217148 | Email discussion summary for 1[04-bis-e][215] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2217249 | WF on R18 eFeRRM - FR2 SCell activation enhancement | Apple |
| R4-2217250 | WF on RRM Core requirements for FR1-FR1 NR-DC | OPPO |

RAN4 #105 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2218140 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2218141 | On L1 part enhancement for FR2 SCell activation | Apple |
| R4-2218142 | On other potential enhancement for FR2 SCell activation | Apple |
| R4-2218143 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2218332 | Discussion on FR1-FR1 NR-DC | Intel Corporation |
| R4-2218333 | Discussion on L3 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2218334 | Discussion on L1 part enhancement for FR2 SCell activation | Intel Corporation |
| R4-2218395 | Discussion on L1 part enhancement for FR2 SCell activation | CMCC |
| R4-2218396 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2218540 | Remaining issues on RRM requirements about FR2 unknown Scell activation enhancements L3 part | Qualcomm Incorporated |
| R4-2218541 | Remaining issues on RRM requirements about FR2 unknown Scell activation enhancements L1 part | Qualcomm Incorporated |
| R4-2218542 | Remaining issues on RRM requirements about FR1+FR1 NR-DC | Qualcomm Incorporated |
| R4-2218545 | Discussion on L3 part enhancement for FR2 SCell activation | China Telecom |
| R4-2218546 | Discussion on L1 part enhancement for FR2 SCell activation | China Telecom |
| R4-2218547 | Discussion on other potential enhancement for FR2 SCell activation | China Telecom |
| R4-2218565 | Discussion on L3 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2218566 | Discussion on L1 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2218577 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2218593 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2218594 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2218678 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2218679 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2218680 | Other enhancements on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2218686 | discussion on FR1+FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2218743 | Discussion on R18 RRM for FR1-FR1 NR-DC | MediaTek Inc. |
| R4-2218797 | Further discussion on FR1-FR1 NR-DC requirement | vivo |
| R4-2218982 | Discussion on L3 part enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2218983 | Discussion on L1 part enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2218984 | Discussion on RRM requirements for FR1-FR1 NR-DC | OPPO |
| R4-2219034 | Discussions on SCell activation requirement with different PCI from serving cell | NTT DOCOMO, INC. |
| R4-2219177 | Discussion on L3 aspects of FR2 SCell activation enhancements | vivo |
| R4-2219178 | Discussion on L1 aspects of FR2 SCell activation enhancements | vivo |
| R4-2219233 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2219234 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2219235 | Discussion on FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2219236 | Discussion RRM requirements for FR1-FR1 NR-DC | Huawei, HiSilicon |
| R4-2219438 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2219439 | Discussion on the L1 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2219440 | Discussion on other aspects of RRM requirements enhancement for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2219648 | Discussion on define RRM requirements for FR1-FR1 NR-DC scenarios | Ericsson |
| R4-2219919 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2219920 | Discussion on L1 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2219921 | Discussion on other potential enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2219951 | Discussion on L3 part enhancement for FR2 SCell activation | Ericsson |
| R4-2219952 | Discussion on L1 part enhancement for FR2 SCell activation | Ericsson |
| R4-2219953 | Discussion on other potential enhancement for FR2 SCell activation | Ericsson |
| R4-2220064 | Topic summary for [105][218] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2220065 | Topic summary for [105][219] NR_RRM_enh3_part2 | Moderator (OPPO) |
| R4-2220411 | Ad-hoc meeting minutes: Rel-18 Even Further RRM enhancement for NR and MR-DC WI | Apple |
| R4-2220440 | WF on Even Further RRM enhancement for NR and MR-DC – Part 1 | Apple |
| R4-2220442 | WF on Even Further RRM enhancement for NR and MR-DC – Part 2 | OPPO |
| R4-2220736 | WF on Even Further RRM enhancement for NR and MR-DC – Part 2 | OPPO |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template