3GPP TSG-RAN4 Meeting #105		R4-2220737

# Toulouse, France, November 14 - November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2715 | rev | 2 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Correction to measurement core requirements for PDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Nokia, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR is further revision based on agreed CR R4-2217238. Requirements need to be specified when PDC RS resources overlap with Type 1A/1B/2 PPW. Requirements need to be specified when PDC RS resources overlap with measurement gaps and SMTC. Requirements for DRX were agreed to be introudeced. It was agreed in RAN4#103-e to define scheduling and measurement restriction for PDC measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added requirements when PDC RS resources overlap with Type 1A/1B/2 PPW. - Added requirements when PDC RS resources overlap with measurement gaps and SMTC. - Added requirements for DRX. - Added scheduling and measurement restriction for PDC measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement core requirements for PDC are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised for R4-2218784 |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 9.12	Measurement for Propagation Delay Compensation

### 9.12.1	Introduction

The requirements in this clause are applicable for UE capable of RTT-based propagation delay compensation based on PRS/TRS and SRS, where the UE is configured with measObjectRxTxDiff in MeasObjectToAddModList defined in TS38.331 [2].

### 9.12.2	Requirements Applicability

The requirements in clause 9.12 apply for periodic triggered UE Rx-Tx time difference measurements, provided:

-	If UE Rx-Tx time difference measurement is based on PRS, the related side conditions given in clause 10.1.X.1 are met for a corresponding band.

-	If UE Rx-Tx time difference measurement is based on TRS, the related side conditions given in clause 10.1.X.2 are met for a corresponding band.

-	SRS is configured on at least one of the PCell.

### 9.12.3	Measurement Capability

If UE Rx-Tx time difference measurement is based on PRS, the capability is as indicated by the UE in rtt-BasedPDC-PRS-r17 in TS38.331 [2].

If UE Rx-Tx time difference measurement is based on TRS, the capability is as indicated by the UE in rtt-BasedPDC-CSI-RS-ForTracking-r17 in TS38.331 [2].

### 9.12.4	Measurement period requirements

#### 9.12.4.1	PRS Measurement Period

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*fmaxLCM(TPRS, TDRX)Teffect

Where:

NPRSslot is the maximum number of DL PRS resources configured in a slot,

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumberPRS-ResourceProcessedPerSlot-r17 as specified in TS38.331 [2],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

Teffect

Teffect=TPRS

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable, when Navailable>0

Where,

-	For a window W of duration LCM(TPRS, MGRP, TSMTC), where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, starting at the beginning of any PRS resource occasion:

-	Ntotal is the total number of PRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of PRS resource occasions that are not overlapped with:

-	For FR1: any measurement gap occasions within the window W

-	For FR2: any measurement gap occasions nor any SMTC occasions within the window W.

No requirements shall apply if Navailable=0.

Editor Note: FFS requirements if concurrent measurement gaps are configured.

Klayer3

Klayer3=11-TSSBMGRP

Klayer3

Klayer3=11-TSSBTSMTCperiod

Klayer3

Klayer3=11-TSSBMGRP - TSSBTSMTCperiod

Klayer3=Ksharing factor1-TSSBMGRP

Klayer3=Psharing factor1-TSSBMGRP

Klayer3=Psharing factor1-TSSBMRGP

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the PRS resources in the resource set and for each resource one RS has QCL-TypeD.

If PRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

#### 9.12.4.2	TRS Measurement Period

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*fmaxLCM(TTRS, TDRX)TTRS

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable when Navailable>0

Where,

-	For a window W of duration LCM(TTRS, MGRP, TSMTC), where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, starting at the beginning of any TRS resource occasion:

-	Ntotal is the total number of TRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of TRS resource occasions that are not overlapped with

-	For FR1: any measurement gap occasions within the window W

-	For FR2: any measurement gap occasions nor any SMTC occasions within the window W.

No requirements shall apply if Navailable=0.

Editor Note: FFS requirements if concurrent measurement gaps are configured.

Klayer3

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

For FR2, the requirements apply if QCL information is provided for all the TRS resources in the resource set and for each resource one RS has QCL-TypeD.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

### 9.12.5	Measurement Reporting Requirements

The UE shall report UE Rx-Tx time difference measurement results if UE supports gNB-SideRTT-BasedPDC-r17.

The measurement reporting delay excludes the delay uncertainty caused by inserting the measurement report into the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in clause 10.1.25.3, with k = 5.

The UE Rx-Tx time difference measurement accuracy shall be fulfilled according to the accuracy requirements specified in clause 10.1.X.1 and 10.1.X.2 for PRS and TRS, respectively.

### 9.12.6	Scheduling availability during measurement for Propagation Delay Compensation

During measurement for Propagation Delay Compensation in an FR2 cell,

-	For the case where the PRS or TRS for PDC measurement is QCLed with active TCI state for PDCCH/PDSCH, there are no scheduling restrictions due to PDC measurement performed based on PRS or TRS.

-	Otherwise, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the PRS or TRS resource configured for PDC measurement.

### 9.12.7	Measurement restriction for measurement for Propagation Delay Compensation

During measurement for Propagation Delay Compensation in an FR2 cell, when the PRS or TRS for PDC measurement on one CC is in the same OFDM symbol as SSB or CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both PRS or TRS for PDC measurement and the other RS if they are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE. Longer measurement period for PRS or TRS for PDC measurement is expected.

# <End of change 1>