3GPP TSG-RAN Meeting #98-e	RP-223371

Electronic meeting, 12th – 16th December, 2022

Agenda item:	10.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	IoT NTN enhancement WI objective updates

Document for:	Discussion and Decision

# 1	Introduction

At RAN #96 the release 18 work item for IoT support of non-terrestrial networks was updated [1]. In this contribution we provide input to potential further updates related to support of long connections.

# 2 Discussion

## 2.1 Long connections

The current release 18 WI on IoT NTN includes a performance enhancement as follows [1]:

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

The objective was included in release 18 because the release 17 work was limited to “sporadic short transmission” in the WID [2] concerning GNSS measurements and during RAN1 discussions concerning both SIB acquisition and GNSS measurements:

Agreement: (RAN1 #106-e)

- Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.

- Common TA parameters if indicated and read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

Agreement: (RAN1 #106-e)

For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated.

Agreement (RAN1 #107-e)

Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity:

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

As stated in the release 18 objective above, the scope is to improve GNSS operations during long connection times.

Some disagreements in RAN1 may be attributed to the fact that companies disagree on the concept of “long connection”. Some may interpret “long connection” as a UE in connected mode exchanging several messages with the network, in short bursts of time. But in  our view, the definition of long connections apply, for example when the UE or eNB is utilizing many repetitions to ensure sufficient link budget for the UE or when a connection is long due to large amounts of data being transferred in RRC Connected mode. It is worth noting the use of repetitions in NB-IoT uplink can result in transmissions lasting 10 s and 40 s for NPRACH and NPUSCH, respectively, and thus the long connection may be longer than the NTN cell availability for a certain area.

Observation 1: Long connection might be interpreted as a connection that lasts for several transmissions in short burst or to long repetition-based transmissions – already provided in specification – used for coverage enhancement purposes. The analysis provided in this document is focused on the second case: enabling a feature that is already specified.

This differences on interpretation becomes evident in the FL summary from RAN1 [3]:

Moderator View: This issue was discussed in RAN1 109e, there was FL recommendation “Companies can further align understanding on whether and how re-acquiring the NTN SIB with ephemeris and common TA parameters could improve GNSS operations, and whether there is need and gains for further potential enhancements for re-acquiring NTN SIB in addition to Rel-17.”

In this contribution we explain why we see that such enhancements belong as part of the same issue. The release 17 work only addressed “short sporadic transmission” agreeing the UE can move to RRC Idle in case of a failure, e.g. caused by expiry of the GNSS validity duration or the satellite assistance information validity duration. For a long connection, this approach is problematic because the UE will most likely have to reconnect to continue the data transfer. This results in increased UE energy consumption, latency and signalling.

Observation 2: The WID scope on GNSS operations during long connection times has the intention to provide a more consistent framework for UL synchronization, avoiding unnecessary failures and energy consumption when long connection is needed for coverage enhancements purpose.

As noted above, the release 18 objective is limited to study and specify improved GNSS operations for the long connection. However, the initial intention cannot be fulfilled if other aspects related to UL synchronization in long connections are not addressed. There are additional issues which also need to be studied and potentially specified, because those issues may also occur during a long connection.

Observation 3: Support of long connections in IoT NTN release 18 may require a study of further issues in addition to addressing improved GNSS operations.

It is evident that the UE GNSS operation alone is not sufficient to maintain the UE UL synchronization, as evidenced by the following aspects:

- Satellite assistance information (SIB) acquisition during RRC Connected mode.

The release 17 specification defines the UE can attempt to reacquire the satellite assistance information, when the validity duration of the information has expired (T317). If the UE fails to do so within a second timer (T318), the UE will move to RRC Idle to reacquire the information.

The problem is that during a long connection, which utilizes many (uplink) repetitions, the UE may not be able to “tune away” and acquire the satellite assistance information and then return to continue the repetitions. Instead, the UE would most likely have to restart the data transfer.

- Transmission using different UL segment durations.

In release 17 it was agreed to split transmissions into segments, where the UE can adjust time and frequency pre-compensation. This is necessary to account for the drift that happens due to satellite movement. However, for the short connections of release 17 it was agreed to only use one segment duration per transmission. When using a long connection, it is evident that the minimum segment duration which fulfils the transmission accuracy requirement will vary, because the UE’s experienced drift depends on the relative position of the satellite. If the release 17 approach is applied to a long connection the network would have to use the worst-case assumption i.e., a short segment duration, which means some transmissions are interrupted too frequently, leading to extended transmissions and latency. For cases with repetition of one NPUSCH/NPRACH lasting as long as even 10s or 40s, the worst-case selection for segment and the corresponding gap may cause significant increase of UL pre-compensation per segment and resource waste (because of the gap).

Observation: Long Connections, defined as long-lasting repetitions due to coverage enhancements, are specified but not fully enabled as UL synchronization aspects need to be treated for such scenarios. So we propose the following modification in the WID:

Proposal 1: RAN to update the objective on long connections at RAN#98-e as follows:

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation and  other relevant operations for UL synchronization (e.g. SIB acquisition, enhancements on UL segmentation duration) during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

Another aspect that might be relevant for the NB-IoT implementation, in special for the scenarios with Earth-Moving Cells regards the handling of HARQ and Repetition.

- Repetition/HARQ continuation between cells.

The NB-IoT NTN does not currently allow handover, but, as noted above, the connection time can be longer than the NTN cell availability. If the number of repetitions is reduced to ensure they complete within the NTN cell availability time the link budget is effectively limited, especially if UEs are not ready to transmit right at the start of NTN cell availability. Alternatively, the UE may not be able to transmit or complete the data transmission in the NTN cell or even in any single NTN cell, and all the transmission repetitions may be wasted.

Observation 4: To ensure enhanced system operation during long connections in IoT NTN repetition/HARQ continuation between cells may be required.

Proposal 2: RAN to include the following objective on the WID:

Study and specify, if needed, procedures for repetition periods that extend beyond cell coverage availability. [RAN1, RAN2]

Based on the above discussion, it seems beneficial to expand the scope of the release 18 IoT NTN work item to ensure there is general support for long connections by considering (via a study and discussion) the identified potential issues, which may occur during long connections.

## 2.2 Update on Mobility related aspects.

At RAN2 #120 the following was agreed:

RAN2 doesn’t introduce neighbour cell ephemeris in Rel-17 IoT-NTN, neither for eMTC not for NB-IoT. RAN2 agrees to support this in Rel-18, with details FFS.

Thus, there is an expectation in RAN2 to be able to discuss introduction of neighbor cell ephemeris in release 18 IoT NTN. The question is whether this can be expected to be covered by any of the existing objectives related to mobility enhancements in [1], which are as follows:

4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] Define UE RRM core requirements for the above mobility enhancement features [RAN4].

The first objective is related to neighbor cell measurements and is supposed to reuse Rel-17 TN as a baseline. In Rel-17 TN the 3GPP introduced connected mode measurements for NB-IoT, however the target/neighbor cell’s ephemeris was obviously not discussed and specified, because it is an NTN-specific feature. The addition of neighbor cell ephemeris will be beneficial to speed up the connection reestablishment after the UE experiences RLF on the serving cell.

Therefore, it will be beneficial for RAN to clarify that RAN2 commitment to RAN4 is captured as part of the objective of this WID.

Proposal 3: RAN to update the objective on mobility enhancements at RAN#98-e as follows:

-    Support of neighbour cell measurements, corresponding measurement triggering before RLF, and provisioning of neighbour cell ephemeris information using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

# 3	Conclusion

This document has made the following observations:

Observation 1: Long connection might be interpreted as a connection that lasts for several transmissions in short burst or to long repetition-based transmissions – already provided in specification – used for coverage enhancement purposes. The analysis provided in this document is focused on the second case: enabling a feature that is already specified.

Observation 2: The WID scope on GNSS operations during long connection times has the intention to provide a more consistent framework for UL synchronization, avoiding unnecessary failures and energy consumption when long connection is needed for coverage enhancements purpose.

Observation 3: Support of long connections in IoT NTN release 18 may require a study of further issues in addition to addressing improved GNSS operations.

Observation 4: To ensure enhanced system operation during long connections in IoT NTN repetition/HARQ continuation between cells may be required.

We propose the following:

Proposal 1: RAN to update the objective on long connections at RAN#98-e as follows:

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation and  other relevant operations for UL synchronization (e.g. SIB acquisition, enhancements on UL segmentation duration) during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

Proposal 2: RAN to include the following objective on the WID:

Study and specify, if needed, procedures for repetition periods that extend beyond cell coverage availability. [RAN1, RAN2]

Proposal 3: RAN to update the objective on mobility enhancements at RAN#98-e as follows:

-    Support of neighbour cell measurements, corresponding measurement triggering before RLF, and provisioning of neighbour cell ephemeris information using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

# References

- RP-221806, Mediatek Inc, “Revised WID on IoT NTN enhancements”, RAN #96

- RP-211601, Mediatek Inc, Eutelsat S.A., “WID on NB-IoT/eMTC support for NTN”, RAN #92e

- R1-2207737, Mediated Inc, “Feature lead summary #2 of AI 9.12.4 on improved GNSS operations”, RAN WG1 Meeting #110, Toulouse, France, August 22 – 26, 2022