3GPP TSG RAN meeting #99								        	       RP-230073

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.3.2.5

| WI / SI Name | In-Device Co-existence (IDC) enhancements for NR and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_IDC_Enh |  |  |  |  |
| Unique ID | 941103 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213589 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 70% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Yumin Wu |
|  | Company | Xiaomi |
|  | Email | wuyumin@xiaomi.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#121 meeting agreements:

FDM

- Adopt Option 1 based frequency range reporting to the network i.e Center frequency + bandwidth in KHz/MHz for the actual affected frequencies is reported by the UE to the network for addressing IDC problem in R18.

- Take the ASN.1 framework for option 1 as a starting point in the Text proposal section and work on the following enhancements

  - 1.	Add granular values for band width (including BW in KHz/Mhz) to cover all the scenarios involving Wi-Fi, GNSS, BT

  - 2.	Add the other IEs such as direction of interference.

  - 3.	Add combination of frequencies’ range for addressing IMD scenarios.

  - 4.	Check whether to reuse maxFreqIDC-r16, or define maxFreqIDC-r18

- In MR-DC scenarios, SN can also configure the UE for IDC reporting in SN, including both FDM and TDM solution.

- no additional co-ordination is needed for IDC configuration, apart from the existing mechanism between MN and SN (i.e. candidateServingFreqListNR in CG-Config for EN-DC).

- The gNB configures the candidate frequency ranges using (centre frequency + bandwidth) for which the UE should report IDC issues. Network may indicate the whole bandwidth of the freq.

- The frequency range (centre frequency + bandwidth) reported by the UE shall at least overlap with the frequency range (centre frequency + bandwidth) configured by the network.

- The centre frequency reported by the UE is within the frequency range (centre frequency + bandwidth indicated by network in the configuration) configured by the network.

- If the UE detects interference in both directions for one candidate frequency range indicated by the gNB, the UE can report two affected frequency ranges with the respective interference direction, as legacy. No extra specification change is required.

- LTE MN does not configure the UE with R18 NR IDC configuration.

TDM

- The NR values of long/short DRX cycle and start offset are used for periodic pattern. RAN2 will not introduce new DRX value for network configuration for IDC purpose.

- The slot offset with 1/32ms granularity is included in UEAssistanceInformation-v18xy-IEs for start offset.

- Multiple periodic patterns for IDC are not supported in R18.

- Per CG pattern is supported for MR-DC.  SN can configure the UE to report the TDM assistance information directly to SN, either through SRB 1 (if SRB3 is not configured) or SRB 3.

- FFS whether any additional coordination is needed for network to resolve the problem when network receives the reporting from UE.

- Slot as time unit for autonomous denial

- Agree to send LS to RAN4, indicate the progress in RAN2

- For NR-DC, per CG idc-AssistanceConfigTDM-r18  is introduced to indicate whether TDM assistant information needs to be reported.

- For NR-DC, per CG idc-AssistanceConfigFDM-r18 is introduced to indicate whether FDM assistant information needs to be reported. FFS on dependency between FDM and TDM configuration.

- The values of drx-onDurationTimer in NR is used as the baseline for active Duration in UE assistant information. FFS on other values.

- The same values of validity period and number of denial slots as in LTE is reused. FFS on other values.

- The autonomous denial configuration is per CG.

UE capabilities

- Rel-18 IDC UE capability(ies) defined in NR side is/are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

- In NR side, 3 capability bit is introduced for FDM, periodic pattern and autonomous denial separately.

- The pre-requisite of autonomous denial is FDM solution (R16 or R18) or periodic pattern.

#### 2.2.2	Remaining Open issues

The following issues could be considered:

- Reporting signalling details and procedures for both FDM and TDM solutions for NRDC

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not started.

#### 2.4.2	Remaining Open issues

The following issues could be considered:

- The performance requirements for autonomous denial

## 4.	References

- R2-2300827	Draft 36.306 CR for Rel-18 IDC UE capabilities	Intel Corporation	draftCR	Rel-18	36.306	17.3.0	NR_IDC_enh-Core

- R2-2300828	Draft 36.331 CR for Rel-18 IDC UE capabilities	Intel Corporation	draftCR	Rel-18	36.331	17.3.0	NR_IDC_enh-Core

- R2-2300829	Draft 38.306 CR for Rel-18 IDC UE capabilities	Intel Corporation	draftCR	Rel-18	38.306	17.3.0	NR_IDC_enh-Core

- R2-2300830	Draft 38.331 CR for Rel-18 IDC UE capabilities	Intel Corporation	draftCR	Rel-18	38.331	17.3.0	NR_IDC_enh-Core

- R2-2301485	Draft 38.300 CR for IDC Enhancements	Huawei, HiSilicon	draftCR	Rel-18	38.300	17.3.0	B	NR_IDC_enh-Core

- R2-2301486	Summary of [Post120][652][IDC] Further details of FDM solution (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core

- R2-2302071 	[AT121][652][IDC]  Discussion on FDM solution(Huawei)	Huawei	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300522	More granular FDM indications	Ericsson	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300523	IDC configuration and report in MR-DC	Ericsson	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300543	FDM Solutions in IDC	Qualcomm Incorporated	discussion	Rel-18

- R2-2300743	Discussion on detailed FDM solutions in IDC	Apple	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300831	Enhanced FDM solution for IDC	Intel Corporation	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300874	FDM solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300968	FDM solution for IDC	Lenovo	discussion	Rel-18

- R2-2301108	Remaining issues for FDM and MRDC coordination	Xiaomi	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301326	Discussion on FDM solution for IDC	Samsung	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301487	Further discussion on details of FDM enhancement for NR IDC	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301598	Discussion on FDM solution for R18 IDC	vivo	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301706	Further Consideration on the IDC FDM Solutions	ZTE Corporation, Sanechips	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301799	Discussion on IDC FDM solution enhancement	CATT	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301599	Summary of [Post120][651][IDC]Further details of TDM solution (vivo)	vivo	discussion	Rel-18	NR_IDC_enh-Core

- R2-2302261 Summary of [AT121][651][IDC]Discussion on TDM solution		vivo, xiaomi	discussion	Rel-18	NR_IDC_enh-Core

- R2-2302263 LS to RAN4 on autonomous denial for IDC	LS out	Rel-18 	NR_IDC_enh-Core	To:RAN4

- R2-2302264 TP for IDC TDM solution	vivo, xiaomi	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300524	NR IDC TDM solutions and indications	Ericsson	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300544	TDM Solutions in IDC	Qualcomm Incorporated	discussion	Rel-18

- R2-2300744	Discussion on TDM solutions in IDC	Apple	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300832	TDM solution for IDC	Intel Corporation	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300875	TDM solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300943	Discussion on TDM solution enhancements	Sharp	discussion

- R2-2301109	Remaining issues for TDM solutions	Xiaomi	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301327	Discussion on TDM solution for IDC	Samsung	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301488	Further discussion on details of TDM solution for NR IDC	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301600	Discussion on IDC TDM solution	vivo	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301707	Further Consideration on the IDC TDM Solutions	ZTE Corporation, Sanechips	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301920	[Pre121][654][IDC] Summary of agenda item 8.10.4 UE capabilities(Intel)

- R2-2302073 	[AT121][653][IDC]  Discussion on IDC capabilities (Intel)	Intel	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300745	IDC UE capability	Apple	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300833	UE capabilities for IDC	Intel Corporation	discussion	Rel-18	NR_IDC_enh-Core

- R2-2300873	UE capabilities	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301110	UE capability bits for IDC	Xiaomi	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301489	Discussion on UE capability for IDC enhancement	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301601	Discussion on IDC UE Capabilities	vivo	discussion	Rel-18	NR_IDC_enh-Core

- R2-2301708	Consideration on the IDC Capabilities	ZTE Corporation, Sanechips	discussion	Rel-18	NR_IDC_enh-Core

- R2-2302074, LSout, to RAN4, “LS to RAN4 on autonomous denial for IDC”