3GPP TSG-RAN Meeting #99	RP-230106

Rotterdam, Netherlands, 20–23 March 2023

# Agenda item:	9.3.2.5

# Source:	Samsung

# Title:	Support of hardware sharing indication in IDC

# Document for:	Discussion and Agreement

# 1	Introduction

The contribution discusses the support of hardware sharing indication in Rel-18 IDC.

# 2	Discussion

The latest WID for IDC in RP-221281 [1] contains the following objectives, which consists of 'enhanced FDM' and 'TDM solution':

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2) Note: Enhancements to FDM solution is prioritized. - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. - Specify RRM requirements for TDM solution (RAN4) Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

During RAN2#119-e in August 2022, RAN2 discussed whether the indication hardwareSharingProblem-r13 from LTE is part of the WI [2], and then concluded the following agreements:

RAN2 consider “hardware sharing indication” is out of scope. Company should bring this to RANP if want to support it.

Even if the hardware sharing indication feature–which exists in LTE from Rel-13–was ruled out at that time, we, as a UE vendor, believe this feature is indeed essential when the NR-U is deployed. That is, when a user of the UE wants to use WLAN of the UE, UE should be able to indicate to the network that it now uses WLAN–which shares the RF components with NR-U communication–and thus, the network should not configure NR-U to the UE, since user preference takes precedence over the configuration.

The required specification work would also be marginal, assuming simple porting from LTE to NR: UE capability, configuration from the network, and one-bit indication from the UE side (as UEAssistanceInformation). Hence, we propose:

Proposal:	To add the following objective to the latest IDC WID (RP-221281): 
•	Introduction of hardware sharing indication. (RAN2)

# 3	Conclusion

Proposal:	To add the following objective to the latest IDC WID (RP-221281): 
•	Introduction of hardware sharing indication. (RAN2)

# 4	References

[1]	RP-221281	Revised WID: In-Device Co-existence (IDC) enhancements for NR and MR-DC	Xiaomi

[2]	R2-2208952 	[AT119-e][652][IDC] TDM solution (Xiaomi)	Xiaomi	discussion	Rel-18	NR_IDC_Enh-Core