3GPP RAN Meeting #99																				RP-230151

Rotterdam, Netherlands, March 20-23, 2023

Agenda item:	9.3.4.1

Source:	Apple

Title:	Discussion on RAN4 LS answer for less than 5MHz NR channels

Release:	Rel-18

Document for:	Approval

# 1  Introduction

In [1] a work item for support of spectrum less than 5MHz was agreed in RAN plenary. Subsequently the work on this WI started in RAN4, however, in this discussion some questions came up that have been addressed in an LS from RAN4 to RP in [2].

# 2  Discussion about “Dedicated Spectrum

The WI title stated “NR support for dedicated spectrum less than 5MHz for FR1”. However, there were also legacy bands n8, n26, n28 listed in the WID, where NR UEs are operating since Rel. 15. Obviously defining a new channel bandwidth in a legacy band doesn’t use “dedicated spectrum” and will affect already existing deployment of NR with 5MHz CBW or more, since especially at cell search the UE may come across the new <5MHz signals within the band and it may try to decode it and not be successful, since the 3MHz signal is unknown. But it will spend significant time trying to decode the signal delaying the cell search for the legacy cells.

Observation 1: Having a <5MHz carrier within a legacy band is likely to disturb the cell search of legacy UEs resulting in delays for cell search for legacy carriers

If <5 MHz CBW is to be defined for legacy bands like n8, n26, n28, channel raster and sync channels need to be designed in a way not to disturb the cell search for legacy UEs in legacy networks. This will restrict the design of the <5MHz raster channels.

“Dedicated spectrum” implies that the spectrum of a band is only used for new deployments and there are no legacy networks, for example n100 or the upcoming 900MHz band for the US would be dedicated spectrum, but a legacy band cannot be dedicated spectrum, since within the band there will be multiple services already available. Using the “dedicated spectrum” term contradicts the use of legacy bands like n8, n26 or n28.

Observation 2: A legacy band like n8, n26 or n28 is not dedicated spectrum, since there are multiple legacy networks operating within the spectrum of the band

Proposal 1: RP to decide, if this work item either needs to be extended to explicitly cover legacy bands and the “dedicated spectrum” term is removed, or if the <5MHz channel request for legacy bands n8, n26 or n28 will be removed

Proposal 2: Remove legacy bands n8, n26 or n28 from the WID, since this will give more freedom to optimize the <3MHz channel for dedicated spectrum in new bands

# 3  Features to be considered for the <5MHz CBW

In the WID it is not mentioned, if <5 MHz CBW is just considered as a new CBW for single carriers, or if other features also need to be taken into acocunt. Especially unclear is, if Carrier Aggregation for Dual Connectivity need to be taken into account. Also unclear is, if this new CBW also applies to RedCap devices.

Observation 3: The features to be considered in conjunction with the new <5MHz CBW, for example CA, DC or RedCap, are not clearly defined.

Proposal 3: RP to define the features to be used in conjunction with <5MHz CBW, such as CA, DC or RedCap.

# 6  Conclusions

This contribution describes clarifications needed to answer the LS from RAN4. The following observations and proposals are made:

Observation 1: Having a <5MHz carrier within a legacy band is likely to disturb the cell search of legacy UEs resulting in delays for cell search for legacy carriers

Observation 2: A legacy band like n8, n26 or n28 is not dedicated spectrum, since there are multiple legacy networks operating within the spectrum of the band

Observation 3: The features to be considered in conjunction with the new <5MHz CBW, for example CA, DC or RedCap, are not clearly defined.

Proposal 1: RP to decide, if this work item either needs to be extended to explicitly cover legacy bands and the “dedicated spectrum” term is removed, or if the <5MHz channel request for legacy bands n8, n26 or n28 will be removed

Proposal 2: Remove legacy bands n8, n26 or n28 from the WID, since this will give more freedom to optimize the <3MHz channel for dedicated spectrum in new bands

Proposal 3: RP to define the features to be used in conjunction with <5MHz CBW, such as CA, DC or RedCap.

# 4  References

- RP-222645, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN Meeting #95e

- R4-2303713, “LS for spectrum less than 5 MHz”, Apple, 3GPP RAN WG4 Meeting #106