3GPP TSG-RAN Meeting #99 	                                                                 RP-130159

Rotterdam, Netherlands, March 20-23, 2023				(revision of RP-223112)

# Source:	Apple

# Title:	Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

# Document for:	Endorsement

Agenda Item:	9.3.4.13

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

Acronym: NR_FR1_TRP_TRS_enh

Unique identifier: 970082

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-18

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| NR_FR1_TRP_TRS | RAN4 | 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| NR_FR1_TRP_TRS-UEConTest | 960088 | UE Conformance - Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | RAN5 conformance test WI |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

This work item introduces a number of enhancements for TRP TRS requirements as well as corresponding verification methodologies:

- Enhancements of TRP TRS test methodology

The baseline test method (anechoic-chamber based system) of FR1 TRP TRS was introduced in Rel-17 for FR1 TRP TRS. However, the specified test method is only applicable to UE with NR 1Tx capability, the advanced UE features with more Tx chains such as TxD, UL MIMO, and CA are not testable. Enhancement of test methodologies to support the verification of above features is required to ensure the radiated performance of the devices.

Besides, the RedCap UE was introduced and finalized in Rel-17 with maximum channel bandwidth 20MHz and minimum 1Rx, the OTA test configuration and corresponding new phantoms are not defined. Enhancement of TRP TRS test method to support RedCap UE performance verification is important.

In addition, the reverberation-chamber based system was discussed but deprioritized in Rel-17, which is also valuable to perform TRP TRS measurement as an alternative method to save measurement time and re-use large scale of legacy OTA systems.

- TRP TRS requirements for NR 1Tx (SA and EN-DC)

In Rel-17, the requirement is only defined for selected bands (n41 and n78) under SA mode with very specific configuration, i.e., wide handset UE (width ≥72mm) operated with 1Tx at PC2 under browsing mode scenario (hand phantom only), while other cases are all deprioritized, e.g., other NR bands, EN-DC configurations, talk mode scenarios (head and hand phantom), narrow devices (width ≤72mm), and other power classes. Requirements for these types of UEs may also be required.

For other bands listed in the first priority list but not defined in Rel-17 (n28 and n79), should be finalized in Rel-18 as high priority.

- TRP TRS requirements for NR 2Tx configuration (SA and CA)

In Rel-16, many requests from operators and vendors on bands support UL-MIMO were seen. With increased interests from industry on UL coverage performance, a significant number of WIs in Rel-17 were focused on high power solutions and multi-antenna Tx features, which include, e.g., UL MIMO, PC2, TxD, CA high power limit, and PC1.5. All of these enhancements are anticipated to enhance the performance of NR networks overall; thus, developing radiated requirements for UEs which are capable of these features is a priority for the industry.

# 4	Objective

### 4.1	Objective of Core part WI

The following objectives are considered in this WI:

- Enhancements of TRP TRS test methodology

  - Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline):

    - UE with NR 2Tx configuration

      - Case 1: TxD (i.e., TxD capability supported)

      - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported)

        - Study proper configuration from UE implementation and test system feasibility perspective

      - Define test case applicability for case 1 and case 2

    - RedCap devices (focused on wearable device only)

      - Wrist-worn device as 1st priority

      - Considering UEs with antenna configurations of 1 Tx, 1 Rx, 2 Rx

      - Forearm phantom should be specified

        - Consider the coordination with CTIA on this aspect

      - Other phantoms are not precluded

  - Study and, if feasible, specify reverb-chamber based test methodology to support SA and EN-DC TRP TRS testing:

    - Define test methodology for UE type with 1Tx and 2Tx configuration

      - Test scenarios support: at least browsing mode, talk mode, and wrist-worn mode

    - Perform harmonization activity with reference test method

      - Define harmonization framework and pass/fail criteria

      - Perform harmonization test campaign according to the framework

      - Note: the harmonization between anechoic chamber and Reverb-chamber based method can be applied independently for the conditions between 1Tx and 2Tx, among browsing mode, talk mode, and wrist-worn mode.

    - If reverb-chamber based test methodology is found harmonized with the reference test method, then further discuss applicability of reverb-chamber based test methodology for requirements conformance testing

  - Develop the Measurement Uncertainty (MU) assessment of the above newly defined test methodologies and test configurations [RAN5]:

    - Measurement Uncertainty (MU) aspects will be handled by RAN5 and the conclusions can be captured in a separate section of the TR

    - Browsing mode, talk mode, and wrist-worn mode should be considered

  - Study and specify the testing time reduction methodology for TRP and TRS testing

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9 WG1, GCF, GSMA TSG-AP, ETSI MSG TFES, and PTCRB shall be maintained to ensure industry coordination on this topic

### 4.2	Objective of Performance part WI

Specify the performance part framework (general for both 1Tx and 2Tx requirements work):

    - Lab alignment outcome and performance campaign framework in Rel-17 should be baseline with the following considerations:

      - Decide whether lab alignment is needed for new test bands, new test scenarios and EN-DC configuration

      - Handheld UE type is the first priority

      - Consider UE size 1 and size 2

      - UE size 1 is the first priority

      - Both browsing mode and talk mode requirements are introduced per band

      - Consider Power class 2 and 1.5 for 2Tx requirements, Power class 3 and 2 for 1Tx requirements

      - Requirement applicability of TxD and single-layer UL MIMO of the same UE shall be specified

      - Consider further prioritization of test cases

      - Taking following as a starting point to further discuss which information needed including for information disclosure:

      - [Number of models tested by the labs]

      - Number of vendors that produced the models

      - percentage of tested devices per vendor

      - Percentage of models per production year

      - [Power Class of the devices]

- Note 1: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm);

- Note 2: Browsing mode (hand phantom only), Talk mode (head and hand phantom);

- Specify TRP TRS requirements and recommended tolerance for UE with NR 2Tx for handheld UE based on enhanced reference test method and defined performance part framework

  - Specify the requirements and test tolerance for UE with SA mode

    - Band n41, n77 and n78, as the first priority

    - FDD bands are not precluded

    - SA with 1 CC is the first priority

- Specify TRP TRS requirements and recommended tolerance for UE with NR 1Tx (SA and EN-DC) based on reference test method and defined performance part framework

  - Specify the requirements and recommended tolerance for UE

    - SA: n28 with PC3, n77 with PC2 and PC3

      - For talk mode requirements, n41 and n78 with PC2 and PC3 shall also be specified

    - EN-DC: NR SA bands related EN-DC band combinations are the first priority, only NR carrier requirement will be specified

      - Rel-17 band combination conclusions in TS 38.161 should be the basis

      - For each newly added NR band to the scope, at least one example band combination shall be defined

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.870 | Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) | TSG#101 | TSG#102 | Related to Core part; Leading WG: RAN4 Rapporteur: Wang, Ruixin, vivo, ruixin.wang@vivo.com |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS38.161 | TRP TRS requirements related test aspects | RAN#102 | Core part |
| TS38.161 | TRP TRS 1Tx and 2Tx requirements | RAN#104 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Wang, Ruixin, vivo, ruixin.wang@vivo.com

# 7	Work item leadership

RAN WG4

Secondary responsibility: RAN WG5

# 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

vivo

Apple

CAICT

CBN

China Telecom

China Unicom

CHTTL

CMCC

ETS-Lindgren

Element Materials Technology

Genius technology

Huawei

HiSilicon

Intel Corporation

Keysight

MediaTek inc.

MVG

OPPO

Orange

Qualcomm Incorporated

Rohde & Schwarz

Samsung

Sanechips

Spirent

Sporton international

SAICT

Telecom Italia

Vodafone

Xiaomi

ZTE