3GPP TSG-RAN Meeting #99		RP-230185

Rotterdam, Netherlands, 20th – 23rd March, 2023

Agenda item:	9.3.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Response to LS from RAN4 on spectrum less than 5 MHz

Document for:	Discussion and Decision

Introduction

In last RAN4#106 meeting, RAN4 initiated the discussions related to <5MHz WI. One aspect was discussed related to whether it is planned to deploy the less than 5MHz channels in legacy bands, where there are already legacy UEs and networks operating which could be affected by the new channel. Another aspect discussed related to whether it is assumed that a device supporting <5MHz is assumed to possibly support UE requirements for all features/capabilities defined up to and including Rel-17.

RAN4 decided to ask guidance from RAN in LS [1]:

Question 1: Is it planned to deploy the less than 5MHz channels in legacy bands, where there are already legacy UEs and networks operating and could be affected by the new channel?

Question 2: Which features are expected to be considered or not to be considered in less than 5MHz deployments considered in this WI, e.g., CA, DC, RedCap?

Discussion of legacy bands

For the question on legacy bands and UE operation, we believe this depends on the deployment scenario for each operator in each band. For example, there will be no legacy UEs in band n100 while there could be legacy UEs in other bands.

Discussion of supported features

Here we give our view on which features we see would not necessarily needing to be able to be supported while the UE is operating in <5MHz.

Hence, the intention is to list UE features, and related UE requirements, which a UE supporting <5MHz might assume being supported or not while operating in <5MHz deployments in the Rel-18 timeframe. If the UE indicates support for a feature and is capable of supporting <5MHz, which current UE requirements are applicable when operating in <5MHz deployments.

For example, for a UE supporting <5MHz:

Case 1) UE supports CA and <5MHz but is not currently operating in <5MHz deployment: UE can be configured CA and if configured with CA the UE shall fulfil all existing related UE requirements.

Case 2) UE supports CA and <5MHz and is currently operating in <5MHz deployment:  UE can assume not being configured with CA while operating in <5MHz and hence no CA requirements are applicable in Rel-18

Whether RAN4 shall define <5MHz UE requirements, assuming all existing features are supported in <5MHz deployments or not, needs discussion. To facilitate this discussion two different approaches were discussed in RAN4:

- Use R15 as a baseline and evaluate for which capabilities/features the <5MHz UE requirements shall be defined (which features are assumed applicable in <5MHz deployments in Rel18).

- Use R17 as a baseline and evaluate which capabilities/features do not need requirements for <5MHz to be defined (i.e., which features are not assumed applicable in <5MHz deployments in Rel18).

One can argue that <5MHz is a Rel-18 feature and hence it is assumed that it builds on top of the latest release. Hence, Release 17 and the all features defined up to and including Rel-17 need to be considered when defining the UE requirements for <5MHz. It may be simplest to take this approach and assume using release 17 as baseline and down select which features would not be assumed supported in a <5MHz deployments.

Initially we believe that it must be up to UE indication whether a certain optional feature is supported by the UE. Same for UE support of <5MHz. However, the question is whether there is a need to define the UE requirements for <5MHz for all already existing defined features and UE requirements?

For example, should RAN4 define <5MHz specific UE requirements for all release 16 and 17 defined features e.g. NR-U, RedCap etc.?

It may also be questioned whether Rel-18 <5MHz UE requirements need to be defined for all basic Rel-15 features like for example assuming CA is also applicable in <5MHz deployments?

Hence, the question is which features are expected to be used together in <5MHz deployments in Release 18 timeframe? In the following we look at which features we believe need not be considered.

In general, we think the UE supporting <5MHz and operating in <5MHz deployment may assume that most of the features for which RAN4 has introduced requirements after Rel-15 are not supported in these deployments (hence, any new requirements introduced in Rel-16 and Rel-17). Hence, at least the following features do not need to be considered, and RAN4 need not define <5MHz UE requirements for these:

- NR-U (the relevant spectrum for <5MHz operation is not unlicensed)

- RedCap (the intended device types for <5MHz operation do not have the RedCap characteristics)

- satellite access (the intended use cases are terrestrial)

- CSI-RS based L3 mobility (mobility can be based on L3 SSB based measurements in Rel-18. If needed, CSI-RS L3 mobility can be discussed in a later release)

- CA (In a future release, it could be helpful to add support for CA, for example between bands n100 and n101. But in Release 18 it does not seem essential, and the amount of work involved to define the requirements would be significant.)

- DC (In a future release, it could be helpful to add support for DC, for example between bands n100 and n101. But in Release 18 it does not seem essential, and the amount of work involved to define the requirements would be significant.)

- BWP switch (In such narrow bandwidths, BWP switching does not seem the most useful feature.)

- mTRP support (This does not seem essential for the intended use cases.)

- Measurement gap enhancement after Rel-15  (Basic Rel-15 measurement gap support should be sufficient to enable SSB based mobility)

- CLI measurements (The intended spectrum is FDD.)

- L1-RSRP measurements for a cell with different PCI from serving cell (Currently we do not see that inter-cell beam management feature would be essential feature for <5MHz deployments)

A number of requirements are unclear and would need more discussion:

- CHO (CHO may be useful in case HO delays are increased due to relaxed UE procedures)

- Only LTE inter-RAT (if mobility to other RAT’s than LTE this can be supported)

- SFTD (This deployment should be well planned but if needed support for SFTD reading can be supported)

- CGI (This deployment should be well planned but if needed support for CGI reading can be supported)

- Positioning (Accuracy of positioning in narrow BW may need further discussions)

- Measurement for Propagation Delay Compensation

Hence, we suggest:

RAN4 should define <5MHz UE requirements under the assumption that features 1 – 11 above are not supported in <5MHz deployment scenarios in release 18.

RAN to further discuss and decide whether to define <5MHz UE requirements under the assumption that features 12 – 17 may be deployed in <5MHz deployment scenarios in release 18.

In the appendix we have provided a draft LS reply.

Supported velocity

Another aspect discussed during the meeting was the assumed speed to be supported. It is not clear from the WID which maximum speed RAN4 should assume is supported. Our view here is that UE speeds up to 500km/h should be supported for Band n100 in the WI.

The WI shall support UE speeds up to 500km/h.

Conclusions

- Proposal 1: RAN4 should define <5MHz UE requirements under the assumption that features 1 – 11 above are not supported in <5MHz deployment scenarios in release 18.

- Proposal 2: RAN to further discuss and decide whether to define <5MHz UE requirements under the assumption that features 12 – 17 may be deployed in <5MHz deployment scenarios in release 18.

- Proposal 3: The WI shall support UE speeds up to 500km/h.

- A draft LS response is provided below for discussion and approval.

References

- R4-2303713, LS for spectrum less than 5 MHz, Apple.

# A	Draft LS Reply to RAN4

3GPP TSG RAN#99			RP- 23xxxx

Rotterdam, Netherlands, 20th – 23rd March, 2023

Title:	[DRAFT] LS to RAN4 for spectrum less than 5 MHz

Response to:	R4-2303713

Release:	Rel-18

Work Item:	          NR_FR1_lessthan_5MHz_BW-Core

Source:	Nokia, Nokia Shanghai Bell [RAN]

To:	RAN4

Cc:

Contact Person:

#### Name:	Matthew Baker

E-mail Address:	<Matthew.Baker@nokia.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN would like to thank RAN WG4 for their LS for spectrum less than 5 MHz.

RAN have discussed question 1 on legacy bands and UE operation, and concluded that this depends on the deployment scenario for each operator in each band; for example, there will be no legacy UEs in band n100, while there could be legacy UEs in other bands.

RAN have discussed question 2 on the feature list, and concluded that at least the following features need not to be considered in Release 18, and RAN4 need not define <5MHz UE requirements for these:

1.	NR-U

2.	RedCap

3.	satellite access

4.	CSI-RS based L3 mobility

5.	CA

6.	DC

7.	BWP switch

8.	mTRP support

9.	Measurement gap enhancement after Rel15

10.	CLI measurements

11.	L1-RSRP measurements for a cell with different PCI from serving cell

[12.	CHO]

[13.	Only LTE inter-RAT]

[14.	SFTD]

[15.	CGI]

[16.	Positioning]

[17.	Measurement for Propagation Delay Compensation]

Moreover, RAN have discussed and concluded that UE speed up to 500km/h should be supported for Band n100 in the WI.

2. Actions: RAN respectfully requests RAN4 to take the above information into account for their further work.

3. Dates of Next RAN Meetings:

TSG RAN Meeting #100		June 12th – 14th, 2023	Taipei, Taiwan

TSG RAN Meeting #101		September 11th – 15th, 2023	TBC, India

References

[1] RP-222645, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Nokia Shanghai Bell, RAN#95e